PHASE 2

TRANSFORMING METHANE FROM HAZARD TO RESOURCE: JSW’S INTEGRATED PROGRAMME FOR METHANE EMISSION REDUCTION AND HYDROGEN PRODUCTION

Artur Badylak

Jastrzębska Spółka Węglowa S.A.

Production Department

Methane Drainage and Management Team

ABSTRACT

Methane emissions represent the most significant environmental challenge in coal mining, particularly in coking coal operations. Jastrzębska Spółka Węglowa S.A. (JSW S.A.), the European Union’s largest producer of high-quality hard coking coal, has identified methane as a key component of its carbon footprint, accounting for nearly 72% of total greenhouse gas emissions. To address this, JSW has launched a comprehensive Methane Emissions Reduction Programme aimed at achieving 50% methane capture and 95% energy utilization by 2027, as part of its roadmap toward climate neutrality by 2050. Currently, methane management in coal mines relies on a combination of pre-drainage, post-drainage, and ventilation air handling, yet the efficiency of these systems remains limited by geological conditions, low methane concentrations, and the lack of integrated digital control. Recognizing these gaps, JSW has initiated four strategic EU co-funded projects under the Horizon Europe and RFCS frameworks—MASTERMINE, REM, ProVAM, and METH2GEN, with a total budget exceeding €63 million, including over €37 million directly invested by JSW. These projects introduce innovative solutions across the full methane management chain: MASTERMINE- automates methane drainage systems using digital twin models, AI-assisted control, and real-time data analytics, REM- develops parallel drainage networks to enable the generation of electricity and heat from captured methane, ProVAM- addresses one of the most difficult challenges—reducing methane from ventilation air (VAM)—by improving oxidation efficiency under variable airflow, humidity, and dust conditions, METH2GEN- advances the conversion of residual methane to hydrogen through directional drilling, catalytic reforming, and CO₂ utilization, contributing to circular energy systems. Collectively, these initiatives demonstrate a new paradigm in coal mine methane management: integrating digitalization, decarbonization, and energy valorisation. By turning methane from an emission source into 
a renewable energy sector, JSW contributes both to operational safety and to the EU’s Green Deal objectives. 
The implementation of these projects establishes a scalable model for deep emission reduction and energy recovery in coal mining, positioning JSW as a European leader in low-carbon transition within the metallurgical coal sector.

- INTRODUCTION

The JSW Group is the largest producer of high quality hard coking coal in the European Union and one of the leading producers of coke used for smelting steel. Production and sale of coking coal and production and sale of coke and hydrocarbons constitute JSW Group’s core business. Coking coal, also known as metallurgical coal, has been identified as one of the 34 critical raw materials for the European Union, signifying strategic economic importance and entailing a high supply risk for European industry. Coking coal is an essential ingredient in steel production - a key component for the development of an innovative economy and modern infrastructure.

Mining of coking coal involves significant methane emissions. Methane is one of the main hazards associated with coal mining, alongside fire, water, rock bursts and gas and rock ejections. In addition to the direct threat to the safety of mining crews, methane, and specifically its emission into the atmosphere, has a significant impact on the environment. It should be remembered that methane and carbon dioxide are considered to be the two gases that have the greatest impact on global warming and climate change. The GWP (Global Warming Potential) for methane over a 100-year period (GWP100) is approximately 28, which means that it is 28 times more potent as a greenhouse gas than carbon dioxide over the same period, although over a 20-year period (GWP20) it is significantly higher at around 86 due to the shorter lifetime of methane in the atmosphere. The European Union's policy is also extremely important in this regard. In 2021, the European Parliament Resolution on the EU strategy for reducing methane emissions was announced. One of the elements of this strategy is the Regulation of the European Parliament and of the Council on the reduction of methane emissions in the energy sector, known as the ‘Methane Regulation’, which entered into force on 5 August 2025. The provisions of this Regulation also take into account methane emissions from the mining sector and apply to both underground and opencast mining. It defines in detail the rules for monitoring, reporting and verifying methane emissions (MRV) and sets emission limits for individual sources.

Methane accounts for approximately 72% of the carbon footprint of the Jastrzębska Spółka Węglowa Group. In 2022, JSW presented its business strategy for the entire Capital Group until 2030. Environmental protection was and remains an important element of this strategy. Due to the significant share of methane in the Group's carbon footprint, reducing methane emissions was an obvious assumption. Therefore, the Methane Drainage and Management Office developed and launched an extremely ambitious Methane Emission Reduction Programme.

Figure 1 - Methane at JSW in 2020-2024 [kilotons]

This programme is based on two pillars:

- Increasing the efficiency of methane drainage up to 50% – capturing 50% of the methane released during mining by the methane drainage system.

- 95% utilization of methane captured by the methane drainage system – maximum utilization of methane for energy production, as dictated by technological constraints.

The Methane Emission Reduction Programme was developed with the participation of the Central Mining Institute – National Research Institute and the AGH University of Science and Technology in Krakow and is currently being implemented at all JSW S.A. plants. The basic projects carried out under the programme include: modernization of methane drainage stations, construction and modernization of underground methane drainage systems, sealing of post-mining isolation dams, drilling of drainage holes for post-mining goafs in closed mine areas, and expansion of infrastructure related to the use of captured methane. A specific change in thinking among JSW's management and employees was important, consisting in changing the awareness that methane is not only a direct threat to their lives and health, that it contributes to global warming, but that it is also a valuable energy resource. Therefore, all measures should be taken to capture the methane released during mining with a methane drainage system and use it for energy production.

Among the many projects implemented as part of the Methane Emission Reduction Programme, four have received funding from the European Union. These are the REM, MASTERMINE, ProVAM and METH2GEN projects. The total budget for these projects is as much as €63.8 million.

- ROCK MASS DEGASSING IS KEY TO REDUCING METHANE EMISSIONS 
FROM COAL MINES

The basic and most effective way to combat the methane hazard in hard coal mines is rock mass degassing. The efficiency of methane drainage at JSW mines in 2024 was almost 38%, with the efficiency of degassing in longwall mining areas with methane drainage reaching almost 60%. This means that as much as 60% of the methane released during coal mining is captured by the methane drainage system, but only 38% of the total methane released into the mine workings. This is due to the fact that the structure of methane release includes not only methane from areas under exploitation, but also from drilled corridor excavations and post-mining goafs. From closed post-mining areas, methane migrates through cracks in the rock mass to active workings that serve as ventilation routes on inactive levels of the mine. This does not pose a direct threat to the working crew because the methane concentration is low. However, the amount of methane released in this way over a long period of time is significant and can amount to several hundred thousand cubic meters per year.

Table 1 -  Methane drainage efficiency at JSW in 2018-2024

| Year | Mine methane drainage efficiency | Methane drainage efficiency from walls with methane drainage system |
| --- | --- | --- |
| 2018 | 32,3% | 51,9% |
| 2019 | 34,9% | 47,6% |
| 2020 | 39,5% | 52,8% |
| 2021 | 42,8% | 55,3% |
| 2022 | 34,0% | 40,1% |
| 2023 | 34,3% | 47,3% |
| 2024 | 38,0% | 58,7% |

Typically, the methane concentration in gas captured by the methane drainage system ranges from 40% to 60%, and according to Polish mining regulations, it cannot be lower than 30%. Often, the gas emitted from post-mining goafs has a lower methane concentration and capturing it would be unprofitable and non-compliant with regulations. This causes gas release into ventilation system, from where it is discharged into the atmosphere through ventilation shafts.

To address this problem the project was launched on 1 March 2023, entitled ‘Reduction of methane emissions from post-mining areas in order to minimize the flow of methane into the ventilation air’. The project, known by the acronym REM, aims to reduce methane emissions from active mines into the ventilation air, which is then discharged into the atmosphere. The project budget is nearly EUR 21.5 million, with co-financing covering 50% of eligible costs. In addition to JSW, the consortium includes five research institutions. From Poland, these are the National Research Institutes – the Central Mining Institute, the Oil and Gas Institute, and the Polish Geological Institute. The Polish institutes are supported by two foreign entities: INSEMEX from Romania and the University of Oviedo in Spain. The project aims to determine the possibility of obtaining methane from the post-mining workings of an active mine by implementing a parallel methane drainage system for low concentration methane. This also required the consent of the Higher Mining Authority President to waive the applicable regulations on the minimum methane concentration in the gas captured at the methane drainage station. In Polish regulations, it is 30%, while in the planned installation, the minimum concentration will be 20%. Such a solution obviously requires the use of additional technical requirements to increase safety. Prototype installation will be built in the ‘Pniówek’ mine, owned by Jastrzębska Spółka Węglowa.

Figure 2 - Reduced concentration methane drainage system diagram at Pniówek coal mine

The methane obtained from post-mining goafs will be used to produce electricity in dedicated gas engines connected to electricity generators enabling the production of electricity from gas with a methane concentration of 20÷40%. The solutions tested in this project will ultimately be implemented in all JSW mines. They can also be successfully used in any methane-bearing hard coal mine in the world with a multi-seam deposit.

Figure 2 presents a diagram of a methane drainage network with reduced methane concentration, which is being developed at the Pniówek coal mine. The project identifies possible methane intake points, where drainage boreholes will be made and, after confirming the appropriate gas parameters, connected to the methane drainage network. An essential element are the dams isolating the post-mining goafs. They will be sealed and connected to the system. For this purpose, tests of rock mass fractures around the dams were carried out and a design for their sealing was developed. The gas captured by the methane drainage station at shaft #V of the Pniówek coal mine will be delivered to energy generation modules. This solution allows methane, which was previously released into the atmosphere with ventilation air, to be used for electricity production. The benefits in terms of safety and environmental protection are obvious, as are the financial benefits resulting from savings in electricity purchases.

- METHANE FROM VENTILATION AIR VAM – DISPOSAL OR UTILISATION?

The purpose of ventilation in mines is to ensure a supply of fresh air and remove harmful substances in order to create safe working conditions. As mentioned earlier, a significant amount of methane is released during mining, some of which is captured through methane drainage, while the rest mixes with the air and is discharged into the atmosphere through exhaust shafts. For safety reasons, Polish mining regulations allow a maximum methane concentration of 0.75% in ventilation shafts. These values are much lower in shafts, averaging between 0.2% and 0.4%.

The Methane Regulation adopted in 2024 concerning the energy sector imposes limits on methane emissions from ventilation shafts. It is not yet clear at what level they will be set for JSW S.A., because as a producer of coking coal, a strategic raw material for the EU economy, it is not currently covered by them.

Table 2 - Air flow and methane concentration in the exhaustion shafts at JSW S.A.

| Plant | Shaft | Average air flow [m3/min] |  |  |  | Average methane concentration |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 2021 | 2022 | 2023 | 2024 | 2021 | 2022 | 2023 | 2024 |
| "Borynia" Site | # III | 25 660 | 26 783 | 26 398 | 24 878 | 0,06% | 0,10% | 0,07% | 0,06% |
|  | # VI | 11 712 | 12 829 | 14 487 | 13 578 | 0,23% | 0,13% | 0,10% | 0,10% |
| "Budryk" Mine | # II | 19 780 | 19 660 | 19 257 | 19 526 | 0,29% | 0,19% | 0,33% | 0,21% |
|  | # V | 16 190 | 15 706 | 14 687 | 9 764 | 0,25% | 0,35% | 0,27% | 0,35% |
| Knurów Site | "Aniołki" | 7 741 | 8 250 | 7 063 | 7 521 | 0,08% | 0,08% | 0,31% | 0,16% |
|  | # V | 12 393 | 12 996 | 10 192 | 9 863 | 0,02% | 0,06% | 0,13% | 0,06% |
| "Pniówek" Mine | # III | 19 921 | 18 954 | 20 166 | 18 716 | 0,12% | 0,12% | 0,10% | 0,13% |
|  | # IV | 10 758 | 10 768 | 10 825 | 10 713 | 0,34% | 0,35% | 0,41% | 0,34% |
|  | # V | 14 845 | 14 329 | 13 492 | 12 660 | 0,32% | 0,33% | 0,33% | 0,33% |
| Szczygłowice Site | # IV | 13 950 | 9 238 | 9 200 | 8 963 | 0,31% | 0,20% | 0,20% | 0,32% |
|  | # VI | 19 900 | 16 624 | 16 740 | 17 013 | 0,36% | 0,27% | 0,34% | 0,41% |
| "Zofiówka" Site | # IVz | 11 920 | 11 930 | 12 242 | 18 623 | 0,08% | 0,07% | 0,08% | 0,13% |
|  | # Vz | 29 810 | 28 441 | 28 605 | 28 173 | 0,11% | 0,35% | 0,29% | 0,21% |

However, the issue of VAM reduction, if not soon, will certainly sooner or later also affect coking coal mines, and thus JSW S.A. In the assessment of the impact of the regulation and during discussions with representatives of the European Commission, it was pointed out that there are already installations in operation around the world for the utilisation and economic use of methane from VAM ventilation air, but none of these installations operate in the European Union under European Union market conditions and applicable regulations. Jastrzębska Spółka Węglowa does not ignore the problem of methane emissions from ventilation air, quite the contrary. In addition to extensive market research and the search for business partners, we are participating in the ProVAM research project. The main objective of the project is to examine the conditions in the ventilation shafts of Polish and Romanian mines, review the available VAM technologies and check the possibility of installing VAM utilisation units in European hard coal mining conditions. A key aspect that was examined was the possibility of using methane from ventilation air to produce heat and energy. The concentration of methane in the exhaust shafts of JSW S.A. mines does not allow for the use of this methane; in this case, only its disposal can be considered. It should be emphasised that, as shown by tests carried out as part of the project at the Barbara Experimental Mine, this process requires the supply of significant amounts of electricity for the proper operation of the installation when the methane concentration in the air is below 0.4%.

Ventilation air parameters such as dust content, humidity, and gas concentrations vary over time and depend on the characteristics of the currently exploited seams, atmospheric conditions on the surface, as well as random events occurring during the operation of the mining plant. The above factors have a significant impact on the design, construction, and operation of such a system, and thus also on the costs, which are already extremely high. The shrinking European hard coal mining sector is certainly not able to bear the cost of introducing such technology on an industrial scale on its own. The construction of a VAM disposal system has a positive impact on the environment but is economically unjustified. Furthermore, there are still many unknowns. These include the size of the plant and the possibility of its construction on limited, available land, as well as its legal basis in mining regulations.

- AUTOMATION OF MEASUREMENTS AND CONTROL

Improving the efficiency of mine methane drainage should not be based solely on improving the efficiency of demethanation in mining faces. Every source of methane emissions, even the smallest one, has an impact on the overall annual methane content. In such large multi-seam mining plants as JSW S.A. mines, there are many methane collection points. There are entire closed post-mining areas from which methane is captured along with dozens of isolation dams. Methane extraction pipelines and equipment are under constant supervision by the plant's ventilation services. Gas parameters in the methane extraction network are not constant; the flow rate and methane concentration in the captured gas vary over time. Atmospheric pressure has a significant impact on methane emissions and, consequently, on the performance of the entire methane extraction network. In the case of methane extraction in areas where there is no mining activity, this impact is even greater. Until now, network regulation and measurements at these points have been carried out by an employee of the in-situ ventilation department.

The MASTERMINE project, financed by the Horizon Europe research fund, is being implemented by a consortium of 22 entities, including two from Poland: GIG-PIB and JSW S.A. The project aims to create a system enabling mines to digitize, develop sustainably while respecting the environment, and monitor productivity.

The task of the Polish entities is to develop and install an automatic control and measurement point for the existing methane drainage network. The installation of such points in critical methane intake locations in underground workings is intended to solve the problems described above.

Figure 3 - An automatic control and measurement point diagram MASTERMINE

This set of devices (figure 3), which has already been installed in one of the selected methane capture points at the Pniówek coal mine, allows for remote viewing of the parameters of the flowing gas and for automatic regulation of the flow of the captured methane-air mixture. The solution currently being tested will facilitate the work of the mine's ventilation services and allow for a faster response to changes in atmospheric pressure, which will ultimately contribute to a reduction of methane emissions into the atmosphere. In addition, the solution proposed in the project can be implemented in all methane intakes in all mines using methane drainage.

- METHANE UTILIZATION – TOWARDS THE FUTURE

The average methane drainage efficiency of JSW S.A. mines is 35-40%. This indicator is most significantly affected by the effectiveness of methane drainage in mining workings. The effectiveness of methane drainage in the longwalls operated by JSW S.A. in 2024 was 59%. Currently, two methods of methane drainage in mining workings are used:

- Classic methane drainage – degassing using drainage holes 60 to 120 m long and 65-95 mm in diameter, made from wall galleries. The methane drainage efficiency achieved ranges from 20 to 60%.

- Methane drainage using a drainage gallery – this method allows for remarkably high demethanation efficiency of 80-90%. Due to its very high cost, it is rarely used, only in cases of extremely high absolute methane content in the wall and under favourable mining and geological conditions.

The natural development of methane drainage at JSW S.A. plants is the implementation of directional drilling technology. The use of directional drilling techniques and technologies will primarily increase the efficiency of longwall methane drainage. A directional borehole of a suitable diameter and length, with a horizontal or slightly inclined section, located appropriately in relation to the exploited seam, plays a key role and is comparable in concept to a drainage gallery. This technology is intended to replace classic methane drainage. Of course, the number of directional boreholes drilled in relation to the absolute methane content of the wall will determine the desired effectiveness of methane extraction. With this solution, the directional boreholes remain active behind the wall face, which further extends their operating time, ultimately transitioning to the post-mining methane extraction stage. This solution should increase the efficiency of methane removal to around 70%. Methane capture will therefore be intensified, which will have a measurable impact on improving work safety and reducing methane emissions into the atmosphere.

Methane drainage of longwall using directional drilling is the first pillar of the METH2GEN Project, which is being implemented at JSW by a consortium consisting of four research units in Poland: the Oil and Gas Research Institute, the Central Mining Institute-National Research Institute, the Polish Geological Institute, and the AGH University of Science and Technology, as well as two foreign entities: INSEMEX from Romania and the University of Oviedo from Spain. Methane drainage using directional drilling is an opportunity for the Company to implement modern technology, reduce costs, and upskilling its own personnel. Until now, the use of this method for methane drainage in mining workings has been limited due to the low supply of such services among Polish drilling companies. Therefore, building up our own expertise in this area will, in the long term, improve safety, reduce methane emissions into the atmosphere and reduce drainage costs at all JSW plants.

The second pillar of the METH2GEN project concerns the production of hydrogen from unused residual gas from methane drainage stations. What is residual gas? It is gas captured and unused in installations for the production of electricity and heat. For technological reasons, it is extremely difficult to fully use the methane captured in these installations. The METH2GEN project aims to solve this problem through the use of gas storage facilities. Gas that was released into the atmosphere for technological reasons will be captured in intermediate storage facilities and then sent to a purification plant to remove unwanted substances and gases, before finally reaching the hydrogen production plant. This is also extremely important in view of the provisions of the “Methane Regulation.”  According to Article 22(1), methane emissions from methane drainage stations are completely prohibited. Together with leading research institutes in Poland, we have decided to combine the solution to this problem with hydrogen production.

The hydrogen part of the project will be based on steam methane reforming (SMR) technology, which is a mature hydrogen production technology used worldwide. However, it has not yet been used to use gas from mine methane drainage. Steam methane reforming (SMR) is the most common and cheapest method of hydrogen production, but it is not environmentally friendly as it also generates CO₂.  The project involves the use of CO₂ as an inert gas. Carbon dioxide plays a key role in preventing endogenous fires in coal mines by creating an inert environment and reducing the risk of coal self-ignition. The planned hydrogen production is approximately 700 tons per year. In the face of the crisis in the coal mining industry in Europe, the unusual combination of the development of underground technologies—directional drilling—and an innovative solution—the production of hydrogen from mine gas—is a way to show that hard coal mines can operate in the era of decarbonisation.

- SUMMARY

Except European Union, hard coal mining continues to grow. At the same time, global awareness of environmental protection and climate change is increasing. Jastrzębska Spółka Węglowa, operating in Poland, which is a member of the European Union, is required to comply with EU regulations. In 2022, the Company adopted and is implementing a Business Strategy showing that there is room for coal production and sustainable development. The projects implemented by the Company combine improved safety, reduced methane emissions, and the economic use of methane. All of them are economically justified. At the same time, the solutions implemented in the presented projects can be successfully implemented in hard coal mines not only in Europe but also around the world. That is why these projects have gained recognition and received funding from the European Union, whose policy on reducing anthropogenic methane emissions is breaking new ground. The best example is the repeatedly cited Regulation of the European Parliament and of the Council on the reduction of methane emissions in the energy sector, known as the “Methane Regulation.”

INTEGRATING COMMINGLING INTO THE STRATEGIC WASTE ROCK PLANNING AT ANTAMINA MINE

Christa Quiroz1, David Machin1, Olimpia Cabrera1, Fernando Angeles1

1 Antamina Mine, Lima, Peru

(*Presenting author: cquirozc@antamina.com)

ABSTRACT

Antamina is a large open-pit polymetallic mine in Peru, operating with a processing capacity of 145 ktpd. In operation since 2001, the current mine life extends to 2036. Mining is conducted at a rate of 290 Mtpa using conventional truck-and-shovel equipment. This technical paper presents a comprehensive assessment of commingling as a transformative technology in mine planning, positioning waste rock and tailings management as a key factor in the decision-making process. The primary objective of this study is to demonstrate the benefits of this technology within the mine planning process, highlighting waste rock and tailings management as one of the main drivers in strategic decision-making. The methodology applied is a Comparative Case Analysis, which includes a review of the design and sequencing process for mining infrastructure such as waste rock storage facilities, along with a discussion of the key parameters considered. The results show that the implementation of this technology delivers significant economic benefits, including an approximate 30% reduction in capital costs for waste rock storage facilities, improved land-use efficiency, and optimized infrastructure development. Furthermore, the approach enhances alignment with the Global Industry Standard on Tailings Management (GISTM).

KEYWORDS

commingling, mine planning, waste rock management, tailings management, GISTM compliance

1	INTRODUCTION

The mining industry faces growing challenges related to the sustainable management of waste rock and tailings, especially after catastrophic events such as those that occurred in Brumadinho and Samarco (Green Policy Platform, 2024; ICMM, 2020). In this context, the Antamina mine, operating since 2001 as one of the largest polymetallic operations in the world, has developed a pioneering approach that integrates commingling as a central element of its strategic planning (Wheaton Precious Metals, 2015).

Commingling, defined as the homogeneous mixing of dewatered tailings with waste rock to form a geotechnically stable material, represents a natural evolution of traditional co-disposal practices (Burden & Wilson, 2023). However, what distinguishes the Antamina case is the integration of this technology from the initial phases of mine planning, making waste rock management the main driver of operational and strategic decisions.

2	OBJECTIVES

•	Demonstrate the benefits of integrating commingling into mine planning, positioning waste rock and tailings management as a strategic driver for decision-making in open-pit operations.

•	Optimize operational efficiency and environmental sustainability through the adoption of commingling in waste rock and tailings management.

•	Align waste rock and tailings management with international sustainability and safety standards, such as the GISTM, through the application of commingling.

•	Analyze, through comparative case studies, the impact of commingling on land-use optimization and mine life extension.

•	Reduce capital and operating costs associated with waste rock and tailings disposal by implementing commingling technologies.

•	Present the key findings and lessons learned from the integration of commingling into long-term mine planning.

3	TECHNICAL FOUNDATIONS OF COMMINGLING IN MINE PLANNING

3.1	Desing Principles and Material Characterization

Commingling at Antamina is based on engineering principles that seek to combine the superior structural properties of waste rock with the low permeability characteristics of tailings (Boshoff, 2023). This combination results in a material with shear strength similar to waste rock and permeability comparable to tailings, creating conditions that restrict oxygen entry and water filtration, significantly reducing the potential for acid drainage generation (Ulrich & Coffin, 2015; Burden & Wilson, 2023).

The characterization of commingled material is performed through analysis of five distinct structural configurations: rock-dominated uncompacted, fines-dominated uncompacted, rock-dominated compacted, fines-dominated compacted, and the "just-filled" configuration where fines exactly fill the voids between rock particles (Burden & Wilson, 2023). This latter configuration represents the optimal target of the commingling process at Antamina.

Figure 1 Blend configurations after Wickland et al. (2006). Source: Burden & Wilson (2023).

3.2	Integration into the Strategic Planning Process

The integration of commingling into Antamina's mine planning follows a structured methodological approach that places tailings management with waste rock as central elements of decision-making. This global strategy allows for optimizing the mixture of materials from the mine to the plant, considering the operational and environmental constraints associated with the tailings storage facility. By incorporating tailings behavior in the early stages of planning, greater efficiency in resource use and better risk management are achieved.

The global strategy is divided into three key axes:

•	Strategy 1: Ensure Future Expansions: The first strategy focuses on maximizing tailings storage capacity for current and future needs, leveraging Antamina's mineral resource and applying advanced technology to increase TSF (Tailings Storage Facility) capacity to ensure the mine's long-term operational continuity.

•	Strategy 2: Optimized footprint: The second strategy seeks to optimize the overall project footprint by balancing waste and tailings storage space. This approach aligns directly with commingling principles by maximizing efficient use of available land within the current operational footprint.

•	Strategy 3: Reduce upfront CAPEX: The third strategy focuses on reducing initial CAPEX to improve key financial indicators (IRR), which directly aligns with the demonstrated economic benefits of commingling, minimizing operational costs with a nearby TSF and optimizing CAPEX timing.

Figure 2 Strategic vision of mine planning approach with commingling

The strategies implemented in tailings dam and waste dump management seek to maximize the operational life of the mine and optimize asset value. These decisions translate into concrete actions that address sustainability, operational efficiency, and cost reduction, ensuring continuity and long-term success of the mining operation.

•	Long Life Asset: Antamina's vision as a long-life asset involves making decisions based on sustainable exploitation of its exceptional mineral resources. This includes adopting advanced technologies, such as commingling, to improve storage capacity and ensure operational continuity until 2036 and beyond, thus maximizing the value of the mineral resource.

•	Reduce Environmental Social Footprint: The strategic approach prioritizes reducing environmental and social impact through operation within the current footprint, increasing water availability through filtration, optimizing space use, and minimizing risks associated with tailings dams. Commingling contributes not only to technical and economic efficiency but also to sustainability objectives and corporate social responsibility.

•	Reduce Capital and Operational Cost: The cost reduction strategy focuses on optimizing capital investments (CAPEX) and their schedule, as well as minimizing operational costs through strategic facility location and leveraging synergies to avoid duplicate infrastructure. This enables more efficient and profitable resource management.

During the strategic planning phase, conceptual foundations for commingling implementation are established, including site characterization, regulatory framework development, and technical-economic feasibility analysis.

4	ANALYSIS OF THE MINING SYSTEM WITH COMMINGLING APPLICATION

4.1	Methodological Process Structure

The methodological process is organized in sequential stages that ensure effective integration of commingling into mine planning:

•	Comprehensive material evaluation: Detailed analysis of geotechnical and geochemical characteristics of available materials, identifying their suitability for mixtures and joint disposal.

•	Mixture design development: Design of optimized mixtures that maximize desired properties of commingled material, such as low permeability and high stability, minimizing acid water generation and improving mechanical behavior of the deposit.

•	Pilot test implementation: Execution of pilot and industrial scale tests to validate mixture performance and adjust design parameters according to results obtained under real operating conditions.

•	Monitoring and adjustment: Establishment of continuous monitoring system to evaluate consolidation, stability, and geochemical behavior of deposits, allowing operational and design adjustments as new information is collected.

5	IMPLEMENTATION AND EVALUATION METHODOLOGY FOR COMMINGLING IN MINE PLANNING

5.1	Comparative Case Analysis

The methodology applied at Antamina is based on Comparative Case Analysis, allowing for comprehensive review of the design and sequencing process of mining infrastructure, with emphasis on dams, waste rock dumps, and integration of commingling technologies. This methodological approach comprises:

•	Review of design and sequencing processes: Analysis of procedures used in waste dump planning and disposal, considering both geotechnical and geochemical criteria to ensure infrastructure stability and sustainability.

•	Identification of key variables: Selection of fundamental operational parameters, such as granulometry, permeability, geochemical behavior, and storage capacity, which directly affect the viability and performance of dumps and material mixtures.

•	Definition of case tree: Structure of a decision tree that contemplates long-term strategic scenarios, allowing comparison of alternatives under different combinations of variables and operational constraints.

•	Results evaluation: Application of sensitivity analyses and cost-benefit evaluations to determine the impact of each alternative on mine life extension, environmental risk reduction, and resource optimization.

5.2	Decision Tree for Strategic Planning

The decision tree developed for long-term strategic planning at Antamina follows sequential logic that allows evaluation of feasibility and impact of main operational alternatives:

•	Commingling feasibility: The first node of the tree determines whether integration of tailings and waste rock is technically and economically viable, considering material characteristics and regulatory constraints.

•	Entry into new area: If commingling is feasible, the next node evaluates the possibility of disposing material in a new area, analyzing topographic, environmental, and access aspects.

•	Need for new dam: Finally, it determines whether construction of a new tailings dam is essential or if the commingling solution allows dispensing with this infrastructure, optimizing investment and reducing environmental impact.

Each of these decisions conditions mine life extension, efficient space use, and environmental risk management, allowing selection of the alternative that maximizes economic, social, and environmental value of the project.

Figure 3  Decision Tree for Long-Term Strategic Scenario Planning

5.3	Evaluation of Operational Parameters and Modifications by Commingling

Key operational parameters—including waste rock-tailings ratio, mixing methods, transport systems, and processing criteria—are assessed for their impact on commingled system efficiency. The evaluation considers site-specific challenges such as extreme Andean weather, high-altitude logistics, and Peruvian regulations, all of which are especially critical at Antamina and require customized commingling solutions.

•	Modifications to tailings management by commingling:

The commingling process fundamentally transforms tailings management by utilizing voids within waste rock for storage, with available space typically equivalent to about 20% of the crushed rock volume transported by conveyors. To enable mixing, tailings must first be dewatered. The following image shows both the current process (without mechanization) and the transformation that occurs when commingling is incorporated.

Figure 4 Transformation of Tailings Management: Evolution from Conventional Process to Commingling|

•	 Operational parameters and assumptions

Commingling extends tailings facility life by optimizing spatial use of waste rock voids. Implementation requires mechanized systems (WCCS) for transporting crushed material mixed with dewatered tailings, while maintaining two key operational parameters: a) Unchanged transport capacity: 83.2 Mtpa in the conveyor system and b) Volumetric stability: No increase in total volume of waste rock dump.

This approach ensures efficient resource utilization and supports sustainable mine operations.

Table 1: Key operational parameters and assumptions

Component	Units	Without - Case Conventional	With - Case Commingling

Cycloned/Filtered Max. Capacity	Mtpa	-	32.0

Percentage of Voids in Waste Rock	% Volume 	 	20 %

Waste Rock / Tailings Ratio 	Mt Rock/

Mt Tails	-	83 / 16

Conventional Tailings Density	t/m3 	1.81	1.81

Fines Tailings Density	 t/m3	NA	1.60

Tailings Volume, TDR 4195 (to Dic-22)	Mm3	376	376

Tailings Capacity, TDR 4195 (to Dic-22)	Mt	682	652

Mill Feed Rate 	Ktpd	145	145

The following image shows how incorporation of mechanized systems and commingling add capacity, and therefore extend tailings facility life, to increase the life of mine and therefore the overall asset value.

Figure 5 Commingling Extension of Tailings Facility Life through Commingling

The curve below shows the relationship between dry density and optimal mixing proportion, determinant for maximizing commingling efficiency.

Figure 6: Dry density curve vs. optimal mixing proportion for commingling

•	Analysis of Mineable Resources, Production and Value

A three-dimensional diagram is used to strategically analyze the interactions between mineable resource volume, production rate, and generated value—whether economic, environmental, or operational. This tool helps identify how increasing resource volume and production can maximize value, but also highlights key constraints such as tailings dam capacity and property limits. The approach supports comprehensive decision-making to optimize asset value and ensure project sustainability across various operational scenarios.

Figure 7 Value Optimization: Interaction between Resource Volume, Production Rate and Key Restrictions

6	ECONOMIC AND OPERATIONAL ADVANTAGES

6.1	Perspective of Conventional Value Drivers

The following figure illustrates Antamina's main conventional value drivers from a financial perspective. In the current scenario, the amount of mineral resource that can be utilized directly depends on the capital that the company decides to invest (Capex), both in new investments (Growth Capex) and in maintaining existing operations (Sustaining Capex). This generates a necessary balance to determine the optimal scenario that maximizes the value of the existing asset.

Figure 8: Conventional Antamina Value Drivers: Financial Perspective (Without Commingling)

6.2	Impact of Commingling on Financial Value Drivers

From a financial perspective, the adoption of commingling among Antamina’s value drivers removes constraints on mineral resources by increasing tailings storage capacity, enabling the utilization of new dumps, and allowing for an expansion in the volume of usable resources. As a result, the mine’s operational life is extended and asset value is enhanced. Regarding dump utilization, the implementation of commingling has the potential to reduce the requirement for dumps located in karstic zones, which demand significant capital expenditure (Capex). This can lead to a reduction in initial investment by up to 30% in scenarios comparing commingling versus non-commingling approaches. This saving is achieved through the integration of previously separate infrastructures into a single system, which optimizes the use of available area in the East Extension dumps and prioritizes the use of mechanized systems over conventional trucking methods.

Figure 9: Antamina Value Drivers: Financial Perspective with Commingling

The optimization of land use represents another important economic benefit, especially relevant in mountainous locations like Antamina where available land for mining facilities is limited. Commingling allows for the creation of deposits with a smaller footprint compared to separate facilities, freeing up land for other productive uses or reducing the need for additional land acquisition.

7	ALIGNMENT WITH GISTM STANDARDS AND SUSTAINABILITY

The integration of commingling into mine planning strengthens compliance with the Global Industry Standard on Tailings Management (GISTM), by facilitating safer and more sustainable management of waste materials. The GISTM requires the planning, construction, operation, and closure of tailings facilities with a focus on risk reduction and continuous monitoring throughout the lifecycle. Commingling contributes to this objective by improving the geotechnical and geochemical stability of deposits, reducing the risk of failure and acid drainage generation, which in turn reduces environmental impact and long-term treatment costs. Additionally, the consolidation of facilities through commingling reduces the environmental footprint and facilitates supervision and control, aligning with the principles of sustainability and social responsibility in the mining industry.

8	LESSONS LEARNED AND CHALLENGES

Among the main lessons learned and challenges of implementing commingling in mine planning at Antamina, the following stand out:

•	Large-scale operations and mechanization: Commingling is primarily viable in large-volume mechanized operations, where it is possible to achieve a homogeneous and controlled mixture of materials. However, the magnitude of waste rock and tailings tonnages at Antamina implies a new challenge, never seen before.

•	Flexible planning and pilot tests: It is essential to include areas for pilot tests in short and medium-term plans, allowing adjustment of mixing parameters according to material variability.

•	Regulatory communication: Technology acceptance requires identifying precedents and working closely with authorities to develop appropriate regulatory frameworks.

•	Organizational change management: The transition to commingling involves challenges in personnel training and adaptation of operational processes, especially in mature operations.

9	CONCLUSIONS

•	The comparative discussion and case analysis of commingling implementation at Antamina have enabled the identification and selection of strategic alternatives that maximize the economic, environmental, and social value of the project.

•	The use of decision trees and scenario evaluation has facilitated informed decision-making, taking into account technical and economic feasibility as well as regulatory and environmental constraints.

•	The integration of commingling into strategic mine planning has proven to be a key tool for optimizing waste rock and tailings management, achieving a significant reduction in capital costs (up to 30%), greater efficiency in land use, and an extension of tailings storage facility (TSF) life. This approach has also aligned operations with international sustainability standards (GISTM) and strengthened risk management, contributing to operational continuity and long-term reduction of environmental impacts.

•	The Antamina experience demonstrates that structured case discussions and the application of comparative methodologies are fundamental for strategic planning, as they allow anticipation of challenges, validation of solutions through pilot testing, and adjustment of operational parameters based on real outcomes. Thus, commingling is consolidated as a transformative practice in modern mining, fostering more robust and sustainable decision-making for the development of long-life mining assets.

ACKNOWLEDGEMENTS

The authors express their sincere gratitude to Antamina for their invaluable support. Special thanks are extended to the Tailings Strategy Management team and the Long-Term Planning team for their collaboration and insights.

REFERENCES

Boshoff, J. et al. (2023). A case study on the commingling of tailings and waste rock at a Brownfields open cast mine in Ghana. University of Alberta.

Burden, R., & Wilson, G. W. (2023). Commingling of waste rock and tailings to improve “dry stack” performance: Design and evaluation of mixtures. Minerals, 13(2), 295.

Deza, N., & Montes, K. (2022). Mecanización del desmonte en Compañía Minera Antamina: Propuesta para capturar el valor tangible e intangible de la mecanización. Compañía Minera Antamina.

Green Policy Platform. (2024). Knowledge Gaps Report: Environmental Aspects of Tailings Management (January 2024).

ICMM. (2020). New global industry standard on tailings management. International Council on Mining and Metals.

Ulrich, B. & Coffin, J. (2015). TMW 2015 – Combined Tailings and Mine Waste.

Minemax. (2015). Why mine planning is all about collaboration. Minemax News.

Wheaton Precious Metals. (2015). Antamina Project overview.

INTEGRATING COMMINGLING INTO THE STRATEGIC WASTE ROCK PLANNING AT ANTAMINA MINE

Christa Quiroz1, David Machin1, Olimpia Cabrera1, Fernando Angeles1

1 Antamina Mine, Lima, Peru

(*Presenting author: cquirozc@antamina.com)

ABSTRACT

Antamina is a large open-pit polymetallic mine in Peru, operating with a processing capacity of 145 ktpd. In operation since 2001, the current mine life extends to 2036. Mining is conducted at a rate of 290 Mtpa using conventional truck-and-shovel equipment. This technical paper presents a comprehensive assessment of commingling as a transformative technology in mine planning, positioning waste rock and tailings management as a key factor in the decision-making process. The primary objective of this study is to demonstrate the benefits of this technology within the mine planning process, highlighting waste rock and tailings management as one of the main drivers in strategic decision-making. The methodology applied is a Comparative Case Analysis, which includes a review of the design and sequencing process for mining infrastructure such as waste rock storage facilities, along with a discussion of the key parameters considered. The results show that the implementation of this technology delivers significant economic benefits, including an approximate 30% reduction in capital costs for waste rock storage facilities, improved land-use efficiency, and optimized infrastructure development. Furthermore, the approach enhances alignment with the Global Industry Standard on Tailings Management (GISTM).

KEYWORDS

commingling, mine planning, waste rock management, tailings management, GISTM compliance

10	INTRODUCTION

The mining industry faces growing challenges related to the sustainable management of waste rock and tailings, especially after catastrophic events such as those that occurred in Brumadinho and Samarco (Green Policy Platform, 2024; ICMM, 2020). In this context, the Antamina mine, operating since 2001 as one of the largest polymetallic operations in the world, has developed a pioneering approach that integrates commingling as a central element of its strategic planning (Wheaton Precious Metals, 2015).

Commingling, defined as the homogeneous mixing of dewatered tailings with waste rock to form a geotechnically stable material, represents a natural evolution of traditional co-disposal practices (Burden & Wilson, 2023). However, what distinguishes the Antamina case is the integration of this technology from the initial phases of mine planning, making waste rock management the main driver of operational and strategic decisions.

11	OBJECTIVES

•	Demonstrate the benefits of integrating commingling into mine planning, positioning waste rock and tailings management as a strategic driver for decision-making in open-pit operations.

•	Optimize operational efficiency and environmental sustainability through the adoption of commingling in waste rock and tailings management.

•	Align waste rock and tailings management with international sustainability and safety standards, such as the GISTM, through the application of commingling.

•	Analyze, through comparative case studies, the impact of commingling on land-use optimization and mine life extension.

•	Reduce capital and operating costs associated with waste rock and tailings disposal by implementing commingling technologies.

•	Present the key findings and lessons learned from the integration of commingling into long-term mine planning.

12	TECHNICAL FOUNDATIONS OF COMMINGLING IN MINE PLANNING

12.1	Desing Principles and Material Characterization

Commingling at Antamina is based on engineering principles that seek to combine the superior structural properties of waste rock with the low permeability characteristics of tailings (Boshoff, 2023). This combination results in a material with shear strength similar to waste rock and permeability comparable to tailings, creating conditions that restrict oxygen entry and water filtration, significantly reducing the potential for acid drainage generation (Ulrich & Coffin, 2015; Burden & Wilson, 2023).

The characterization of commingled material is performed through analysis of five distinct structural configurations: rock-dominated uncompacted, fines-dominated uncompacted, rock-dominated compacted, fines-dominated compacted, and the "just-filled" configuration where fines exactly fill the voids between rock particles (Burden & Wilson, 2023). This latter configuration represents the optimal target of the commingling process at Antamina.

Figure 1 Blend configurations after Wickland et al. (2006). Source: Burden & Wilson (2023).

12.2	Integration into the Strategic Planning Process

The integration of commingling into Antamina's mine planning follows a structured methodological approach that places tailings management with waste rock as central elements of decision-making. This global strategy allows for optimizing the mixture of materials from the mine to the plant, considering the operational and environmental constraints associated with the tailings storage facility. By incorporating tailings behavior in the early stages of planning, greater efficiency in resource use and better risk management are achieved.

The global strategy is divided into three key axes:

•	Strategy 1: Ensure Future Expansions: The first strategy focuses on maximizing tailings storage capacity for current and future needs, leveraging Antamina's mineral resource and applying advanced technology to increase TSF (Tailings Storage Facility) capacity to ensure the mine's long-term operational continuity.

•	Strategy 2: Optimized footprint: The second strategy seeks to optimize the overall project footprint by balancing waste and tailings storage space. This approach aligns directly with commingling principles by maximizing efficient use of available land within the current operational footprint.

•	Strategy 3: Reduce upfront CAPEX: The third strategy focuses on reducing initial CAPEX to improve key financial indicators (IRR), which directly aligns with the demonstrated economic benefits of commingling, minimizing operational costs with a nearby TSF and optimizing CAPEX timing.

Figure 2 Strategic vision of mine planning approach with commingling

The strategies implemented in tailings dam and waste dump management seek to maximize the operational life of the mine and optimize asset value. These decisions translate into concrete actions that address sustainability, operational efficiency, and cost reduction, ensuring continuity and long-term success of the mining operation.

•	Long Life Asset: Antamina's vision as a long-life asset involves making decisions based on sustainable exploitation of its exceptional mineral resources. This includes adopting advanced technologies, such as commingling, to improve storage capacity and ensure operational continuity until 2036 and beyond, thus maximizing the value of the mineral resource.

•	Reduce Environmental Social Footprint: The strategic approach prioritizes reducing environmental and social impact through operation within the current footprint, increasing water availability through filtration, optimizing space use, and minimizing risks associated with tailings dams. Commingling contributes not only to technical and economic efficiency but also to sustainability objectives and corporate social responsibility.

•	Reduce Capital and Operational Cost: The cost reduction strategy focuses on optimizing capital investments (CAPEX) and their schedule, as well as minimizing operational costs through strategic facility location and leveraging synergies to avoid duplicate infrastructure. This enables more efficient and profitable resource management.

During the strategic planning phase, conceptual foundations for commingling implementation are established, including site characterization, regulatory framework development, and technical-economic feasibility analysis.

13	ANALYSIS OF THE MINING SYSTEM WITH COMMINGLING APPLICATION

13.1	Methodological Process Structure

The methodological process is organized in sequential stages that ensure effective integration of commingling into mine planning:

•	Comprehensive material evaluation: Detailed analysis of geotechnical and geochemical characteristics of available materials, identifying their suitability for mixtures and joint disposal.

•	Mixture design development: Design of optimized mixtures that maximize desired properties of commingled material, such as low permeability and high stability, minimizing acid water generation and improving mechanical behavior of the deposit.

•	Pilot test implementation: Execution of pilot and industrial scale tests to validate mixture performance and adjust design parameters according to results obtained under real operating conditions.

•	Monitoring and adjustment: Establishment of continuous monitoring system to evaluate consolidation, stability, and geochemical behavior of deposits, allowing operational and design adjustments as new information is collected.

14	IMPLEMENTATION AND EVALUATION METHODOLOGY FOR COMMINGLING IN MINE PLANNING

14.1	Comparative Case Analysis

The methodology applied at Antamina is based on Comparative Case Analysis, allowing for comprehensive review of the design and sequencing process of mining infrastructure, with emphasis on dams, waste rock dumps, and integration of commingling technologies. This methodological approach comprises:

•	Review of design and sequencing processes: Analysis of procedures used in waste dump planning and disposal, considering both geotechnical and geochemical criteria to ensure infrastructure stability and sustainability.

•	Identification of key variables: Selection of fundamental operational parameters, such as granulometry, permeability, geochemical behavior, and storage capacity, which directly affect the viability and performance of dumps and material mixtures.

•	Definition of case tree: Structure of a decision tree that contemplates long-term strategic scenarios, allowing comparison of alternatives under different combinations of variables and operational constraints.

•	Results evaluation: Application of sensitivity analyses and cost-benefit evaluations to determine the impact of each alternative on mine life extension, environmental risk reduction, and resource optimization.

14.2	Decision Tree for Strategic Planning

The decision tree developed for long-term strategic planning at Antamina follows sequential logic that allows evaluation of feasibility and impact of main operational alternatives:

•	Commingling feasibility: The first node of the tree determines whether integration of tailings and waste rock is technically and economically viable, considering material characteristics and regulatory constraints.

•	Entry into new area: If commingling is feasible, the next node evaluates the possibility of disposing material in a new area, analyzing topographic, environmental, and access aspects.

•	Need for new dam: Finally, it determines whether construction of a new tailings dam is essential or if the commingling solution allows dispensing with this infrastructure, optimizing investment and reducing environmental impact.

Each of these decisions conditions mine life extension, efficient space use, and environmental risk management, allowing selection of the alternative that maximizes economic, social, and environmental value of the project.

Figure 3  Decision Tree for Long-Term Strategic Scenario Planning

14.3	Evaluation of Operational Parameters and Modifications by Commingling

Key operational parameters—including waste rock-tailings ratio, mixing methods, transport systems, and processing criteria—are assessed for their impact on commingled system efficiency. The evaluation considers site-specific challenges such as extreme Andean weather, high-altitude logistics, and Peruvian regulations, all of which are especially critical at Antamina and require customized commingling solutions.

•	Modifications to tailings management by commingling:

The commingling process fundamentally transforms tailings management by utilizing voids within waste rock for storage, with available space typically equivalent to about 20% of the crushed rock volume transported by conveyors. To enable mixing, tailings must first be dewatered. The following image shows both the current process (without mechanization) and the transformation that occurs when commingling is incorporated.

Figure 4 Transformation of Tailings Management: Evolution from Conventional Process to Commingling|

•	 Operational parameters and assumptions

Commingling extends tailings facility life by optimizing spatial use of waste rock voids. Implementation requires mechanized systems (WCCS) for transporting crushed material mixed with dewatered tailings, while maintaining two key operational parameters: a) Unchanged transport capacity: 83.2 Mtpa in the conveyor system and b) Volumetric stability: No increase in total volume of waste rock dump.

This approach ensures efficient resource utilization and supports sustainable mine operations.

Table 1: Key operational parameters and assumptions

Component	Units	Without - Case Conventional	With - Case Commingling

Cycloned/Filtered Max. Capacity	Mtpa	-	32.0

Percentage of Voids in Waste Rock	% Volume 	 	20 %

Waste Rock / Tailings Ratio 	Mt Rock/

Mt Tails	-	83 / 16

Conventional Tailings Density	t/m3 	1.81	1.81

Fines Tailings Density	 t/m3	NA	1.60

Tailings Volume, TDR 4195 (to Dic-22)	Mm3	376	376

Tailings Capacity, TDR 4195 (to Dic-22)	Mt	682	652

Mill Feed Rate 	Ktpd	145	145

The following image shows how incorporation of mechanized systems and commingling add capacity, and therefore extend tailings facility life, to increase the life of mine and therefore the overall asset value.

Figure 5 Commingling Extension of Tailings Facility Life through Commingling

The curve below shows the relationship between dry density and optimal mixing proportion, determinant for maximizing commingling efficiency.

Figure 6: Dry density curve vs. optimal mixing proportion for commingling

•	Analysis of Mineable Resources, Production and Value

A three-dimensional diagram is used to strategically analyze the interactions between mineable resource volume, production rate, and generated value—whether economic, environmental, or operational. This tool helps identify how increasing resource volume and production can maximize value, but also highlights key constraints such as tailings dam capacity and property limits. The approach supports comprehensive decision-making to optimize asset value and ensure project sustainability across various operational scenarios.

Figure 7 Value Optimization: Interaction between Resource Volume, Production Rate and Key Restrictions

15	ECONOMIC AND OPERATIONAL ADVANTAGES

15.1	Perspective of Conventional Value Drivers

The following figure illustrates Antamina's main conventional value drivers from a financial perspective. In the current scenario, the amount of mineral resource that can be utilized directly depends on the capital that the company decides to invest (Capex), both in new investments (Growth Capex) and in maintaining existing operations (Sustaining Capex). This generates a necessary balance to determine the optimal scenario that maximizes the value of the existing asset.

Figure 8: Conventional Antamina Value Drivers: Financial Perspective (Without Commingling)

15.2	Impact of Commingling on Financial Value Drivers

From a financial perspective, the adoption of commingling among Antamina’s value drivers removes constraints on mineral resources by increasing tailings storage capacity, enabling the utilization of new dumps, and allowing for an expansion in the volume of usable resources. As a result, the mine’s operational life is extended and asset value is enhanced. Regarding dump utilization, the implementation of commingling has the potential to reduce the requirement for dumps located in karstic zones, which demand significant capital expenditure (Capex). This can lead to a reduction in initial investment by up to 30% in scenarios comparing commingling versus non-commingling approaches. This saving is achieved through the integration of previously separate infrastructures into a single system, which optimizes the use of available area in the East Extension dumps and prioritizes the use of mechanized systems over conventional trucking methods.

Figure 9: Antamina Value Drivers: Financial Perspective with Commingling

The optimization of land use represents another important economic benefit, especially relevant in mountainous locations like Antamina where available land for mining facilities is limited. Commingling allows for the creation of deposits with a smaller footprint compared to separate facilities, freeing up land for other productive uses or reducing the need for additional land acquisition.

16	ALIGNMENT WITH GISTM STANDARDS AND SUSTAINABILITY

The integration of commingling into mine planning strengthens compliance with the Global Industry Standard on Tailings Management (GISTM), by facilitating safer and more sustainable management of waste materials. The GISTM requires the planning, construction, operation, and closure of tailings facilities with a focus on risk reduction and continuous monitoring throughout the lifecycle. Commingling contributes to this objective by improving the geotechnical and geochemical stability of deposits, reducing the risk of failure and acid drainage generation, which in turn reduces environmental impact and long-term treatment costs. Additionally, the consolidation of facilities through commingling reduces the environmental footprint and facilitates supervision and control, aligning with the principles of sustainability and social responsibility in the mining industry.

17	LESSONS LEARNED AND CHALLENGES

Among the main lessons learned and challenges of implementing commingling in mine planning at Antamina, the following stand out:

•	Large-scale operations and mechanization: Commingling is primarily viable in large-volume mechanized operations, where it is possible to achieve a homogeneous and controlled mixture of materials. However, the magnitude of waste rock and tailings tonnages at Antamina implies a new challenge, never seen before.

•	Flexible planning and pilot tests: It is essential to include areas for pilot tests in short and medium-term plans, allowing adjustment of mixing parameters according to material variability.

•	Regulatory communication: Technology acceptance requires identifying precedents and working closely with authorities to develop appropriate regulatory frameworks.

•	Organizational change management: The transition to commingling involves challenges in personnel training and adaptation of operational processes, especially in mature operations.

18	CONCLUSIONS

•	The comparative discussion and case analysis of commingling implementation at Antamina have enabled the identification and selection of strategic alternatives that maximize the economic, environmental, and social value of the project.

•	The use of decision trees and scenario evaluation has facilitated informed decision-making, taking into account technical and economic feasibility as well as regulatory and environmental constraints.

•	The integration of commingling into strategic mine planning has proven to be a key tool for optimizing waste rock and tailings management, achieving a significant reduction in capital costs (up to 30%), greater efficiency in land use, and an extension of tailings storage facility (TSF) life. This approach has also aligned operations with international sustainability standards (GISTM) and strengthened risk management, contributing to operational continuity and long-term reduction of environmental impacts.

•	The Antamina experience demonstrates that structured case discussions and the application of comparative methodologies are fundamental for strategic planning, as they allow anticipation of challenges, validation of solutions through pilot testing, and adjustment of operational parameters based on real outcomes. Thus, commingling is consolidated as a transformative practice in modern mining, fostering more robust and sustainable decision-making for the development of long-life mining assets.

ACKNOWLEDGEMENTS

The authors express their sincere gratitude to Antamina for their invaluable support. Special thanks are extended to the Tailings Strategy Management team and the Long-Term Planning team for their collaboration and insights.

REFERENCES

Boshoff, J. et al. (2023). A case study on the commingling of tailings and waste rock at a Brownfields open cast mine in Ghana. University of Alberta.

Burden, R., & Wilson, G. W. (2023). Commingling of waste rock and tailings to improve “dry stack” performance: Design and evaluation of mixtures. Minerals, 13(2), 295.

Deza, N., & Montes, K. (2022). Mecanización del desmonte en Compañía Minera Antamina: Propuesta para capturar el valor tangible e intangible de la mecanización. Compañía Minera Antamina.

Green Policy Platform. (2024). Knowledge Gaps Report: Environmental Aspects of Tailings Management (January 2024).

ICMM. (2020). New global industry standard on tailings management. International Council on Mining and Metals.

Ulrich, B. & Coffin, J. (2015). TMW 2015 – Combined Tailings and Mine Waste.

Minemax. (2015). Why mine planning is all about collaboration. Minemax News.

Wheaton Precious Metals. (2015). Antamina Project overview.

INTEGRATING COMMINGLING INTO THE STRATEGIC WASTE ROCK PLANNING AT ANTAMINA MINE

Christa Quiroz1, David Machin1, Olimpia Cabrera1, Fernando Angeles1

1 Antamina Mine, Lima, Peru

(*Presenting author: cquirozc@antamina.com)

ABSTRACT

Antamina is a large open-pit polymetallic mine in Peru, operating with a processing capacity of 145 ktpd. In operation since 2001, the current mine life extends to 2036. Mining is conducted at a rate of 290 Mtpa using conventional truck-and-shovel equipment. This technical paper presents a comprehensive assessment of commingling as a transformative technology in mine planning, positioning waste rock and tailings management as a key factor in the decision-making process. The primary objective of this study is to demonstrate the benefits of this technology within the mine planning process, highlighting waste rock and tailings management as one of the main drivers in strategic decision-making. The methodology applied is a Comparative Case Analysis, which includes a review of the design and sequencing process for mining infrastructure such as waste rock storage facilities, along with a discussion of the key parameters considered. The results show that the implementation of this technology delivers significant economic benefits, including an approximate 30% reduction in capital costs for waste rock storage facilities, improved land-use efficiency, and optimized infrastructure development. Furthermore, the approach enhances alignment with the Global Industry Standard on Tailings Management (GISTM).

KEYWORDS

commingling, mine planning, waste rock management, tailings management, GISTM compliance

1	INTRODUCTION

The mining industry faces growing challenges related to the sustainable management of waste rock and tailings, especially after catastrophic events such as those that occurred in Brumadinho and Samarco (Green Policy Platform, 2024; ICMM, 2020). In this context, the Antamina mine, operating since 2001 as one of the largest polymetallic operations in the world, has developed a pioneering approach that integrates commingling as a central element of its strategic planning (Wheaton Precious Metals, 2015).

Commingling, defined as the homogeneous mixing of dewatered tailings with waste rock to form a geotechnically stable material, represents a natural evolution of traditional co-disposal practices (Burden & Wilson, 2023). However, what distinguishes the Antamina case is the integration of this technology from the initial phases of mine planning, making waste rock management the main driver of operational and strategic decisions.

2	OBJECTIVES

•	Demonstrate the benefits of integrating commingling into mine planning, positioning waste rock and tailings management as a strategic driver for decision-making in open-pit operations.

•	Optimize operational efficiency and environmental sustainability through the adoption of commingling in waste rock and tailings management.

•	Align waste rock and tailings management with international sustainability and safety standards, such as the GISTM, through the application of commingling.

•	Analyze, through comparative case studies, the impact of commingling on land-use optimization and mine life extension.

•	Reduce capital and operating costs associated with waste rock and tailings disposal by implementing commingling technologies.

•	Present the key findings and lessons learned from the integration of commingling into long-term mine planning.

3	TECHNICAL FOUNDATIONS OF COMMINGLING IN MINE PLANNING

3.1	Desing Principles and Material Characterization

Commingling at Antamina is based on engineering principles that seek to combine the superior structural properties of waste rock with the low permeability characteristics of tailings (Boshoff, 2023). This combination results in a material with shear strength similar to waste rock and permeability comparable to tailings, creating conditions that restrict oxygen entry and water filtration, significantly reducing the potential for acid drainage generation (Ulrich & Coffin, 2015; Burden & Wilson, 2023).

The characterization of commingled material is performed through analysis of five distinct structural configurations: rock-dominated uncompacted, fines-dominated uncompacted, rock-dominated compacted, fines-dominated compacted, and the "just-filled" configuration where fines exactly fill the voids between rock particles (Burden & Wilson, 2023). This latter configuration represents the optimal target of the commingling process at Antamina.

Figure 1 Blend configurations after Wickland et al. (2006). Source: Burden & Wilson (2023).

3.2	Integration into the Strategic Planning Process

The integration of commingling into Antamina's mine planning follows a structured methodological approach that places tailings management with waste rock as central elements of decision-making. This global strategy allows for optimizing the mixture of materials from the mine to the plant, considering the operational and environmental constraints associated with the tailings storage facility. By incorporating tailings behavior in the early stages of planning, greater efficiency in resource use and better risk management are achieved.

The global strategy is divided into three key axes:

•	Strategy 1: Ensure Future Expansions: The first strategy focuses on maximizing tailings storage capacity for current and future needs, leveraging Antamina's mineral resource and applying advanced technology to increase TSF (Tailings Storage Facility) capacity to ensure the mine's long-term operational continuity.

•	Strategy 2: Optimized footprint: The second strategy seeks to optimize the overall project footprint by balancing waste and tailings storage space. This approach aligns directly with commingling principles by maximizing efficient use of available land within the current operational footprint.

•	Strategy 3: Reduce upfront CAPEX: The third strategy focuses on reducing initial CAPEX to improve key financial indicators (IRR), which directly aligns with the demonstrated economic benefits of commingling, minimizing operational costs with a nearby TSF and optimizing CAPEX timing.

Figure 2 Strategic vision of mine planning approach with commingling

The strategies implemented in tailings dam and waste dump management seek to maximize the operational life of the mine and optimize asset value. These decisions translate into concrete actions that address sustainability, operational efficiency, and cost reduction, ensuring continuity and long-term success of the mining operation.

•	Long Life Asset: Antamina's vision as a long-life asset involves making decisions based on sustainable exploitation of its exceptional mineral resources. This includes adopting advanced technologies, such as commingling, to improve storage capacity and ensure operational continuity until 2036 and beyond, thus maximizing the value of the mineral resource.

•	Reduce Environmental Social Footprint: The strategic approach prioritizes reducing environmental and social impact through operation within the current footprint, increasing water availability through filtration, optimizing space use, and minimizing risks associated with tailings dams. Commingling contributes not only to technical and economic efficiency but also to sustainability objectives and corporate social responsibility.

•	Reduce Capital and Operational Cost: The cost reduction strategy focuses on optimizing capital investments (CAPEX) and their schedule, as well as minimizing operational costs through strategic facility location and leveraging synergies to avoid duplicate infrastructure. This enables more efficient and profitable resource management.

During the strategic planning phase, conceptual foundations for commingling implementation are established, including site characterization, regulatory framework development, and technical-economic feasibility analysis.

4	ANALYSIS OF THE MINING SYSTEM WITH COMMINGLING APPLICATION

4.1	Methodological Process Structure

The methodological process is organized in sequential stages that ensure effective integration of commingling into mine planning:

•	Comprehensive material evaluation: Detailed analysis of geotechnical and geochemical characteristics of available materials, identifying their suitability for mixtures and joint disposal.

•	Mixture design development: Design of optimized mixtures that maximize desired properties of commingled material, such as low permeability and high stability, minimizing acid water generation and improving mechanical behavior of the deposit.

•	Pilot test implementation: Execution of pilot and industrial scale tests to validate mixture performance and adjust design parameters according to results obtained under real operating conditions.

•	Monitoring and adjustment: Establishment of continuous monitoring system to evaluate consolidation, stability, and geochemical behavior of deposits, allowing operational and design adjustments as new information is collected.

5	IMPLEMENTATION AND EVALUATION METHODOLOGY FOR COMMINGLING IN MINE PLANNING

5.1	Comparative Case Analysis

The methodology applied at Antamina is based on Comparative Case Analysis, allowing for comprehensive review of the design and sequencing process of mining infrastructure, with emphasis on dams, waste rock dumps, and integration of commingling technologies. This methodological approach comprises:

•	Review of design and sequencing processes: Analysis of procedures used in waste dump planning and disposal, considering both geotechnical and geochemical criteria to ensure infrastructure stability and sustainability.

•	Identification of key variables: Selection of fundamental operational parameters, such as granulometry, permeability, geochemical behavior, and storage capacity, which directly affect the viability and performance of dumps and material mixtures.

•	Definition of case tree: Structure of a decision tree that contemplates long-term strategic scenarios, allowing comparison of alternatives under different combinations of variables and operational constraints.

•	Results evaluation: Application of sensitivity analyses and cost-benefit evaluations to determine the impact of each alternative on mine life extension, environmental risk reduction, and resource optimization.

5.2	Decision Tree for Strategic Planning

The decision tree developed for long-term strategic planning at Antamina follows sequential logic that allows evaluation of feasibility and impact of main operational alternatives:

•	Commingling feasibility: The first node of the tree determines whether integration of tailings and waste rock is technically and economically viable, considering material characteristics and regulatory constraints.

•	Entry into new area: If commingling is feasible, the next node evaluates the possibility of disposing material in a new area, analyzing topographic, environmental, and access aspects.

•	Need for new dam: Finally, it determines whether construction of a new tailings dam is essential or if the commingling solution allows dispensing with this infrastructure, optimizing investment and reducing environmental impact.

Each of these decisions conditions mine life extension, efficient space use, and environmental risk management, allowing selection of the alternative that maximizes economic, social, and environmental value of the project.

Figure 3  Decision Tree for Long-Term Strategic Scenario Planning

5.3	Evaluation of Operational Parameters and Modifications by Commingling

Key operational parameters—including waste rock-tailings ratio, mixing methods, transport systems, and processing criteria—are assessed for their impact on commingled system efficiency. The evaluation considers site-specific challenges such as extreme Andean weather, high-altitude logistics, and Peruvian regulations, all of which are especially critical at Antamina and require customized commingling solutions.

•	Modifications to tailings management by commingling:

The commingling process fundamentally transforms tailings management by utilizing voids within waste rock for storage, with available space typically equivalent to about 20% of the crushed rock volume transported by conveyors. To enable mixing, tailings must first be dewatered. The following image shows both the current process (without mechanization) and the transformation that occurs when commingling is incorporated.

Figure 4 Transformation of Tailings Management: Evolution from Conventional Process to Commingling|

•	 Operational parameters and assumptions

Commingling extends tailings facility life by optimizing spatial use of waste rock voids. Implementation requires mechanized systems (WCCS) for transporting crushed material mixed with dewatered tailings, while maintaining two key operational parameters: a) Unchanged transport capacity: 83.2 Mtpa in the conveyor system and b) Volumetric stability: No increase in total volume of waste rock dump.

This approach ensures efficient resource utilization and supports sustainable mine operations.

Table 1: Key operational parameters and assumptions

Component	Units	Without - Case Conventional	With - Case Commingling

Cycloned/Filtered Max. Capacity	Mtpa	-	32.0

Percentage of Voids in Waste Rock	% Volume 	 	20 %

Waste Rock / Tailings Ratio 	Mt Rock/

Mt Tails	-	83 / 16

Conventional Tailings Density	t/m3 	1.81	1.81

Fines Tailings Density	 t/m3	NA	1.60

Tailings Volume, TDR 4195 (to Dic-22)	Mm3	376	376

Tailings Capacity, TDR 4195 (to Dic-22)	Mt	682	652

Mill Feed Rate 	Ktpd	145	145

The following image shows how incorporation of mechanized systems and commingling add capacity, and therefore extend tailings facility life, to increase the life of mine and therefore the overall asset value.

Figure 5 Commingling Extension of Tailings Facility Life through Commingling

The curve below shows the relationship between dry density and optimal mixing proportion, determinant for maximizing commingling efficiency.

Figure 6: Dry density curve vs. optimal mixing proportion for commingling

•	Analysis of Mineable Resources, Production and Value

A three-dimensional diagram is used to strategically analyze the interactions between mineable resource volume, production rate, and generated value—whether economic, environmental, or operational. This tool helps identify how increasing resource volume and production can maximize value, but also highlights key constraints such as tailings dam capacity and property limits. The approach supports comprehensive decision-making to optimize asset value and ensure project sustainability across various operational scenarios.

Figure 7 Value Optimization: Interaction between Resource Volume, Production Rate and Key Restrictions

6	ECONOMIC AND OPERATIONAL ADVANTAGES

6.1	Perspective of Conventional Value Drivers

The following figure illustrates Antamina's main conventional value drivers from a financial perspective. In the current scenario, the amount of mineral resource that can be utilized directly depends on the capital that the company decides to invest (Capex), both in new investments (Growth Capex) and in maintaining existing operations (Sustaining Capex). This generates a necessary balance to determine the optimal scenario that maximizes the value of the existing asset.

Figure 8: Conventional Antamina Value Drivers: Financial Perspective (Without Commingling)

6.2	Impact of Commingling on Financial Value Drivers

From a financial perspective, the adoption of commingling among Antamina’s value drivers removes constraints on mineral resources by increasing tailings storage capacity, enabling the utilization of new dumps, and allowing for an expansion in the volume of usable resources. As a result, the mine’s operational life is extended and asset value is enhanced. Regarding dump utilization, the implementation of commingling has the potential to reduce the requirement for dumps located in karstic zones, which demand significant capital expenditure (Capex). This can lead to a reduction in initial investment by up to 30% in scenarios comparing commingling versus non-commingling approaches. This saving is achieved through the integration of previously separate infrastructures into a single system, which optimizes the use of available area in the East Extension dumps and prioritizes the use of mechanized systems over conventional trucking methods.

Figure 9: Antamina Value Drivers: Financial Perspective with Commingling

The optimization of land use represents another important economic benefit, especially relevant in mountainous locations like Antamina where available land for mining facilities is limited. Commingling allows for the creation of deposits with a smaller footprint compared to separate facilities, freeing up land for other productive uses or reducing the need for additional land acquisition.

7	ALIGNMENT WITH GISTM STANDARDS AND SUSTAINABILITY

The integration of commingling into mine planning strengthens compliance with the Global Industry Standard on Tailings Management (GISTM), by facilitating safer and more sustainable management of waste materials. The GISTM requires the planning, construction, operation, and closure of tailings facilities with a focus on risk reduction and continuous monitoring throughout the lifecycle. Commingling contributes to this objective by improving the geotechnical and geochemical stability of deposits, reducing the risk of failure and acid drainage generation, which in turn reduces environmental impact and long-term treatment costs. Additionally, the consolidation of facilities through commingling reduces the environmental footprint and facilitates supervision and control, aligning with the principles of sustainability and social responsibility in the mining industry.

8	LESSONS LEARNED AND CHALLENGES

Among the main lessons learned and challenges of implementing commingling in mine planning at Antamina, the following stand out:

•	Large-scale operations and mechanization: Commingling is primarily viable in large-volume mechanized operations, where it is possible to achieve a homogeneous and controlled mixture of materials. However, the magnitude of waste rock and tailings tonnages at Antamina implies a new challenge, never seen before.

•	Flexible planning and pilot tests: It is essential to include areas for pilot tests in short and medium-term plans, allowing adjustment of mixing parameters according to material variability.

•	Regulatory communication: Technology acceptance requires identifying precedents and working closely with authorities to develop appropriate regulatory frameworks.

•	Organizational change management: The transition to commingling involves challenges in personnel training and adaptation of operational processes, especially in mature operations.

9	CONCLUSIONS

•	The comparative discussion and case analysis of commingling implementation at Antamina have enabled the identification and selection of strategic alternatives that maximize the economic, environmental, and social value of the project.

•	The use of decision trees and scenario evaluation has facilitated informed decision-making, taking into account technical and economic feasibility as well as regulatory and environmental constraints.

•	The integration of commingling into strategic mine planning has proven to be a key tool for optimizing waste rock and tailings management, achieving a significant reduction in capital costs (up to 30%), greater efficiency in land use, and an extension of tailings storage facility (TSF) life. This approach has also aligned operations with international sustainability standards (GISTM) and strengthened risk management, contributing to operational continuity and long-term reduction of environmental impacts.

•	The Antamina experience demonstrates that structured case discussions and the application of comparative methodologies are fundamental for strategic planning, as they allow anticipation of challenges, validation of solutions through pilot testing, and adjustment of operational parameters based on real outcomes. Thus, commingling is consolidated as a transformative practice in modern mining, fostering more robust and sustainable decision-making for the development of long-life mining assets.

ACKNOWLEDGEMENTS

The authors express their sincere gratitude to Antamina for their invaluable support. Special thanks are extended to the Tailings Strategy Management team and the Long-Term Planning team for their collaboration and insights.

REFERENCES

Boshoff, J. et al. (2023). A case study on the commingling of tailings and waste rock at a Brownfields open cast mine in Ghana. University of Alberta.

Burden, R., & Wilson, G. W. (2023). Commingling of waste rock and tailings to improve “dry stack” performance: Design and evaluation of mixtures. Minerals, 13(2), 295.

Deza, N., & Montes, K. (2022). Mecanización del desmonte en Compañía Minera Antamina: Propuesta para capturar el valor tangible e intangible de la mecanización. Compañía Minera Antamina.

Green Policy Platform. (2024). Knowledge Gaps Report: Environmental Aspects of Tailings Management (January 2024).

ICMM. (2020). New global industry standard on tailings management. International Council on Mining and Metals.

Ulrich, B. & Coffin, J. (2015). TMW 2015 – Combined Tailings and Mine Waste.

Minemax. (2015). Why mine planning is all about collaboration. Minemax News.

Wheaton Precious Metals. (2015). Antamina Project overview.

INTEGRATING COMMINGLING INTO THE STRATEGIC WASTE ROCK PLANNING AT ANTAMINA MINE

Christa Quiroz1, David Machin1, Olimpia Cabrera1, Fernando Angeles1

1 Antamina Mine, Lima, Peru

(*Presenting author: cquirozc@antamina.com)

ABSTRACT

Antamina is a large open-pit polymetallic mine in Peru, operating with a processing capacity of 145 ktpd. In operation since 2001, the current mine life extends to 2036. Mining is conducted at a rate of 290 Mtpa using conventional truck-and-shovel equipment. This technical paper presents a comprehensive assessment of commingling as a transformative technology in mine planning, positioning waste rock and tailings management as a key factor in the decision-making process. The primary objective of this study is to demonstrate the benefits of this technology within the mine planning process, highlighting waste rock and tailings management as one of the main drivers in strategic decision-making. The methodology applied is a Comparative Case Analysis, which includes a review of the design and sequencing process for mining infrastructure such as waste rock storage facilities, along with a discussion of the key parameters considered. The results show that the implementation of this technology delivers significant economic benefits, including an approximate 30% reduction in capital costs for waste rock storage facilities, improved land-use efficiency, and optimized infrastructure development. Furthermore, the approach enhances alignment with the Global Industry Standard on Tailings Management (GISTM).

KEYWORDS

commingling, mine planning, waste rock management, tailings management, GISTM compliance

10	INTRODUCTION

The mining industry faces growing challenges related to the sustainable management of waste rock and tailings, especially after catastrophic events such as those that occurred in Brumadinho and Samarco (Green Policy Platform, 2024; ICMM, 2020). In this context, the Antamina mine, operating since 2001 as one of the largest polymetallic operations in the world, has developed a pioneering approach that integrates commingling as a central element of its strategic planning (Wheaton Precious Metals, 2015).

Commingling, defined as the homogeneous mixing of dewatered tailings with waste rock to form a geotechnically stable material, represents a natural evolution of traditional co-disposal practices (Burden & Wilson, 2023). However, what distinguishes the Antamina case is the integration of this technology from the initial phases of mine planning, making waste rock management the main driver of operational and strategic decisions.

11	OBJECTIVES

•	Demonstrate the benefits of integrating commingling into mine planning, positioning waste rock and tailings management as a strategic driver for decision-making in open-pit operations.

•	Optimize operational efficiency and environmental sustainability through the adoption of commingling in waste rock and tailings management.

•	Align waste rock and tailings management with international sustainability and safety standards, such as the GISTM, through the application of commingling.

•	Analyze, through comparative case studies, the impact of commingling on land-use optimization and mine life extension.

•	Reduce capital and operating costs associated with waste rock and tailings disposal by implementing commingling technologies.

•	Present the key findings and lessons learned from the integration of commingling into long-term mine planning.

12	TECHNICAL FOUNDATIONS OF COMMINGLING IN MINE PLANNING

12.1	Desing Principles and Material Characterization

Commingling at Antamina is based on engineering principles that seek to combine the superior structural properties of waste rock with the low permeability characteristics of tailings (Boshoff, 2023). This combination results in a material with shear strength similar to waste rock and permeability comparable to tailings, creating conditions that restrict oxygen entry and water filtration, significantly reducing the potential for acid drainage generation (Ulrich & Coffin, 2015; Burden & Wilson, 2023).

The characterization of commingled material is performed through analysis of five distinct structural configurations: rock-dominated uncompacted, fines-dominated uncompacted, rock-dominated compacted, fines-dominated compacted, and the "just-filled" configuration where fines exactly fill the voids between rock particles (Burden & Wilson, 2023). This latter configuration represents the optimal target of the commingling process at Antamina.

Figure 1 Blend configurations after Wickland et al. (2006). Source: Burden & Wilson (2023).

12.2	Integration into the Strategic Planning Process

The integration of commingling into Antamina's mine planning follows a structured methodological approach that places tailings management with waste rock as central elements of decision-making. This global strategy allows for optimizing the mixture of materials from the mine to the plant, considering the operational and environmental constraints associated with the tailings storage facility. By incorporating tailings behavior in the early stages of planning, greater efficiency in resource use and better risk management are achieved.

The global strategy is divided into three key axes:

•	Strategy 1: Ensure Future Expansions: The first strategy focuses on maximizing tailings storage capacity for current and future needs, leveraging Antamina's mineral resource and applying advanced technology to increase TSF (Tailings Storage Facility) capacity to ensure the mine's long-term operational continuity.

•	Strategy 2: Optimized footprint: The second strategy seeks to optimize the overall project footprint by balancing waste and tailings storage space. This approach aligns directly with commingling principles by maximizing efficient use of available land within the current operational footprint.

•	Strategy 3: Reduce upfront CAPEX: The third strategy focuses on reducing initial CAPEX to improve key financial indicators (IRR), which directly aligns with the demonstrated economic benefits of commingling, minimizing operational costs with a nearby TSF and optimizing CAPEX timing.

Figure 2 Strategic vision of mine planning approach with commingling

The strategies implemented in tailings dam and waste dump management seek to maximize the operational life of the mine and optimize asset value. These decisions translate into concrete actions that address sustainability, operational efficiency, and cost reduction, ensuring continuity and long-term success of the mining operation.

•	Long Life Asset: Antamina's vision as a long-life asset involves making decisions based on sustainable exploitation of its exceptional mineral resources. This includes adopting advanced technologies, such as commingling, to improve storage capacity and ensure operational continuity until 2036 and beyond, thus maximizing the value of the mineral resource.

•	Reduce Environmental Social Footprint: The strategic approach prioritizes reducing environmental and social impact through operation within the current footprint, increasing water availability through filtration, optimizing space use, and minimizing risks associated with tailings dams. Commingling contributes not only to technical and economic efficiency but also to sustainability objectives and corporate social responsibility.

•	Reduce Capital and Operational Cost: The cost reduction strategy focuses on optimizing capital investments (CAPEX) and their schedule, as well as minimizing operational costs through strategic facility location and leveraging synergies to avoid duplicate infrastructure. This enables more efficient and profitable resource management.

During the strategic planning phase, conceptual foundations for commingling implementation are established, including site characterization, regulatory framework development, and technical-economic feasibility analysis.

13	ANALYSIS OF THE MINING SYSTEM WITH COMMINGLING APPLICATION

13.1	Methodological Process Structure

The methodological process is organized in sequential stages that ensure effective integration of commingling into mine planning:

•	Comprehensive material evaluation: Detailed analysis of geotechnical and geochemical characteristics of available materials, identifying their suitability for mixtures and joint disposal.

•	Mixture design development: Design of optimized mixtures that maximize desired properties of commingled material, such as low permeability and high stability, minimizing acid water generation and improving mechanical behavior of the deposit.

•	Pilot test implementation: Execution of pilot and industrial scale tests to validate mixture performance and adjust design parameters according to results obtained under real operating conditions.

•	Monitoring and adjustment: Establishment of continuous monitoring system to evaluate consolidation, stability, and geochemical behavior of deposits, allowing operational and design adjustments as new information is collected.

14	IMPLEMENTATION AND EVALUATION METHODOLOGY FOR COMMINGLING IN MINE PLANNING

14.1	Comparative Case Analysis

The methodology applied at Antamina is based on Comparative Case Analysis, allowing for comprehensive review of the design and sequencing process of mining infrastructure, with emphasis on dams, waste rock dumps, and integration of commingling technologies. This methodological approach comprises:

•	Review of design and sequencing processes: Analysis of procedures used in waste dump planning and disposal, considering both geotechnical and geochemical criteria to ensure infrastructure stability and sustainability.

•	Identification of key variables: Selection of fundamental operational parameters, such as granulometry, permeability, geochemical behavior, and storage capacity, which directly affect the viability and performance of dumps and material mixtures.

•	Definition of case tree: Structure of a decision tree that contemplates long-term strategic scenarios, allowing comparison of alternatives under different combinations of variables and operational constraints.

•	Results evaluation: Application of sensitivity analyses and cost-benefit evaluations to determine the impact of each alternative on mine life extension, environmental risk reduction, and resource optimization.

14.2	Decision Tree for Strategic Planning

The decision tree developed for long-term strategic planning at Antamina follows sequential logic that allows evaluation of feasibility and impact of main operational alternatives:

•	Commingling feasibility: The first node of the tree determines whether integration of tailings and waste rock is technically and economically viable, considering material characteristics and regulatory constraints.

•	Entry into new area: If commingling is feasible, the next node evaluates the possibility of disposing material in a new area, analyzing topographic, environmental, and access aspects.

•	Need for new dam: Finally, it determines whether construction of a new tailings dam is essential or if the commingling solution allows dispensing with this infrastructure, optimizing investment and reducing environmental impact.

Each of these decisions conditions mine life extension, efficient space use, and environmental risk management, allowing selection of the alternative that maximizes economic, social, and environmental value of the project.

Figure 3  Decision Tree for Long-Term Strategic Scenario Planning

14.3	Evaluation of Operational Parameters and Modifications by Commingling

Key operational parameters—including waste rock-tailings ratio, mixing methods, transport systems, and processing criteria—are assessed for their impact on commingled system efficiency. The evaluation considers site-specific challenges such as extreme Andean weather, high-altitude logistics, and Peruvian regulations, all of which are especially critical at Antamina and require customized commingling solutions.

•	Modifications to tailings management by commingling:

The commingling process fundamentally transforms tailings management by utilizing voids within waste rock for storage, with available space typically equivalent to about 20% of the crushed rock volume transported by conveyors. To enable mixing, tailings must first be dewatered. The following image shows both the current process (without mechanization) and the transformation that occurs when commingling is incorporated.

Figure 4 Transformation of Tailings Management: Evolution from Conventional Process to Commingling|

•	 Operational parameters and assumptions

Commingling extends tailings facility life by optimizing spatial use of waste rock voids. Implementation requires mechanized systems (WCCS) for transporting crushed material mixed with dewatered tailings, while maintaining two key operational parameters: a) Unchanged transport capacity: 83.2 Mtpa in the conveyor system and b) Volumetric stability: No increase in total volume of waste rock dump.

This approach ensures efficient resource utilization and supports sustainable mine operations.

Table 1: Key operational parameters and assumptions

Component	Units	Without - Case Conventional	With - Case Commingling

Cycloned/Filtered Max. Capacity	Mtpa	-	32.0

Percentage of Voids in Waste Rock	% Volume 	 	20 %

Waste Rock / Tailings Ratio 	Mt Rock/

Mt Tails	-	83 / 16

Conventional Tailings Density	t/m3 	1.81	1.81

Fines Tailings Density	 t/m3	NA	1.60

Tailings Volume, TDR 4195 (to Dic-22)	Mm3	376	376

Tailings Capacity, TDR 4195 (to Dic-22)	Mt	682	652

Mill Feed Rate 	Ktpd	145	145

The following image shows how incorporation of mechanized systems and commingling add capacity, and therefore extend tailings facility life, to increase the life of mine and therefore the overall asset value.

Figure 5 Commingling Extension of Tailings Facility Life through Commingling

The curve below shows the relationship between dry density and optimal mixing proportion, determinant for maximizing commingling efficiency.

Figure 6: Dry density curve vs. optimal mixing proportion for commingling

•	Analysis of Mineable Resources, Production and Value

A three-dimensional diagram is used to strategically analyze the interactions between mineable resource volume, production rate, and generated value—whether economic, environmental, or operational. This tool helps identify how increasing resource volume and production can maximize value, but also highlights key constraints such as tailings dam capacity and property limits. The approach supports comprehensive decision-making to optimize asset value and ensure project sustainability across various operational scenarios.

Figure 7 Value Optimization: Interaction between Resource Volume, Production Rate and Key Restrictions

15	ECONOMIC AND OPERATIONAL ADVANTAGES

15.1	Perspective of Conventional Value Drivers

The following figure illustrates Antamina's main conventional value drivers from a financial perspective. In the current scenario, the amount of mineral resource that can be utilized directly depends on the capital that the company decides to invest (Capex), both in new investments (Growth Capex) and in maintaining existing operations (Sustaining Capex). This generates a necessary balance to determine the optimal scenario that maximizes the value of the existing asset.

Figure 8: Conventional Antamina Value Drivers: Financial Perspective (Without Commingling)

15.2	Impact of Commingling on Financial Value Drivers

From a financial perspective, the adoption of commingling among Antamina’s value drivers removes constraints on mineral resources by increasing tailings storage capacity, enabling the utilization of new dumps, and allowing for an expansion in the volume of usable resources. As a result, the mine’s operational life is extended and asset value is enhanced. Regarding dump utilization, the implementation of commingling has the potential to reduce the requirement for dumps located in karstic zones, which demand significant capital expenditure (Capex). This can lead to a reduction in initial investment by up to 30% in scenarios comparing commingling versus non-commingling approaches. This saving is achieved through the integration of previously separate infrastructures into a single system, which optimizes the use of available area in the East Extension dumps and prioritizes the use of mechanized systems over conventional trucking methods.

Figure 9: Antamina Value Drivers: Financial Perspective with Commingling

The optimization of land use represents another important economic benefit, especially relevant in mountainous locations like Antamina where available land for mining facilities is limited. Commingling allows for the creation of deposits with a smaller footprint compared to separate facilities, freeing up land for other productive uses or reducing the need for additional land acquisition.

16	ALIGNMENT WITH GISTM STANDARDS AND SUSTAINABILITY

The integration of commingling into mine planning strengthens compliance with the Global Industry Standard on Tailings Management (GISTM), by facilitating safer and more sustainable management of waste materials. The GISTM requires the planning, construction, operation, and closure of tailings facilities with a focus on risk reduction and continuous monitoring throughout the lifecycle. Commingling contributes to this objective by improving the geotechnical and geochemical stability of deposits, reducing the risk of failure and acid drainage generation, which in turn reduces environmental impact and long-term treatment costs. Additionally, the consolidation of facilities through commingling reduces the environmental footprint and facilitates supervision and control, aligning with the principles of sustainability and social responsibility in the mining industry.

17	LESSONS LEARNED AND CHALLENGES

Among the main lessons learned and challenges of implementing commingling in mine planning at Antamina, the following stand out:

•	Large-scale operations and mechanization: Commingling is primarily viable in large-volume mechanized operations, where it is possible to achieve a homogeneous and controlled mixture of materials. However, the magnitude of waste rock and tailings tonnages at Antamina implies a new challenge, never seen before.

•	Flexible planning and pilot tests: It is essential to include areas for pilot tests in short and medium-term plans, allowing adjustment of mixing parameters according to material variability.

•	Regulatory communication: Technology acceptance requires identifying precedents and working closely with authorities to develop appropriate regulatory frameworks.

•	Organizational change management: The transition to commingling involves challenges in personnel training and adaptation of operational processes, especially in mature operations.

18	CONCLUSIONS

•	The comparative discussion and case analysis of commingling implementation at Antamina have enabled the identification and selection of strategic alternatives that maximize the economic, environmental, and social value of the project.

•	The use of decision trees and scenario evaluation has facilitated informed decision-making, taking into account technical and economic feasibility as well as regulatory and environmental constraints.

•	The integration of commingling into strategic mine planning has proven to be a key tool for optimizing waste rock and tailings management, achieving a significant reduction in capital costs (up to 30%), greater efficiency in land use, and an extension of tailings storage facility (TSF) life. This approach has also aligned operations with international sustainability standards (GISTM) and strengthened risk management, contributing to operational continuity and long-term reduction of environmental impacts.

•	The Antamina experience demonstrates that structured case discussions and the application of comparative methodologies are fundamental for strategic planning, as they allow anticipation of challenges, validation of solutions through pilot testing, and adjustment of operational parameters based on real outcomes. Thus, commingling is consolidated as a transformative practice in modern mining, fostering more robust and sustainable decision-making for the development of long-life mining assets.

ACKNOWLEDGEMENTS

The authors express their sincere gratitude to Antamina for their invaluable support. Special thanks are extended to the Tailings Strategy Management team and the Long-Term Planning team for their collaboration and insights.

REFERENCES

Boshoff, J. et al. (2023). A case study on the commingling of tailings and waste rock at a Brownfields open cast mine in Ghana. University of Alberta.

Burden, R., & Wilson, G. W. (2023). Commingling of waste rock and tailings to improve “dry stack” performance: Design and evaluation of mixtures. Minerals, 13(2), 295.

Deza, N., & Montes, K. (2022). Mecanización del desmonte en Compañía Minera Antamina: Propuesta para capturar el valor tangible e intangible de la mecanización. Compañía Minera Antamina.

Green Policy Platform. (2024). Knowledge Gaps Report: Environmental Aspects of Tailings Management (January 2024).

ICMM. (2020). New global industry standard on tailings management. International Council on Mining and Metals.

Ulrich, B. & Coffin, J. (2015). TMW 2015 – Combined Tailings and Mine Waste.

Minemax. (2015). Why mine planning is all about collaboration. Minemax News.

Wheaton Precious Metals. (2015). Antamina Project overview.

INTEGRATING COMMINGLING INTO THE STRATEGIC WASTE ROCK PLANNING AT ANTAMINA MINE

Christa Quiroz1, David Machin1, Olimpia Cabrera1, Fernando Angeles1

1 Antamina Mine, Lima, Peru

(*Presenting author: cquirozc@antamina.com)

ABSTRACT

Antamina is a large open-pit polymetallic mine in Peru, operating with a processing capacity of 145 ktpd. In operation since 2001, the current mine life extends to 2036. Mining is conducted at a rate of 290 Mtpa using conventional truck-and-shovel equipment. This technical paper presents a comprehensive assessment of commingling as a transformative technology in mine planning, positioning waste rock and tailings management as a key factor in the decision-making process. The primary objective of this study is to demonstrate the benefits of this technology within the mine planning process, highlighting waste rock and tailings management as one of the main drivers in strategic decision-making. The methodology applied is a Comparative Case Analysis, which includes a review of the design and sequencing process for mining infrastructure such as waste rock storage facilities, along with a discussion of the key parameters considered. The results show that the implementation of this technology delivers significant economic benefits, including an approximate 30% reduction in capital costs for waste rock storage facilities, improved land-use efficiency, and optimized infrastructure development. Furthermore, the approach enhances alignment with the Global Industry Standard on Tailings Management (GISTM).

KEYWORDS

commingling, mine planning, waste rock management, tailings management, GISTM compliance

1	INTRODUCTION

The mining industry faces growing challenges related to the sustainable management of waste rock and tailings, especially after catastrophic events such as those that occurred in Brumadinho and Samarco (Green Policy Platform, 2024; ICMM, 2020). In this context, the Antamina mine, operating since 2001 as one of the largest polymetallic operations in the world, has developed a pioneering approach that integrates commingling as a central element of its strategic planning (Wheaton Precious Metals, 2015).

Commingling, defined as the homogeneous mixing of dewatered tailings with waste rock to form a geotechnically stable material, represents a natural evolution of traditional co-disposal practices (Burden & Wilson, 2023). However, what distinguishes the Antamina case is the integration of this technology from the initial phases of mine planning, making waste rock management the main driver of operational and strategic decisions.

2	OBJECTIVES

•	Demonstrate the benefits of integrating commingling into mine planning, positioning waste rock and tailings management as a strategic driver for decision-making in open-pit operations.

•	Optimize operational efficiency and environmental sustainability through the adoption of commingling in waste rock and tailings management.

•	Align waste rock and tailings management with international sustainability and safety standards, such as the GISTM, through the application of commingling.

•	Analyze, through comparative case studies, the impact of commingling on land-use optimization and mine life extension.

•	Reduce capital and operating costs associated with waste rock and tailings disposal by implementing commingling technologies.

•	Present the key findings and lessons learned from the integration of commingling into long-term mine planning.

3	TECHNICAL FOUNDATIONS OF COMMINGLING IN MINE PLANNING

3.1	Desing Principles and Material Characterization

Commingling at Antamina is based on engineering principles that seek to combine the superior structural properties of waste rock with the low permeability characteristics of tailings (Boshoff, 2023). This combination results in a material with shear strength similar to waste rock and permeability comparable to tailings, creating conditions that restrict oxygen entry and water filtration, significantly reducing the potential for acid drainage generation (Ulrich & Coffin, 2015; Burden & Wilson, 2023).

The characterization of commingled material is performed through analysis of five distinct structural configurations: rock-dominated uncompacted, fines-dominated uncompacted, rock-dominated compacted, fines-dominated compacted, and the "just-filled" configuration where fines exactly fill the voids between rock particles (Burden & Wilson, 2023). This latter configuration represents the optimal target of the commingling process at Antamina.

Figure 1 Blend configurations after Wickland et al. (2006). Source: Burden & Wilson (2023).

3.2	Integration into the Strategic Planning Process

The integration of commingling into Antamina's mine planning follows a structured methodological approach that places tailings management with waste rock as central elements of decision-making. This global strategy allows for optimizing the mixture of materials from the mine to the plant, considering the operational and environmental constraints associated with the tailings storage facility. By incorporating tailings behavior in the early stages of planning, greater efficiency in resource use and better risk management are achieved.

The global strategy is divided into three key axes:

•	Strategy 1: Ensure Future Expansions: The first strategy focuses on maximizing tailings storage capacity for current and future needs, leveraging Antamina's mineral resource and applying advanced technology to increase TSF (Tailings Storage Facility) capacity to ensure the mine's long-term operational continuity.

•	Strategy 2: Optimized footprint: The second strategy seeks to optimize the overall project footprint by balancing waste and tailings storage space. This approach aligns directly with commingling principles by maximizing efficient use of available land within the current operational footprint.

•	Strategy 3: Reduce upfront CAPEX: The third strategy focuses on reducing initial CAPEX to improve key financial indicators (IRR), which directly aligns with the demonstrated economic benefits of commingling, minimizing operational costs with a nearby TSF and optimizing CAPEX timing.

Figure 2 Strategic vision of mine planning approach with commingling

The strategies implemented in tailings dam and waste dump management seek to maximize the operational life of the mine and optimize asset value. These decisions translate into concrete actions that address sustainability, operational efficiency, and cost reduction, ensuring continuity and long-term success of the mining operation.

•	Long Life Asset: Antamina's vision as a long-life asset involves making decisions based on sustainable exploitation of its exceptional mineral resources. This includes adopting advanced technologies, such as commingling, to improve storage capacity and ensure operational continuity until 2036 and beyond, thus maximizing the value of the mineral resource.

•	Reduce Environmental Social Footprint: The strategic approach prioritizes reducing environmental and social impact through operation within the current footprint, increasing water availability through filtration, optimizing space use, and minimizing risks associated with tailings dams. Commingling contributes not only to technical and economic efficiency but also to sustainability objectives and corporate social responsibility.

•	Reduce Capital and Operational Cost: The cost reduction strategy focuses on optimizing capital investments (CAPEX) and their schedule, as well as minimizing operational costs through strategic facility location and leveraging synergies to avoid duplicate infrastructure. This enables more efficient and profitable resource management.

During the strategic planning phase, conceptual foundations for commingling implementation are established, including site characterization, regulatory framework development, and technical-economic feasibility analysis.

4	ANALYSIS OF THE MINING SYSTEM WITH COMMINGLING APPLICATION

4.1	Methodological Process Structure

The methodological process is organized in sequential stages that ensure effective integration of commingling into mine planning:

•	Comprehensive material evaluation: Detailed analysis of geotechnical and geochemical characteristics of available materials, identifying their suitability for mixtures and joint disposal.

•	Mixture design development: Design of optimized mixtures that maximize desired properties of commingled material, such as low permeability and high stability, minimizing acid water generation and improving mechanical behavior of the deposit.

•	Pilot test implementation: Execution of pilot and industrial scale tests to validate mixture performance and adjust design parameters according to results obtained under real operating conditions.

•	Monitoring and adjustment: Establishment of continuous monitoring system to evaluate consolidation, stability, and geochemical behavior of deposits, allowing operational and design adjustments as new information is collected.

5	IMPLEMENTATION AND EVALUATION METHODOLOGY FOR COMMINGLING IN MINE PLANNING

5.1	Comparative Case Analysis

The methodology applied at Antamina is based on Comparative Case Analysis, allowing for comprehensive review of the design and sequencing process of mining infrastructure, with emphasis on dams, waste rock dumps, and integration of commingling technologies. This methodological approach comprises:

•	Review of design and sequencing processes: Analysis of procedures used in waste dump planning and disposal, considering both geotechnical and geochemical criteria to ensure infrastructure stability and sustainability.

•	Identification of key variables: Selection of fundamental operational parameters, such as granulometry, permeability, geochemical behavior, and storage capacity, which directly affect the viability and performance of dumps and material mixtures.

•	Definition of case tree: Structure of a decision tree that contemplates long-term strategic scenarios, allowing comparison of alternatives under different combinations of variables and operational constraints.

•	Results evaluation: Application of sensitivity analyses and cost-benefit evaluations to determine the impact of each alternative on mine life extension, environmental risk reduction, and resource optimization.

5.2	Decision Tree for Strategic Planning

The decision tree developed for long-term strategic planning at Antamina follows sequential logic that allows evaluation of feasibility and impact of main operational alternatives:

•	Commingling feasibility: The first node of the tree determines whether integration of tailings and waste rock is technically and economically viable, considering material characteristics and regulatory constraints.

•	Entry into new area: If commingling is feasible, the next node evaluates the possibility of disposing material in a new area, analyzing topographic, environmental, and access aspects.

•	Need for new dam: Finally, it determines whether construction of a new tailings dam is essential or if the commingling solution allows dispensing with this infrastructure, optimizing investment and reducing environmental impact.

Each of these decisions conditions mine life extension, efficient space use, and environmental risk management, allowing selection of the alternative that maximizes economic, social, and environmental value of the project.

Figure 3  Decision Tree for Long-Term Strategic Scenario Planning

5.3	Evaluation of Operational Parameters and Modifications by Commingling

Key operational parameters—including waste rock-tailings ratio, mixing methods, transport systems, and processing criteria—are assessed for their impact on commingled system efficiency. The evaluation considers site-specific challenges such as extreme Andean weather, high-altitude logistics, and Peruvian regulations, all of which are especially critical at Antamina and require customized commingling solutions.

•	Modifications to tailings management by commingling:

The commingling process fundamentally transforms tailings management by utilizing voids within waste rock for storage, with available space typically equivalent to about 20% of the crushed rock volume transported by conveyors. To enable mixing, tailings must first be dewatered. The following image shows both the current process (without mechanization) and the transformation that occurs when commingling is incorporated.

Figure 4 Transformation of Tailings Management: Evolution from Conventional Process to Commingling|

•	 Operational parameters and assumptions

Commingling extends tailings facility life by optimizing spatial use of waste rock voids. Implementation requires mechanized systems (WCCS) for transporting crushed material mixed with dewatered tailings, while maintaining two key operational parameters: a) Unchanged transport capacity: 83.2 Mtpa in the conveyor system and b) Volumetric stability: No increase in total volume of waste rock dump.

This approach ensures efficient resource utilization and supports sustainable mine operations.

Table 1: Key operational parameters and assumptions

Component	Units	Without - Case Conventional	With - Case Commingling

Cycloned/Filtered Max. Capacity	Mtpa	-	32.0

Percentage of Voids in Waste Rock	% Volume 	 	20 %

Waste Rock / Tailings Ratio 	Mt Rock/

Mt Tails	-	83 / 16

Conventional Tailings Density	t/m3 	1.81	1.81

Fines Tailings Density	 t/m3	NA	1.60

Tailings Volume, TDR 4195 (to Dic-22)	Mm3	376	376

Tailings Capacity, TDR 4195 (to Dic-22)	Mt	682	652

Mill Feed Rate 	Ktpd	145	145

The following image shows how incorporation of mechanized systems and commingling add capacity, and therefore extend tailings facility life, to increase the life of mine and therefore the overall asset value.

Figure 5 Commingling Extension of Tailings Facility Life through Commingling

The curve below shows the relationship between dry density and optimal mixing proportion, determinant for maximizing commingling efficiency.

Figure 6: Dry density curve vs. optimal mixing proportion for commingling

•	Analysis of Mineable Resources, Production and Value

A three-dimensional diagram is used to strategically analyze the interactions between mineable resource volume, production rate, and generated value—whether economic, environmental, or operational. This tool helps identify how increasing resource volume and production can maximize value, but also highlights key constraints such as tailings dam capacity and property limits. The approach supports comprehensive decision-making to optimize asset value and ensure project sustainability across various operational scenarios.

Figure 7 Value Optimization: Interaction between Resource Volume, Production Rate and Key Restrictions

6	ECONOMIC AND OPERATIONAL ADVANTAGES

6.1	Perspective of Conventional Value Drivers

The following figure illustrates Antamina's main conventional value drivers from a financial perspective. In the current scenario, the amount of mineral resource that can be utilized directly depends on the capital that the company decides to invest (Capex), both in new investments (Growth Capex) and in maintaining existing operations (Sustaining Capex). This generates a necessary balance to determine the optimal scenario that maximizes the value of the existing asset.

Figure 8: Conventional Antamina Value Drivers: Financial Perspective (Without Commingling)

6.2	Impact of Commingling on Financial Value Drivers

From a financial perspective, the adoption of commingling among Antamina’s value drivers removes constraints on mineral resources by increasing tailings storage capacity, enabling the utilization of new dumps, and allowing for an expansion in the volume of usable resources. As a result, the mine’s operational life is extended and asset value is enhanced. Regarding dump utilization, the implementation of commingling has the potential to reduce the requirement for dumps located in karstic zones, which demand significant capital expenditure (Capex). This can lead to a reduction in initial investment by up to 30% in scenarios comparing commingling versus non-commingling approaches. This saving is achieved through the integration of previously separate infrastructures into a single system, which optimizes the use of available area in the East Extension dumps and prioritizes the use of mechanized systems over conventional trucking methods.

Figure 9: Antamina Value Drivers: Financial Perspective with Commingling

The optimization of land use represents another important economic benefit, especially relevant in mountainous locations like Antamina where available land for mining facilities is limited. Commingling allows for the creation of deposits with a smaller footprint compared to separate facilities, freeing up land for other productive uses or reducing the need for additional land acquisition.

7	ALIGNMENT WITH GISTM STANDARDS AND SUSTAINABILITY

The integration of commingling into mine planning strengthens compliance with the Global Industry Standard on Tailings Management (GISTM), by facilitating safer and more sustainable management of waste materials. The GISTM requires the planning, construction, operation, and closure of tailings facilities with a focus on risk reduction and continuous monitoring throughout the lifecycle. Commingling contributes to this objective by improving the geotechnical and geochemical stability of deposits, reducing the risk of failure and acid drainage generation, which in turn reduces environmental impact and long-term treatment costs. Additionally, the consolidation of facilities through commingling reduces the environmental footprint and facilitates supervision and control, aligning with the principles of sustainability and social responsibility in the mining industry.

8	LESSONS LEARNED AND CHALLENGES

Among the main lessons learned and challenges of implementing commingling in mine planning at Antamina, the following stand out:

•	Large-scale operations and mechanization: Commingling is primarily viable in large-volume mechanized operations, where it is possible to achieve a homogeneous and controlled mixture of materials. However, the magnitude of waste rock and tailings tonnages at Antamina implies a new challenge, never seen before.

•	Flexible planning and pilot tests: It is essential to include areas for pilot tests in short and medium-term plans, allowing adjustment of mixing parameters according to material variability.

•	Regulatory communication: Technology acceptance requires identifying precedents and working closely with authorities to develop appropriate regulatory frameworks.

•	Organizational change management: The transition to commingling involves challenges in personnel training and adaptation of operational processes, especially in mature operations.

9	CONCLUSIONS

•	The comparative discussion and case analysis of commingling implementation at Antamina have enabled the identification and selection of strategic alternatives that maximize the economic, environmental, and social value of the project.

•	The use of decision trees and scenario evaluation has facilitated informed decision-making, taking into account technical and economic feasibility as well as regulatory and environmental constraints.

•	The integration of commingling into strategic mine planning has proven to be a key tool for optimizing waste rock and tailings management, achieving a significant reduction in capital costs (up to 30%), greater efficiency in land use, and an extension of tailings storage facility (TSF) life. This approach has also aligned operations with international sustainability standards (GISTM) and strengthened risk management, contributing to operational continuity and long-term reduction of environmental impacts.

•	The Antamina experience demonstrates that structured case discussions and the application of comparative methodologies are fundamental for strategic planning, as they allow anticipation of challenges, validation of solutions through pilot testing, and adjustment of operational parameters based on real outcomes. Thus, commingling is consolidated as a transformative practice in modern mining, fostering more robust and sustainable decision-making for the development of long-life mining assets.

ACKNOWLEDGEMENTS

The authors express their sincere gratitude to Antamina for their invaluable support. Special thanks are extended to the Tailings Strategy Management team and the Long-Term Planning team for their collaboration and insights.

REFERENCES

Boshoff, J. et al. (2023). A case study on the commingling of tailings and waste rock at a Brownfields open cast mine in Ghana. University of Alberta.

Burden, R., & Wilson, G. W. (2023). Commingling of waste rock and tailings to improve “dry stack” performance: Design and evaluation of mixtures. Minerals, 13(2), 295.

Deza, N., & Montes, K. (2022). Mecanización del desmonte en Compañía Minera Antamina: Propuesta para capturar el valor tangible e intangible de la mecanización. Compañía Minera Antamina.

Green Policy Platform. (2024). Knowledge Gaps Report: Environmental Aspects of Tailings Management (January 2024).

ICMM. (2020). New global industry standard on tailings management. International Council on Mining and Metals.

Ulrich, B. & Coffin, J. (2015). TMW 2015 – Combined Tailings and Mine Waste.

Minemax. (2015). Why mine planning is all about collaboration. Minemax News.

Wheaton Precious Metals. (2015). Antamina Project overview.

INTEGRATING COMMINGLING INTO THE STRATEGIC WASTE ROCK PLANNING AT ANTAMINA MINE

Christa Quiroz1, David Machin1, Olimpia Cabrera1, Fernando Angeles1

1 Antamina Mine, Lima, Peru

(*Presenting author: cquirozc@antamina.com)

ABSTRACT

Antamina is a large open-pit polymetallic mine in Peru, operating with a processing capacity of 145 ktpd. In operation since 2001, the current mine life extends to 2036. Mining is conducted at a rate of 290 Mtpa using conventional truck-and-shovel equipment. This technical paper presents a comprehensive assessment of commingling as a transformative technology in mine planning, positioning waste rock and tailings management as a key factor in the decision-making process. The primary objective of this study is to demonstrate the benefits of this technology within the mine planning process, highlighting waste rock and tailings management as one of the main drivers in strategic decision-making. The methodology applied is a Comparative Case Analysis, which includes a review of the design and sequencing process for mining infrastructure such as waste rock storage facilities, along with a discussion of the key parameters considered. The results show that the implementation of this technology delivers significant economic benefits, including an approximate 30% reduction in capital costs for waste rock storage facilities, improved land-use efficiency, and optimized infrastructure development. Furthermore, the approach enhances alignment with the Global Industry Standard on Tailings Management (GISTM).

KEYWORDS

commingling, mine planning, waste rock management, tailings management, GISTM compliance

10	INTRODUCTION

The mining industry faces growing challenges related to the sustainable management of waste rock and tailings, especially after catastrophic events such as those that occurred in Brumadinho and Samarco (Green Policy Platform, 2024; ICMM, 2020). In this context, the Antamina mine, operating since 2001 as one of the largest polymetallic operations in the world, has developed a pioneering approach that integrates commingling as a central element of its strategic planning (Wheaton Precious Metals, 2015).

Commingling, defined as the homogeneous mixing of dewatered tailings with waste rock to form a geotechnically stable material, represents a natural evolution of traditional co-disposal practices (Burden & Wilson, 2023). However, what distinguishes the Antamina case is the integration of this technology from the initial phases of mine planning, making waste rock management the main driver of operational and strategic decisions.

11	OBJECTIVES

•	Demonstrate the benefits of integrating commingling into mine planning, positioning waste rock and tailings management as a strategic driver for decision-making in open-pit operations.

•	Optimize operational efficiency and environmental sustainability through the adoption of commingling in waste rock and tailings management.

•	Align waste rock and tailings management with international sustainability and safety standards, such as the GISTM, through the application of commingling.

•	Analyze, through comparative case studies, the impact of commingling on land-use optimization and mine life extension.

•	Reduce capital and operating costs associated with waste rock and tailings disposal by implementing commingling technologies.

•	Present the key findings and lessons learned from the integration of commingling into long-term mine planning.

12	TECHNICAL FOUNDATIONS OF COMMINGLING IN MINE PLANNING

12.1	Desing Principles and Material Characterization

Commingling at Antamina is based on engineering principles that seek to combine the superior structural properties of waste rock with the low permeability characteristics of tailings (Boshoff, 2023). This combination results in a material with shear strength similar to waste rock and permeability comparable to tailings, creating conditions that restrict oxygen entry and water filtration, significantly reducing the potential for acid drainage generation (Ulrich & Coffin, 2015; Burden & Wilson, 2023).

The characterization of commingled material is performed through analysis of five distinct structural configurations: rock-dominated uncompacted, fines-dominated uncompacted, rock-dominated compacted, fines-dominated compacted, and the "just-filled" configuration where fines exactly fill the voids between rock particles (Burden & Wilson, 2023). This latter configuration represents the optimal target of the commingling process at Antamina.

Figure 1 Blend configurations after Wickland et al. (2006). Source: Burden & Wilson (2023).

12.2	Integration into the Strategic Planning Process

The integration of commingling into Antamina's mine planning follows a structured methodological approach that places tailings management with waste rock as central elements of decision-making. This global strategy allows for optimizing the mixture of materials from the mine to the plant, considering the operational and environmental constraints associated with the tailings storage facility. By incorporating tailings behavior in the early stages of planning, greater efficiency in resource use and better risk management are achieved.

The global strategy is divided into three key axes:

•	Strategy 1: Ensure Future Expansions: The first strategy focuses on maximizing tailings storage capacity for current and future needs, leveraging Antamina's mineral resource and applying advanced technology to increase TSF (Tailings Storage Facility) capacity to ensure the mine's long-term operational continuity.

•	Strategy 2: Optimized footprint: The second strategy seeks to optimize the overall project footprint by balancing waste and tailings storage space. This approach aligns directly with commingling principles by maximizing efficient use of available land within the current operational footprint.

•	Strategy 3: Reduce upfront CAPEX: The third strategy focuses on reducing initial CAPEX to improve key financial indicators (IRR), which directly aligns with the demonstrated economic benefits of commingling, minimizing operational costs with a nearby TSF and optimizing CAPEX timing.

Figure 2 Strategic vision of mine planning approach with commingling

The strategies implemented in tailings dam and waste dump management seek to maximize the operational life of the mine and optimize asset value. These decisions translate into concrete actions that address sustainability, operational efficiency, and cost reduction, ensuring continuity and long-term success of the mining operation.

•	Long Life Asset: Antamina's vision as a long-life asset involves making decisions based on sustainable exploitation of its exceptional mineral resources. This includes adopting advanced technologies, such as commingling, to improve storage capacity and ensure operational continuity until 2036 and beyond, thus maximizing the value of the mineral resource.

•	Reduce Environmental Social Footprint: The strategic approach prioritizes reducing environmental and social impact through operation within the current footprint, increasing water availability through filtration, optimizing space use, and minimizing risks associated with tailings dams. Commingling contributes not only to technical and economic efficiency but also to sustainability objectives and corporate social responsibility.

•	Reduce Capital and Operational Cost: The cost reduction strategy focuses on optimizing capital investments (CAPEX) and their schedule, as well as minimizing operational costs through strategic facility location and leveraging synergies to avoid duplicate infrastructure. This enables more efficient and profitable resource management.

During the strategic planning phase, conceptual foundations for commingling implementation are established, including site characterization, regulatory framework development, and technical-economic feasibility analysis.

13	ANALYSIS OF THE MINING SYSTEM WITH COMMINGLING APPLICATION

13.1	Methodological Process Structure

The methodological process is organized in sequential stages that ensure effective integration of commingling into mine planning:

•	Comprehensive material evaluation: Detailed analysis of geotechnical and geochemical characteristics of available materials, identifying their suitability for mixtures and joint disposal.

•	Mixture design development: Design of optimized mixtures that maximize desired properties of commingled material, such as low permeability and high stability, minimizing acid water generation and improving mechanical behavior of the deposit.

•	Pilot test implementation: Execution of pilot and industrial scale tests to validate mixture performance and adjust design parameters according to results obtained under real operating conditions.

•	Monitoring and adjustment: Establishment of continuous monitoring system to evaluate consolidation, stability, and geochemical behavior of deposits, allowing operational and design adjustments as new information is collected.

14	IMPLEMENTATION AND EVALUATION METHODOLOGY FOR COMMINGLING IN MINE PLANNING

14.1	Comparative Case Analysis

The methodology applied at Antamina is based on Comparative Case Analysis, allowing for comprehensive review of the design and sequencing process of mining infrastructure, with emphasis on dams, waste rock dumps, and integration of commingling technologies. This methodological approach comprises:

•	Review of design and sequencing processes: Analysis of procedures used in waste dump planning and disposal, considering both geotechnical and geochemical criteria to ensure infrastructure stability and sustainability.

•	Identification of key variables: Selection of fundamental operational parameters, such as granulometry, permeability, geochemical behavior, and storage capacity, which directly affect the viability and performance of dumps and material mixtures.

•	Definition of case tree: Structure of a decision tree that contemplates long-term strategic scenarios, allowing comparison of alternatives under different combinations of variables and operational constraints.

•	Results evaluation: Application of sensitivity analyses and cost-benefit evaluations to determine the impact of each alternative on mine life extension, environmental risk reduction, and resource optimization.

14.2	Decision Tree for Strategic Planning

The decision tree developed for long-term strategic planning at Antamina follows sequential logic that allows evaluation of feasibility and impact of main operational alternatives:

•	Commingling feasibility: The first node of the tree determines whether integration of tailings and waste rock is technically and economically viable, considering material characteristics and regulatory constraints.

•	Entry into new area: If commingling is feasible, the next node evaluates the possibility of disposing material in a new area, analyzing topographic, environmental, and access aspects.

•	Need for new dam: Finally, it determines whether construction of a new tailings dam is essential or if the commingling solution allows dispensing with this infrastructure, optimizing investment and reducing environmental impact.

Each of these decisions conditions mine life extension, efficient space use, and environmental risk management, allowing selection of the alternative that maximizes economic, social, and environmental value of the project.

Figure 3  Decision Tree for Long-Term Strategic Scenario Planning

14.3	Evaluation of Operational Parameters and Modifications by Commingling

Key operational parameters—including waste rock-tailings ratio, mixing methods, transport systems, and processing criteria—are assessed for their impact on commingled system efficiency. The evaluation considers site-specific challenges such as extreme Andean weather, high-altitude logistics, and Peruvian regulations, all of which are especially critical at Antamina and require customized commingling solutions.

•	Modifications to tailings management by commingling:

The commingling process fundamentally transforms tailings management by utilizing voids within waste rock for storage, with available space typically equivalent to about 20% of the crushed rock volume transported by conveyors. To enable mixing, tailings must first be dewatered. The following image shows both the current process (without mechanization) and the transformation that occurs when commingling is incorporated.

Figure 4 Transformation of Tailings Management: Evolution from Conventional Process to Commingling|

•	 Operational parameters and assumptions

Commingling extends tailings facility life by optimizing spatial use of waste rock voids. Implementation requires mechanized systems (WCCS) for transporting crushed material mixed with dewatered tailings, while maintaining two key operational parameters: a) Unchanged transport capacity: 83.2 Mtpa in the conveyor system and b) Volumetric stability: No increase in total volume of waste rock dump.

This approach ensures efficient resource utilization and supports sustainable mine operations.

Table 1: Key operational parameters and assumptions

Component	Units	Without - Case Conventional	With - Case Commingling

Cycloned/Filtered Max. Capacity	Mtpa	-	32.0

Percentage of Voids in Waste Rock	% Volume 	 	20 %

Waste Rock / Tailings Ratio 	Mt Rock/

Mt Tails	-	83 / 16

Conventional Tailings Density	t/m3 	1.81	1.81

Fines Tailings Density	 t/m3	NA	1.60

Tailings Volume, TDR 4195 (to Dic-22)	Mm3	376	376

Tailings Capacity, TDR 4195 (to Dic-22)	Mt	682	652

Mill Feed Rate 	Ktpd	145	145

The following image shows how incorporation of mechanized systems and commingling add capacity, and therefore extend tailings facility life, to increase the life of mine and therefore the overall asset value.

Figure 5 Commingling Extension of Tailings Facility Life through Commingling

The curve below shows the relationship between dry density and optimal mixing proportion, determinant for maximizing commingling efficiency.

Figure 6: Dry density curve vs. optimal mixing proportion for commingling

•	Analysis of Mineable Resources, Production and Value

A three-dimensional diagram is used to strategically analyze the interactions between mineable resource volume, production rate, and generated value—whether economic, environmental, or operational. This tool helps identify how increasing resource volume and production can maximize value, but also highlights key constraints such as tailings dam capacity and property limits. The approach supports comprehensive decision-making to optimize asset value and ensure project sustainability across various operational scenarios.

Figure 7 Value Optimization: Interaction between Resource Volume, Production Rate and Key Restrictions

15	ECONOMIC AND OPERATIONAL ADVANTAGES

15.1	Perspective of Conventional Value Drivers

The following figure illustrates Antamina's main conventional value drivers from a financial perspective. In the current scenario, the amount of mineral resource that can be utilized directly depends on the capital that the company decides to invest (Capex), both in new investments (Growth Capex) and in maintaining existing operations (Sustaining Capex). This generates a necessary balance to determine the optimal scenario that maximizes the value of the existing asset.

Figure 8: Conventional Antamina Value Drivers: Financial Perspective (Without Commingling)

15.2	Impact of Commingling on Financial Value Drivers

From a financial perspective, the adoption of commingling among Antamina’s value drivers removes constraints on mineral resources by increasing tailings storage capacity, enabling the utilization of new dumps, and allowing for an expansion in the volume of usable resources. As a result, the mine’s operational life is extended and asset value is enhanced. Regarding dump utilization, the implementation of commingling has the potential to reduce the requirement for dumps located in karstic zones, which demand significant capital expenditure (Capex). This can lead to a reduction in initial investment by up to 30% in scenarios comparing commingling versus non-commingling approaches. This saving is achieved through the integration of previously separate infrastructures into a single system, which optimizes the use of available area in the East Extension dumps and prioritizes the use of mechanized systems over conventional trucking methods.

Figure 9: Antamina Value Drivers: Financial Perspective with Commingling

The optimization of land use represents another important economic benefit, especially relevant in mountainous locations like Antamina where available land for mining facilities is limited. Commingling allows for the creation of deposits with a smaller footprint compared to separate facilities, freeing up land for other productive uses or reducing the need for additional land acquisition.

16	ALIGNMENT WITH GISTM STANDARDS AND SUSTAINABILITY

The integration of commingling into mine planning strengthens compliance with the Global Industry Standard on Tailings Management (GISTM), by facilitating safer and more sustainable management of waste materials. The GISTM requires the planning, construction, operation, and closure of tailings facilities with a focus on risk reduction and continuous monitoring throughout the lifecycle. Commingling contributes to this objective by improving the geotechnical and geochemical stability of deposits, reducing the risk of failure and acid drainage generation, which in turn reduces environmental impact and long-term treatment costs. Additionally, the consolidation of facilities through commingling reduces the environmental footprint and facilitates supervision and control, aligning with the principles of sustainability and social responsibility in the mining industry.

17	LESSONS LEARNED AND CHALLENGES

Among the main lessons learned and challenges of implementing commingling in mine planning at Antamina, the following stand out:

•	Large-scale operations and mechanization: Commingling is primarily viable in large-volume mechanized operations, where it is possible to achieve a homogeneous and controlled mixture of materials. However, the magnitude of waste rock and tailings tonnages at Antamina implies a new challenge, never seen before.

•	Flexible planning and pilot tests: It is essential to include areas for pilot tests in short and medium-term plans, allowing adjustment of mixing parameters according to material variability.

•	Regulatory communication: Technology acceptance requires identifying precedents and working closely with authorities to develop appropriate regulatory frameworks.

•	Organizational change management: The transition to commingling involves challenges in personnel training and adaptation of operational processes, especially in mature operations.

18	CONCLUSIONS

•	The comparative discussion and case analysis of commingling implementation at Antamina have enabled the identification and selection of strategic alternatives that maximize the economic, environmental, and social value of the project.

•	The use of decision trees and scenario evaluation has facilitated informed decision-making, taking into account technical and economic feasibility as well as regulatory and environmental constraints.

•	The integration of commingling into strategic mine planning has proven to be a key tool for optimizing waste rock and tailings management, achieving a significant reduction in capital costs (up to 30%), greater efficiency in land use, and an extension of tailings storage facility (TSF) life. This approach has also aligned operations with international sustainability standards (GISTM) and strengthened risk management, contributing to operational continuity and long-term reduction of environmental impacts.

•	The Antamina experience demonstrates that structured case discussions and the application of comparative methodologies are fundamental for strategic planning, as they allow anticipation of challenges, validation of solutions through pilot testing, and adjustment of operational parameters based on real outcomes. Thus, commingling is consolidated as a transformative practice in modern mining, fostering more robust and sustainable decision-making for the development of long-life mining assets.

ACKNOWLEDGEMENTS

The authors express their sincere gratitude to Antamina for their invaluable support. Special thanks are extended to the Tailings Strategy Management team and the Long-Term Planning team for their collaboration and insights.

REFERENCES

Boshoff, J. et al. (2023). A case study on the commingling of tailings and waste rock at a Brownfields open cast mine in Ghana. University of Alberta.

Burden, R., & Wilson, G. W. (2023). Commingling of waste rock and tailings to improve “dry stack” performance: Design and evaluation of mixtures. Minerals, 13(2), 295.

Deza, N., & Montes, K. (2022). Mecanización del desmonte en Compañía Minera Antamina: Propuesta para capturar el valor tangible e intangible de la mecanización. Compañía Minera Antamina.

Green Policy Platform. (2024). Knowledge Gaps Report: Environmental Aspects of Tailings Management (January 2024).

ICMM. (2020). New global industry standard on tailings management. International Council on Mining and Metals.

Ulrich, B. & Coffin, J. (2015). TMW 2015 – Combined Tailings and Mine Waste.

Minemax. (2015). Why mine planning is all about collaboration. Minemax News.

Wheaton Precious Metals. (2015). Antamina Project overview.

INTEGRATING COMMINGLING INTO THE STRATEGIC WASTE ROCK PLANNING AT ANTAMINA MINE

Christa Quiroz1, David Machin1, Olimpia Cabrera1, Fernando Angeles1

1 Antamina Mine, Lima, Peru

(*Presenting author: cquirozc@antamina.com)

ABSTRACT

Antamina is a large open-pit polymetallic mine in Peru, operating with a processing capacity of 145 ktpd. In operation since 2001, the current mine life extends to 2036. Mining is conducted at a rate of 290 Mtpa using conventional truck-and-shovel equipment. This technical paper presents a comprehensive assessment of commingling as a transformative technology in mine planning, positioning waste rock and tailings management as a key factor in the decision-making process. The primary objective of this study is to demonstrate the benefits of this technology within the mine planning process, highlighting waste rock and tailings management as one of the main drivers in strategic decision-making. The methodology applied is a Comparative Case Analysis, which includes a review of the design and sequencing process for mining infrastructure such as waste rock storage facilities, along with a discussion of the key parameters considered. The results show that the implementation of this technology delivers significant economic benefits, including an approximate 30% reduction in capital costs for waste rock storage facilities, improved land-use efficiency, and optimized infrastructure development. Furthermore, the approach enhances alignment with the Global Industry Standard on Tailings Management (GISTM).

KEYWORDS

commingling, mine planning, waste rock management, tailings management, GISTM compliance

1	INTRODUCTION

The mining industry faces growing challenges related to the sustainable management of waste rock and tailings, especially after catastrophic events such as those that occurred in Brumadinho and Samarco (Green Policy Platform, 2024; ICMM, 2020). In this context, the Antamina mine, operating since 2001 as one of the largest polymetallic operations in the world, has developed a pioneering approach that integrates commingling as a central element of its strategic planning (Wheaton Precious Metals, 2015).

Commingling, defined as the homogeneous mixing of dewatered tailings with waste rock to form a geotechnically stable material, represents a natural evolution of traditional co-disposal practices (Burden & Wilson, 2023). However, what distinguishes the Antamina case is the integration of this technology from the initial phases of mine planning, making waste rock management the main driver of operational and strategic decisions.

2	OBJECTIVES

•	Demonstrate the benefits of integrating commingling into mine planning, positioning waste rock and tailings management as a strategic driver for decision-making in open-pit operations.

•	Optimize operational efficiency and environmental sustainability through the adoption of commingling in waste rock and tailings management.

•	Align waste rock and tailings management with international sustainability and safety standards, such as the GISTM, through the application of commingling.

•	Analyze, through comparative case studies, the impact of commingling on land-use optimization and mine life extension.

•	Reduce capital and operating costs associated with waste rock and tailings disposal by implementing commingling technologies.

•	Present the key findings and lessons learned from the integration of commingling into long-term mine planning.

3	TECHNICAL FOUNDATIONS OF COMMINGLING IN MINE PLANNING

3.1	Desing Principles and Material Characterization

Commingling at Antamina is based on engineering principles that seek to combine the superior structural properties of waste rock with the low permeability characteristics of tailings (Boshoff, 2023). This combination results in a material with shear strength similar to waste rock and permeability comparable to tailings, creating conditions that restrict oxygen entry and water filtration, significantly reducing the potential for acid drainage generation (Ulrich & Coffin, 2015; Burden & Wilson, 2023).

The characterization of commingled material is performed through analysis of five distinct structural configurations: rock-dominated uncompacted, fines-dominated uncompacted, rock-dominated compacted, fines-dominated compacted, and the "just-filled" configuration where fines exactly fill the voids between rock particles (Burden & Wilson, 2023). This latter configuration represents the optimal target of the commingling process at Antamina.

Figure 1 Blend configurations after Wickland et al. (2006). Source: Burden & Wilson (2023).

3.2	Integration into the Strategic Planning Process

The integration of commingling into Antamina's mine planning follows a structured methodological approach that places tailings management with waste rock as central elements of decision-making. This global strategy allows for optimizing the mixture of materials from the mine to the plant, considering the operational and environmental constraints associated with the tailings storage facility. By incorporating tailings behavior in the early stages of planning, greater efficiency in resource use and better risk management are achieved.

The global strategy is divided into three key axes:

•	Strategy 1: Ensure Future Expansions: The first strategy focuses on maximizing tailings storage capacity for current and future needs, leveraging Antamina's mineral resource and applying advanced technology to increase TSF (Tailings Storage Facility) capacity to ensure the mine's long-term operational continuity.

•	Strategy 2: Optimized footprint: The second strategy seeks to optimize the overall project footprint by balancing waste and tailings storage space. This approach aligns directly with commingling principles by maximizing efficient use of available land within the current operational footprint.

•	Strategy 3: Reduce upfront CAPEX: The third strategy focuses on reducing initial CAPEX to improve key financial indicators (IRR), which directly aligns with the demonstrated economic benefits of commingling, minimizing operational costs with a nearby TSF and optimizing CAPEX timing.

Figure 2 Strategic vision of mine planning approach with commingling

The strategies implemented in tailings dam and waste dump management seek to maximize the operational life of the mine and optimize asset value. These decisions translate into concrete actions that address sustainability, operational efficiency, and cost reduction, ensuring continuity and long-term success of the mining operation.

•	Long Life Asset: Antamina's vision as a long-life asset involves making decisions based on sustainable exploitation of its exceptional mineral resources. This includes adopting advanced technologies, such as commingling, to improve storage capacity and ensure operational continuity until 2036 and beyond, thus maximizing the value of the mineral resource.

•	Reduce Environmental Social Footprint: The strategic approach prioritizes reducing environmental and social impact through operation within the current footprint, increasing water availability through filtration, optimizing space use, and minimizing risks associated with tailings dams. Commingling contributes not only to technical and economic efficiency but also to sustainability objectives and corporate social responsibility.

•	Reduce Capital and Operational Cost: The cost reduction strategy focuses on optimizing capital investments (CAPEX) and their schedule, as well as minimizing operational costs through strategic facility location and leveraging synergies to avoid duplicate infrastructure. This enables more efficient and profitable resource management.

During the strategic planning phase, conceptual foundations for commingling implementation are established, including site characterization, regulatory framework development, and technical-economic feasibility analysis.

4	ANALYSIS OF THE MINING SYSTEM WITH COMMINGLING APPLICATION

4.1	Methodological Process Structure

The methodological process is organized in sequential stages that ensure effective integration of commingling into mine planning:

•	Comprehensive material evaluation: Detailed analysis of geotechnical and geochemical characteristics of available materials, identifying their suitability for mixtures and joint disposal.

•	Mixture design development: Design of optimized mixtures that maximize desired properties of commingled material, such as low permeability and high stability, minimizing acid water generation and improving mechanical behavior of the deposit.

•	Pilot test implementation: Execution of pilot and industrial scale tests to validate mixture performance and adjust design parameters according to results obtained under real operating conditions.

•	Monitoring and adjustment: Establishment of continuous monitoring system to evaluate consolidation, stability, and geochemical behavior of deposits, allowing operational and design adjustments as new information is collected.

5	IMPLEMENTATION AND EVALUATION METHODOLOGY FOR COMMINGLING IN MINE PLANNING

5.1	Comparative Case Analysis

The methodology applied at Antamina is based on Comparative Case Analysis, allowing for comprehensive review of the design and sequencing process of mining infrastructure, with emphasis on dams, waste rock dumps, and integration of commingling technologies. This methodological approach comprises:

•	Review of design and sequencing processes: Analysis of procedures used in waste dump planning and disposal, considering both geotechnical and geochemical criteria to ensure infrastructure stability and sustainability.

•	Identification of key variables: Selection of fundamental operational parameters, such as granulometry, permeability, geochemical behavior, and storage capacity, which directly affect the viability and performance of dumps and material mixtures.

•	Definition of case tree: Structure of a decision tree that contemplates long-term strategic scenarios, allowing comparison of alternatives under different combinations of variables and operational constraints.

•	Results evaluation: Application of sensitivity analyses and cost-benefit evaluations to determine the impact of each alternative on mine life extension, environmental risk reduction, and resource optimization.

5.2	Decision Tree for Strategic Planning

The decision tree developed for long-term strategic planning at Antamina follows sequential logic that allows evaluation of feasibility and impact of main operational alternatives:

•	Commingling feasibility: The first node of the tree determines whether integration of tailings and waste rock is technically and economically viable, considering material characteristics and regulatory constraints.

•	Entry into new area: If commingling is feasible, the next node evaluates the possibility of disposing material in a new area, analyzing topographic, environmental, and access aspects.

•	Need for new dam: Finally, it determines whether construction of a new tailings dam is essential or if the commingling solution allows dispensing with this infrastructure, optimizing investment and reducing environmental impact.

Each of these decisions conditions mine life extension, efficient space use, and environmental risk management, allowing selection of the alternative that maximizes economic, social, and environmental value of the project.

Figure 3  Decision Tree for Long-Term Strategic Scenario Planning

5.3	Evaluation of Operational Parameters and Modifications by Commingling

Key operational parameters—including waste rock-tailings ratio, mixing methods, transport systems, and processing criteria—are assessed for their impact on commingled system efficiency. The evaluation considers site-specific challenges such as extreme Andean weather, high-altitude logistics, and Peruvian regulations, all of which are especially critical at Antamina and require customized commingling solutions.

•	Modifications to tailings management by commingling:

The commingling process fundamentally transforms tailings management by utilizing voids within waste rock for storage, with available space typically equivalent to about 20% of the crushed rock volume transported by conveyors. To enable mixing, tailings must first be dewatered. The following image shows both the current process (without mechanization) and the transformation that occurs when commingling is incorporated.

Figure 4 Transformation of Tailings Management: Evolution from Conventional Process to Commingling|

•	 Operational parameters and assumptions

Commingling extends tailings facility life by optimizing spatial use of waste rock voids. Implementation requires mechanized systems (WCCS) for transporting crushed material mixed with dewatered tailings, while maintaining two key operational parameters: a) Unchanged transport capacity: 83.2 Mtpa in the conveyor system and b) Volumetric stability: No increase in total volume of waste rock dump.

This approach ensures efficient resource utilization and supports sustainable mine operations.

Table 1: Key operational parameters and assumptions

Component	Units	Without - Case Conventional	With - Case Commingling

Cycloned/Filtered Max. Capacity	Mtpa	-	32.0

Percentage of Voids in Waste Rock	% Volume 	 	20 %

Waste Rock / Tailings Ratio 	Mt Rock/

Mt Tails	-	83 / 16

Conventional Tailings Density	t/m3 	1.81	1.81

Fines Tailings Density	 t/m3	NA	1.60

Tailings Volume, TDR 4195 (to Dic-22)	Mm3	376	376

Tailings Capacity, TDR 4195 (to Dic-22)	Mt	682	652

Mill Feed Rate 	Ktpd	145	145

The following image shows how incorporation of mechanized systems and commingling add capacity, and therefore extend tailings facility life, to increase the life of mine and therefore the overall asset value.

Figure 5 Commingling Extension of Tailings Facility Life through Commingling

The curve below shows the relationship between dry density and optimal mixing proportion, determinant for maximizing commingling efficiency.

Figure 6: Dry density curve vs. optimal mixing proportion for commingling

•	Analysis of Mineable Resources, Production and Value

A three-dimensional diagram is used to strategically analyze the interactions between mineable resource volume, production rate, and generated value—whether economic, environmental, or operational. This tool helps identify how increasing resource volume and production can maximize value, but also highlights key constraints such as tailings dam capacity and property limits. The approach supports comprehensive decision-making to optimize asset value and ensure project sustainability across various operational scenarios.

Figure 7 Value Optimization: Interaction between Resource Volume, Production Rate and Key Restrictions

6	ECONOMIC AND OPERATIONAL ADVANTAGES

6.1	Perspective of Conventional Value Drivers

The following figure illustrates Antamina's main conventional value drivers from a financial perspective. In the current scenario, the amount of mineral resource that can be utilized directly depends on the capital that the company decides to invest (Capex), both in new investments (Growth Capex) and in maintaining existing operations (Sustaining Capex). This generates a necessary balance to determine the optimal scenario that maximizes the value of the existing asset.

Figure 8: Conventional Antamina Value Drivers: Financial Perspective (Without Commingling)

6.2	Impact of Commingling on Financial Value Drivers

From a financial perspective, the adoption of commingling among Antamina’s value drivers removes constraints on mineral resources by increasing tailings storage capacity, enabling the utilization of new dumps, and allowing for an expansion in the volume of usable resources. As a result, the mine’s operational life is extended and asset value is enhanced. Regarding dump utilization, the implementation of commingling has the potential to reduce the requirement for dumps located in karstic zones, which demand significant capital expenditure (Capex). This can lead to a reduction in initial investment by up to 30% in scenarios comparing commingling versus non-commingling approaches. This saving is achieved through the integration of previously separate infrastructures into a single system, which optimizes the use of available area in the East Extension dumps and prioritizes the use of mechanized systems over conventional trucking methods.

Figure 9: Antamina Value Drivers: Financial Perspective with Commingling

The optimization of land use represents another important economic benefit, especially relevant in mountainous locations like Antamina where available land for mining facilities is limited. Commingling allows for the creation of deposits with a smaller footprint compared to separate facilities, freeing up land for other productive uses or reducing the need for additional land acquisition.

7	ALIGNMENT WITH GISTM STANDARDS AND SUSTAINABILITY

The integration of commingling into mine planning strengthens compliance with the Global Industry Standard on Tailings Management (GISTM), by facilitating safer and more sustainable management of waste materials. The GISTM requires the planning, construction, operation, and closure of tailings facilities with a focus on risk reduction and continuous monitoring throughout the lifecycle. Commingling contributes to this objective by improving the geotechnical and geochemical stability of deposits, reducing the risk of failure and acid drainage generation, which in turn reduces environmental impact and long-term treatment costs. Additionally, the consolidation of facilities through commingling reduces the environmental footprint and facilitates supervision and control, aligning with the principles of sustainability and social responsibility in the mining industry.

8	LESSONS LEARNED AND CHALLENGES

Among the main lessons learned and challenges of implementing commingling in mine planning at Antamina, the following stand out:

•	Large-scale operations and mechanization: Commingling is primarily viable in large-volume mechanized operations, where it is possible to achieve a homogeneous and controlled mixture of materials. However, the magnitude of waste rock and tailings tonnages at Antamina implies a new challenge, never seen before.

•	Flexible planning and pilot tests: It is essential to include areas for pilot tests in short and medium-term plans, allowing adjustment of mixing parameters according to material variability.

•	Regulatory communication: Technology acceptance requires identifying precedents and working closely with authorities to develop appropriate regulatory frameworks.

•	Organizational change management: The transition to commingling involves challenges in personnel training and adaptation of operational processes, especially in mature operations.

9	CONCLUSIONS

•	The comparative discussion and case analysis of commingling implementation at Antamina have enabled the identification and selection of strategic alternatives that maximize the economic, environmental, and social value of the project.

•	The use of decision trees and scenario evaluation has facilitated informed decision-making, taking into account technical and economic feasibility as well as regulatory and environmental constraints.

•	The integration of commingling into strategic mine planning has proven to be a key tool for optimizing waste rock and tailings management, achieving a significant reduction in capital costs (up to 30%), greater efficiency in land use, and an extension of tailings storage facility (TSF) life. This approach has also aligned operations with international sustainability standards (GISTM) and strengthened risk management, contributing to operational continuity and long-term reduction of environmental impacts.

•	The Antamina experience demonstrates that structured case discussions and the application of comparative methodologies are fundamental for strategic planning, as they allow anticipation of challenges, validation of solutions through pilot testing, and adjustment of operational parameters based on real outcomes. Thus, commingling is consolidated as a transformative practice in modern mining, fostering more robust and sustainable decision-making for the development of long-life mining assets.

ACKNOWLEDGEMENTS

The authors express their sincere gratitude to Antamina for their invaluable support. Special thanks are extended to the Tailings Strategy Management team and the Long-Term Planning team for their collaboration and insights.

REFERENCES

Boshoff, J. et al. (2023). A case study on the commingling of tailings and waste rock at a Brownfields open cast mine in Ghana. University of Alberta.

Burden, R., & Wilson, G. W. (2023). Commingling of waste rock and tailings to improve “dry stack” performance: Design and evaluation of mixtures. Minerals, 13(2), 295.

Deza, N., & Montes, K. (2022). Mecanización del desmonte en Compañía Minera Antamina: Propuesta para capturar el valor tangible e intangible de la mecanización. Compañía Minera Antamina.

Green Policy Platform. (2024). Knowledge Gaps Report: Environmental Aspects of Tailings Management (January 2024).

ICMM. (2020). New global industry standard on tailings management. International Council on Mining and Metals.

Ulrich, B. & Coffin, J. (2015). TMW 2015 – Combined Tailings and Mine Waste.

Minemax. (2015). Why mine planning is all about collaboration. Minemax News.

Wheaton Precious Metals. (2015). Antamina Project overview.

INTEGRATING COMMINGLING INTO THE STRATEGIC WASTE ROCK PLANNING AT ANTAMINA MINE

Christa Quiroz1, David Machin1, Olimpia Cabrera1, Fernando Angeles1

1 Antamina Mine, Lima, Peru

(*Presenting author: cquirozc@antamina.com)

ABSTRACT

Antamina is a large open-pit polymetallic mine in Peru, operating with a processing capacity of 145 ktpd. In operation since 2001, the current mine life extends to 2036. Mining is conducted at a rate of 290 Mtpa using conventional truck-and-shovel equipment. This technical paper presents a comprehensive assessment of commingling as a transformative technology in mine planning, positioning waste rock and tailings management as a key factor in the decision-making process. The primary objective of this study is to demonstrate the benefits of this technology within the mine planning process, highlighting waste rock and tailings management as one of the main drivers in strategic decision-making. The methodology applied is a Comparative Case Analysis, which includes a review of the design and sequencing process for mining infrastructure such as waste rock storage facilities, along with a discussion of the key parameters considered. The results show that the implementation of this technology delivers significant economic benefits, including an approximate 30% reduction in capital costs for waste rock storage facilities, improved land-use efficiency, and optimized infrastructure development. Furthermore, the approach enhances alignment with the Global Industry Standard on Tailings Management (GISTM).

KEYWORDS

commingling, mine planning, waste rock management, tailings management, GISTM compliance

10	INTRODUCTION

The mining industry faces growing challenges related to the sustainable management of waste rock and tailings, especially after catastrophic events such as those that occurred in Brumadinho and Samarco (Green Policy Platform, 2024; ICMM, 2020). In this context, the Antamina mine, operating since 2001 as one of the largest polymetallic operations in the world, has developed a pioneering approach that integrates commingling as a central element of its strategic planning (Wheaton Precious Metals, 2015).

Commingling, defined as the homogeneous mixing of dewatered tailings with waste rock to form a geotechnically stable material, represents a natural evolution of traditional co-disposal practices (Burden & Wilson, 2023). However, what distinguishes the Antamina case is the integration of this technology from the initial phases of mine planning, making waste rock management the main driver of operational and strategic decisions.

11	OBJECTIVES

•	Demonstrate the benefits of integrating commingling into mine planning, positioning waste rock and tailings management as a strategic driver for decision-making in open-pit operations.

•	Optimize operational efficiency and environmental sustainability through the adoption of commingling in waste rock and tailings management.

•	Align waste rock and tailings management with international sustainability and safety standards, such as the GISTM, through the application of commingling.

•	Analyze, through comparative case studies, the impact of commingling on land-use optimization and mine life extension.

•	Reduce capital and operating costs associated with waste rock and tailings disposal by implementing commingling technologies.

•	Present the key findings and lessons learned from the integration of commingling into long-term mine planning.

12	TECHNICAL FOUNDATIONS OF COMMINGLING IN MINE PLANNING

12.1	Desing Principles and Material Characterization

Commingling at Antamina is based on engineering principles that seek to combine the superior structural properties of waste rock with the low permeability characteristics of tailings (Boshoff, 2023). This combination results in a material with shear strength similar to waste rock and permeability comparable to tailings, creating conditions that restrict oxygen entry and water filtration, significantly reducing the potential for acid drainage generation (Ulrich & Coffin, 2015; Burden & Wilson, 2023).

The characterization of commingled material is performed through analysis of five distinct structural configurations: rock-dominated uncompacted, fines-dominated uncompacted, rock-dominated compacted, fines-dominated compacted, and the "just-filled" configuration where fines exactly fill the voids between rock particles (Burden & Wilson, 2023). This latter configuration represents the optimal target of the commingling process at Antamina.

Figure 1 Blend configurations after Wickland et al. (2006). Source: Burden & Wilson (2023).

12.2	Integration into the Strategic Planning Process

The integration of commingling into Antamina's mine planning follows a structured methodological approach that places tailings management with waste rock as central elements of decision-making. This global strategy allows for optimizing the mixture of materials from the mine to the plant, considering the operational and environmental constraints associated with the tailings storage facility. By incorporating tailings behavior in the early stages of planning, greater efficiency in resource use and better risk management are achieved.

The global strategy is divided into three key axes:

•	Strategy 1: Ensure Future Expansions: The first strategy focuses on maximizing tailings storage capacity for current and future needs, leveraging Antamina's mineral resource and applying advanced technology to increase TSF (Tailings Storage Facility) capacity to ensure the mine's long-term operational continuity.

•	Strategy 2: Optimized footprint: The second strategy seeks to optimize the overall project footprint by balancing waste and tailings storage space. This approach aligns directly with commingling principles by maximizing efficient use of available land within the current operational footprint.

•	Strategy 3: Reduce upfront CAPEX: The third strategy focuses on reducing initial CAPEX to improve key financial indicators (IRR), which directly aligns with the demonstrated economic benefits of commingling, minimizing operational costs with a nearby TSF and optimizing CAPEX timing.

Figure 2 Strategic vision of mine planning approach with commingling

The strategies implemented in tailings dam and waste dump management seek to maximize the operational life of the mine and optimize asset value. These decisions translate into concrete actions that address sustainability, operational efficiency, and cost reduction, ensuring continuity and long-term success of the mining operation.

•	Long Life Asset: Antamina's vision as a long-life asset involves making decisions based on sustainable exploitation of its exceptional mineral resources. This includes adopting advanced technologies, such as commingling, to improve storage capacity and ensure operational continuity until 2036 and beyond, thus maximizing the value of the mineral resource.

•	Reduce Environmental Social Footprint: The strategic approach prioritizes reducing environmental and social impact through operation within the current footprint, increasing water availability through filtration, optimizing space use, and minimizing risks associated with tailings dams. Commingling contributes not only to technical and economic efficiency but also to sustainability objectives and corporate social responsibility.

•	Reduce Capital and Operational Cost: The cost reduction strategy focuses on optimizing capital investments (CAPEX) and their schedule, as well as minimizing operational costs through strategic facility location and leveraging synergies to avoid duplicate infrastructure. This enables more efficient and profitable resource management.

During the strategic planning phase, conceptual foundations for commingling implementation are established, including site characterization, regulatory framework development, and technical-economic feasibility analysis.

13	ANALYSIS OF THE MINING SYSTEM WITH COMMINGLING APPLICATION

13.1	Methodological Process Structure

The methodological process is organized in sequential stages that ensure effective integration of commingling into mine planning:

•	Comprehensive material evaluation: Detailed analysis of geotechnical and geochemical characteristics of available materials, identifying their suitability for mixtures and joint disposal.

•	Mixture design development: Design of optimized mixtures that maximize desired properties of commingled material, such as low permeability and high stability, minimizing acid water generation and improving mechanical behavior of the deposit.

•	Pilot test implementation: Execution of pilot and industrial scale tests to validate mixture performance and adjust design parameters according to results obtained under real operating conditions.

•	Monitoring and adjustment: Establishment of continuous monitoring system to evaluate consolidation, stability, and geochemical behavior of deposits, allowing operational and design adjustments as new information is collected.

14	IMPLEMENTATION AND EVALUATION METHODOLOGY FOR COMMINGLING IN MINE PLANNING

14.1	Comparative Case Analysis

The methodology applied at Antamina is based on Comparative Case Analysis, allowing for comprehensive review of the design and sequencing process of mining infrastructure, with emphasis on dams, waste rock dumps, and integration of commingling technologies. This methodological approach comprises:

•	Review of design and sequencing processes: Analysis of procedures used in waste dump planning and disposal, considering both geotechnical and geochemical criteria to ensure infrastructure stability and sustainability.

•	Identification of key variables: Selection of fundamental operational parameters, such as granulometry, permeability, geochemical behavior, and storage capacity, which directly affect the viability and performance of dumps and material mixtures.

•	Definition of case tree: Structure of a decision tree that contemplates long-term strategic scenarios, allowing comparison of alternatives under different combinations of variables and operational constraints.

•	Results evaluation: Application of sensitivity analyses and cost-benefit evaluations to determine the impact of each alternative on mine life extension, environmental risk reduction, and resource optimization.

14.2	Decision Tree for Strategic Planning

The decision tree developed for long-term strategic planning at Antamina follows sequential logic that allows evaluation of feasibility and impact of main operational alternatives:

•	Commingling feasibility: The first node of the tree determines whether integration of tailings and waste rock is technically and economically viable, considering material characteristics and regulatory constraints.

•	Entry into new area: If commingling is feasible, the next node evaluates the possibility of disposing material in a new area, analyzing topographic, environmental, and access aspects.

•	Need for new dam: Finally, it determines whether construction of a new tailings dam is essential or if the commingling solution allows dispensing with this infrastructure, optimizing investment and reducing environmental impact.

Each of these decisions conditions mine life extension, efficient space use, and environmental risk management, allowing selection of the alternative that maximizes economic, social, and environmental value of the project.

Figure 3  Decision Tree for Long-Term Strategic Scenario Planning

14.3	Evaluation of Operational Parameters and Modifications by Commingling

Key operational parameters—including waste rock-tailings ratio, mixing methods, transport systems, and processing criteria—are assessed for their impact on commingled system efficiency. The evaluation considers site-specific challenges such as extreme Andean weather, high-altitude logistics, and Peruvian regulations, all of which are especially critical at Antamina and require customized commingling solutions.

•	Modifications to tailings management by commingling:

The commingling process fundamentally transforms tailings management by utilizing voids within waste rock for storage, with available space typically equivalent to about 20% of the crushed rock volume transported by conveyors. To enable mixing, tailings must first be dewatered. The following image shows both the current process (without mechanization) and the transformation that occurs when commingling is incorporated.

Figure 4 Transformation of Tailings Management: Evolution from Conventional Process to Commingling|

•	 Operational parameters and assumptions

Commingling extends tailings facility life by optimizing spatial use of waste rock voids. Implementation requires mechanized systems (WCCS) for transporting crushed material mixed with dewatered tailings, while maintaining two key operational parameters: a) Unchanged transport capacity: 83.2 Mtpa in the conveyor system and b) Volumetric stability: No increase in total volume of waste rock dump.

This approach ensures efficient resource utilization and supports sustainable mine operations.

Table 1: Key operational parameters and assumptions

Component	Units	Without - Case Conventional	With - Case Commingling

Cycloned/Filtered Max. Capacity	Mtpa	-	32.0

Percentage of Voids in Waste Rock	% Volume 	 	20 %

Waste Rock / Tailings Ratio 	Mt Rock/

Mt Tails	-	83 / 16

Conventional Tailings Density	t/m3 	1.81	1.81

Fines Tailings Density	 t/m3	NA	1.60

Tailings Volume, TDR 4195 (to Dic-22)	Mm3	376	376

Tailings Capacity, TDR 4195 (to Dic-22)	Mt	682	652

Mill Feed Rate 	Ktpd	145	145

The following image shows how incorporation of mechanized systems and commingling add capacity, and therefore extend tailings facility life, to increase the life of mine and therefore the overall asset value.

Figure 5 Commingling Extension of Tailings Facility Life through Commingling

The curve below shows the relationship between dry density and optimal mixing proportion, determinant for maximizing commingling efficiency.

Figure 6: Dry density curve vs. optimal mixing proportion for commingling

•	Analysis of Mineable Resources, Production and Value

A three-dimensional diagram is used to strategically analyze the interactions between mineable resource volume, production rate, and generated value—whether economic, environmental, or operational. This tool helps identify how increasing resource volume and production can maximize value, but also highlights key constraints such as tailings dam capacity and property limits. The approach supports comprehensive decision-making to optimize asset value and ensure project sustainability across various operational scenarios.

Figure 7 Value Optimization: Interaction between Resource Volume, Production Rate and Key Restrictions

15	ECONOMIC AND OPERATIONAL ADVANTAGES

15.1	Perspective of Conventional Value Drivers

The following figure illustrates Antamina's main conventional value drivers from a financial perspective. In the current scenario, the amount of mineral resource that can be utilized directly depends on the capital that the company decides to invest (Capex), both in new investments (Growth Capex) and in maintaining existing operations (Sustaining Capex). This generates a necessary balance to determine the optimal scenario that maximizes the value of the existing asset.

Figure 8: Conventional Antamina Value Drivers: Financial Perspective (Without Commingling)

15.2	Impact of Commingling on Financial Value Drivers

From a financial perspective, the adoption of commingling among Antamina’s value drivers removes constraints on mineral resources by increasing tailings storage capacity, enabling the utilization of new dumps, and allowing for an expansion in the volume of usable resources. As a result, the mine’s operational life is extended and asset value is enhanced. Regarding dump utilization, the implementation of commingling has the potential to reduce the requirement for dumps located in karstic zones, which demand significant capital expenditure (Capex). This can lead to a reduction in initial investment by up to 30% in scenarios comparing commingling versus non-commingling approaches. This saving is achieved through the integration of previously separate infrastructures into a single system, which optimizes the use of available area in the East Extension dumps and prioritizes the use of mechanized systems over conventional trucking methods.

Figure 9: Antamina Value Drivers: Financial Perspective with Commingling

The optimization of land use represents another important economic benefit, especially relevant in mountainous locations like Antamina where available land for mining facilities is limited. Commingling allows for the creation of deposits with a smaller footprint compared to separate facilities, freeing up land for other productive uses or reducing the need for additional land acquisition.

16	ALIGNMENT WITH GISTM STANDARDS AND SUSTAINABILITY

The integration of commingling into mine planning strengthens compliance with the Global Industry Standard on Tailings Management (GISTM), by facilitating safer and more sustainable management of waste materials. The GISTM requires the planning, construction, operation, and closure of tailings facilities with a focus on risk reduction and continuous monitoring throughout the lifecycle. Commingling contributes to this objective by improving the geotechnical and geochemical stability of deposits, reducing the risk of failure and acid drainage generation, which in turn reduces environmental impact and long-term treatment costs. Additionally, the consolidation of facilities through commingling reduces the environmental footprint and facilitates supervision and control, aligning with the principles of sustainability and social responsibility in the mining industry.

17	LESSONS LEARNED AND CHALLENGES

Among the main lessons learned and challenges of implementing commingling in mine planning at Antamina, the following stand out:

•	Large-scale operations and mechanization: Commingling is primarily viable in large-volume mechanized operations, where it is possible to achieve a homogeneous and controlled mixture of materials. However, the magnitude of waste rock and tailings tonnages at Antamina implies a new challenge, never seen before.

•	Flexible planning and pilot tests: It is essential to include areas for pilot tests in short and medium-term plans, allowing adjustment of mixing parameters according to material variability.

•	Regulatory communication: Technology acceptance requires identifying precedents and working closely with authorities to develop appropriate regulatory frameworks.

•	Organizational change management: The transition to commingling involves challenges in personnel training and adaptation of operational processes, especially in mature operations.

18	CONCLUSIONS

•	The comparative discussion and case analysis of commingling implementation at Antamina have enabled the identification and selection of strategic alternatives that maximize the economic, environmental, and social value of the project.

•	The use of decision trees and scenario evaluation has facilitated informed decision-making, taking into account technical and economic feasibility as well as regulatory and environmental constraints.

•	The integration of commingling into strategic mine planning has proven to be a key tool for optimizing waste rock and tailings management, achieving a significant reduction in capital costs (up to 30%), greater efficiency in land use, and an extension of tailings storage facility (TSF) life. This approach has also aligned operations with international sustainability standards (GISTM) and strengthened risk management, contributing to operational continuity and long-term reduction of environmental impacts.

•	The Antamina experience demonstrates that structured case discussions and the application of comparative methodologies are fundamental for strategic planning, as they allow anticipation of challenges, validation of solutions through pilot testing, and adjustment of operational parameters based on real outcomes. Thus, commingling is consolidated as a transformative practice in modern mining, fostering more robust and sustainable decision-making for the development of long-life mining assets.

ACKNOWLEDGEMENTS

The authors express their sincere gratitude to Antamina for their invaluable support. Special thanks are extended to the Tailings Strategy Management team and the Long-Term Planning team for their collaboration and insights.

REFERENCES

Boshoff, J. et al. (2023). A case study on the commingling of tailings and waste rock at a Brownfields open cast mine in Ghana. University of Alberta.

Burden, R., & Wilson, G. W. (2023). Commingling of waste rock and tailings to improve “dry stack” performance: Design and evaluation of mixtures. Minerals, 13(2), 295.

Deza, N., & Montes, K. (2022). Mecanización del desmonte en Compañía Minera Antamina: Propuesta para capturar el valor tangible e intangible de la mecanización. Compañía Minera Antamina.

Green Policy Platform. (2024). Knowledge Gaps Report: Environmental Aspects of Tailings Management (January 2024).

ICMM. (2020). New global industry standard on tailings management. International Council on Mining and Metals.

Ulrich, B. & Coffin, J. (2015). TMW 2015 – Combined Tailings and Mine Waste.

Minemax. (2015). Why mine planning is all about collaboration. Minemax News.

Wheaton Precious Metals. (2015). Antamina Project overview.

INTEGRATING COMMINGLING INTO THE STRATEGIC WASTE ROCK PLANNING AT ANTAMINA MINE

Christa Quiroz1, David Machin1, Olimpia Cabrera1, Fernando Angeles1

1 Antamina Mine, Lima, Peru

(*Presenting author: cquirozc@antamina.com)

ABSTRACT

Antamina is a large open-pit polymetallic mine in Peru, operating with a processing capacity of 145 ktpd. In operation since 2001, the current mine life extends to 2036. Mining is conducted at a rate of 290 Mtpa using conventional truck-and-shovel equipment. This technical paper presents a comprehensive assessment of commingling as a transformative technology in mine planning, positioning waste rock and tailings management as a key factor in the decision-making process. The primary objective of this study is to demonstrate the benefits of this technology within the mine planning process, highlighting waste rock and tailings management as one of the main drivers in strategic decision-making. The methodology applied is a Comparative Case Analysis, which includes a review of the design and sequencing process for mining infrastructure such as waste rock storage facilities, along with a discussion of the key parameters considered. The results show that the implementation of this technology delivers significant economic benefits, including an approximate 30% reduction in capital costs for waste rock storage facilities, improved land-use efficiency, and optimized infrastructure development. Furthermore, the approach enhances alignment with the Global Industry Standard on Tailings Management (GISTM).

KEYWORDS

commingling, mine planning, waste rock management, tailings management, GISTM compliance

1	INTRODUCTION

The mining industry faces growing challenges related to the sustainable management of waste rock and tailings, especially after catastrophic events such as those that occurred in Brumadinho and Samarco (Green Policy Platform, 2024; ICMM, 2020). In this context, the Antamina mine, operating since 2001 as one of the largest polymetallic operations in the world, has developed a pioneering approach that integrates commingling as a central element of its strategic planning (Wheaton Precious Metals, 2015).

Commingling, defined as the homogeneous mixing of dewatered tailings with waste rock to form a geotechnically stable material, represents a natural evolution of traditional co-disposal practices (Burden & Wilson, 2023). However, what distinguishes the Antamina case is the integration of this technology from the initial phases of mine planning, making waste rock management the main driver of operational and strategic decisions.

2	OBJECTIVES

•	Demonstrate the benefits of integrating commingling into mine planning, positioning waste rock and tailings management as a strategic driver for decision-making in open-pit operations.

•	Optimize operational efficiency and environmental sustainability through the adoption of commingling in waste rock and tailings management.

•	Align waste rock and tailings management with international sustainability and safety standards, such as the GISTM, through the application of commingling.

•	Analyze, through comparative case studies, the impact of commingling on land-use optimization and mine life extension.

•	Reduce capital and operating costs associated with waste rock and tailings disposal by implementing commingling technologies.

•	Present the key findings and lessons learned from the integration of commingling into long-term mine planning.

3	TECHNICAL FOUNDATIONS OF COMMINGLING IN MINE PLANNING

3.1	Desing Principles and Material Characterization

Commingling at Antamina is based on engineering principles that seek to combine the superior structural properties of waste rock with the low permeability characteristics of tailings (Boshoff, 2023). This combination results in a material with shear strength similar to waste rock and permeability comparable to tailings, creating conditions that restrict oxygen entry and water filtration, significantly reducing the potential for acid drainage generation (Ulrich & Coffin, 2015; Burden & Wilson, 2023).

The characterization of commingled material is performed through analysis of five distinct structural configurations: rock-dominated uncompacted, fines-dominated uncompacted, rock-dominated compacted, fines-dominated compacted, and the "just-filled" configuration where fines exactly fill the voids between rock particles (Burden & Wilson, 2023). This latter configuration represents the optimal target of the commingling process at Antamina.

Figure 1 Blend configurations after Wickland et al. (2006). Source: Burden & Wilson (2023).

3.2	Integration into the Strategic Planning Process

The integration of commingling into Antamina's mine planning follows a structured methodological approach that places tailings management with waste rock as central elements of decision-making. This global strategy allows for optimizing the mixture of materials from the mine to the plant, considering the operational and environmental constraints associated with the tailings storage facility. By incorporating tailings behavior in the early stages of planning, greater efficiency in resource use and better risk management are achieved.

The global strategy is divided into three key axes:

•	Strategy 1: Ensure Future Expansions: The first strategy focuses on maximizing tailings storage capacity for current and future needs, leveraging Antamina's mineral resource and applying advanced technology to increase TSF (Tailings Storage Facility) capacity to ensure the mine's long-term operational continuity.

•	Strategy 2: Optimized footprint: The second strategy seeks to optimize the overall project footprint by balancing waste and tailings storage space. This approach aligns directly with commingling principles by maximizing efficient use of available land within the current operational footprint.

•	Strategy 3: Reduce upfront CAPEX: The third strategy focuses on reducing initial CAPEX to improve key financial indicators (IRR), which directly aligns with the demonstrated economic benefits of commingling, minimizing operational costs with a nearby TSF and optimizing CAPEX timing.

Figure 2 Strategic vision of mine planning approach with commingling

The strategies implemented in tailings dam and waste dump management seek to maximize the operational life of the mine and optimize asset value. These decisions translate into concrete actions that address sustainability, operational efficiency, and cost reduction, ensuring continuity and long-term success of the mining operation.

•	Long Life Asset: Antamina's vision as a long-life asset involves making decisions based on sustainable exploitation of its exceptional mineral resources. This includes adopting advanced technologies, such as commingling, to improve storage capacity and ensure operational continuity until 2036 and beyond, thus maximizing the value of the mineral resource.

•	Reduce Environmental Social Footprint: The strategic approach prioritizes reducing environmental and social impact through operation within the current footprint, increasing water availability through filtration, optimizing space use, and minimizing risks associated with tailings dams. Commingling contributes not only to technical and economic efficiency but also to sustainability objectives and corporate social responsibility.

•	Reduce Capital and Operational Cost: The cost reduction strategy focuses on optimizing capital investments (CAPEX) and their schedule, as well as minimizing operational costs through strategic facility location and leveraging synergies to avoid duplicate infrastructure. This enables more efficient and profitable resource management.

During the strategic planning phase, conceptual foundations for commingling implementation are established, including site characterization, regulatory framework development, and technical-economic feasibility analysis.

4	ANALYSIS OF THE MINING SYSTEM WITH COMMINGLING APPLICATION

4.1	Methodological Process Structure

The methodological process is organized in sequential stages that ensure effective integration of commingling into mine planning:

•	Comprehensive material evaluation: Detailed analysis of geotechnical and geochemical characteristics of available materials, identifying their suitability for mixtures and joint disposal.

•	Mixture design development: Design of optimized mixtures that maximize desired properties of commingled material, such as low permeability and high stability, minimizing acid water generation and improving mechanical behavior of the deposit.

•	Pilot test implementation: Execution of pilot and industrial scale tests to validate mixture performance and adjust design parameters according to results obtained under real operating conditions.

•	Monitoring and adjustment: Establishment of continuous monitoring system to evaluate consolidation, stability, and geochemical behavior of deposits, allowing operational and design adjustments as new information is collected.

5	IMPLEMENTATION AND EVALUATION METHODOLOGY FOR COMMINGLING IN MINE PLANNING

5.1	Comparative Case Analysis

The methodology applied at Antamina is based on Comparative Case Analysis, allowing for comprehensive review of the design and sequencing process of mining infrastructure, with emphasis on dams, waste rock dumps, and integration of commingling technologies. This methodological approach comprises:

•	Review of design and sequencing processes: Analysis of procedures used in waste dump planning and disposal, considering both geotechnical and geochemical criteria to ensure infrastructure stability and sustainability.

•	Identification of key variables: Selection of fundamental operational parameters, such as granulometry, permeability, geochemical behavior, and storage capacity, which directly affect the viability and performance of dumps and material mixtures.

•	Definition of case tree: Structure of a decision tree that contemplates long-term strategic scenarios, allowing comparison of alternatives under different combinations of variables and operational constraints.

•	Results evaluation: Application of sensitivity analyses and cost-benefit evaluations to determine the impact of each alternative on mine life extension, environmental risk reduction, and resource optimization.

5.2	Decision Tree for Strategic Planning

The decision tree developed for long-term strategic planning at Antamina follows sequential logic that allows evaluation of feasibility and impact of main operational alternatives:

•	Commingling feasibility: The first node of the tree determines whether integration of tailings and waste rock is technically and economically viable, considering material characteristics and regulatory constraints.

•	Entry into new area: If commingling is feasible, the next node evaluates the possibility of disposing material in a new area, analyzing topographic, environmental, and access aspects.

•	Need for new dam: Finally, it determines whether construction of a new tailings dam is essential or if the commingling solution allows dispensing with this infrastructure, optimizing investment and reducing environmental impact.

Each of these decisions conditions mine life extension, efficient space use, and environmental risk management, allowing selection of the alternative that maximizes economic, social, and environmental value of the project.

Figure 3  Decision Tree for Long-Term Strategic Scenario Planning

5.3	Evaluation of Operational Parameters and Modifications by Commingling

Key operational parameters—including waste rock-tailings ratio, mixing methods, transport systems, and processing criteria—are assessed for their impact on commingled system efficiency. The evaluation considers site-specific challenges such as extreme Andean weather, high-altitude logistics, and Peruvian regulations, all of which are especially critical at Antamina and require customized commingling solutions.

•	Modifications to tailings management by commingling:

The commingling process fundamentally transforms tailings management by utilizing voids within waste rock for storage, with available space typically equivalent to about 20% of the crushed rock volume transported by conveyors. To enable mixing, tailings must first be dewatered. The following image shows both the current process (without mechanization) and the transformation that occurs when commingling is incorporated.

Figure 4 Transformation of Tailings Management: Evolution from Conventional Process to Commingling|

•	 Operational parameters and assumptions

Commingling extends tailings facility life by optimizing spatial use of waste rock voids. Implementation requires mechanized systems (WCCS) for transporting crushed material mixed with dewatered tailings, while maintaining two key operational parameters: a) Unchanged transport capacity: 83.2 Mtpa in the conveyor system and b) Volumetric stability: No increase in total volume of waste rock dump.

This approach ensures efficient resource utilization and supports sustainable mine operations.

Table 1: Key operational parameters and assumptions

Component	Units	Without - Case Conventional	With - Case Commingling

Cycloned/Filtered Max. Capacity	Mtpa	-	32.0

Percentage of Voids in Waste Rock	% Volume 	 	20 %

Waste Rock / Tailings Ratio 	Mt Rock/

Mt Tails	-	83 / 16

Conventional Tailings Density	t/m3 	1.81	1.81

Fines Tailings Density	 t/m3	NA	1.60

Tailings Volume, TDR 4195 (to Dic-22)	Mm3	376	376

Tailings Capacity, TDR 4195 (to Dic-22)	Mt	682	652

Mill Feed Rate 	Ktpd	145	145

The following image shows how incorporation of mechanized systems and commingling add capacity, and therefore extend tailings facility life, to increase the life of mine and therefore the overall asset value.

Figure 5 Commingling Extension of Tailings Facility Life through Commingling

The curve below shows the relationship between dry density and optimal mixing proportion, determinant for maximizing commingling efficiency.

Figure 6: Dry density curve vs. optimal mixing proportion for commingling

•	Analysis of Mineable Resources, Production and Value

A three-dimensional diagram is used to strategically analyze the interactions between mineable resource volume, production rate, and generated value—whether economic, environmental, or operational. This tool helps identify how increasing resource volume and production can maximize value, but also highlights key constraints such as tailings dam capacity and property limits. The approach supports comprehensive decision-making to optimize asset value and ensure project sustainability across various operational scenarios.

Figure 7 Value Optimization: Interaction between Resource Volume, Production Rate and Key Restrictions

6	ECONOMIC AND OPERATIONAL ADVANTAGES

6.1	Perspective of Conventional Value Drivers

The following figure illustrates Antamina's main conventional value drivers from a financial perspective. In the current scenario, the amount of mineral resource that can be utilized directly depends on the capital that the company decides to invest (Capex), both in new investments (Growth Capex) and in maintaining existing operations (Sustaining Capex). This generates a necessary balance to determine the optimal scenario that maximizes the value of the existing asset.

Figure 8: Conventional Antamina Value Drivers: Financial Perspective (Without Commingling)

6.2	Impact of Commingling on Financial Value Drivers

From a financial perspective, the adoption of commingling among Antamina’s value drivers removes constraints on mineral resources by increasing tailings storage capacity, enabling the utilization of new dumps, and allowing for an expansion in the volume of usable resources. As a result, the mine’s operational life is extended and asset value is enhanced. Regarding dump utilization, the implementation of commingling has the potential to reduce the requirement for dumps located in karstic zones, which demand significant capital expenditure (Capex). This can lead to a reduction in initial investment by up to 30% in scenarios comparing commingling versus non-commingling approaches. This saving is achieved through the integration of previously separate infrastructures into a single system, which optimizes the use of available area in the East Extension dumps and prioritizes the use of mechanized systems over conventional trucking methods.

Figure 9: Antamina Value Drivers: Financial Perspective with Commingling

The optimization of land use represents another important economic benefit, especially relevant in mountainous locations like Antamina where available land for mining facilities is limited. Commingling allows for the creation of deposits with a smaller footprint compared to separate facilities, freeing up land for other productive uses or reducing the need for additional land acquisition.

7	ALIGNMENT WITH GISTM STANDARDS AND SUSTAINABILITY

The integration of commingling into mine planning strengthens compliance with the Global Industry Standard on Tailings Management (GISTM), by facilitating safer and more sustainable management of waste materials. The GISTM requires the planning, construction, operation, and closure of tailings facilities with a focus on risk reduction and continuous monitoring throughout the lifecycle. Commingling contributes to this objective by improving the geotechnical and geochemical stability of deposits, reducing the risk of failure and acid drainage generation, which in turn reduces environmental impact and long-term treatment costs. Additionally, the consolidation of facilities through commingling reduces the environmental footprint and facilitates supervision and control, aligning with the principles of sustainability and social responsibility in the mining industry.

8	LESSONS LEARNED AND CHALLENGES

Among the main lessons learned and challenges of implementing commingling in mine planning at Antamina, the following stand out:

•	Large-scale operations and mechanization: Commingling is primarily viable in large-volume mechanized operations, where it is possible to achieve a homogeneous and controlled mixture of materials. However, the magnitude of waste rock and tailings tonnages at Antamina implies a new challenge, never seen before.

•	Flexible planning and pilot tests: It is essential to include areas for pilot tests in short and medium-term plans, allowing adjustment of mixing parameters according to material variability.

•	Regulatory communication: Technology acceptance requires identifying precedents and working closely with authorities to develop appropriate regulatory frameworks.

•	Organizational change management: The transition to commingling involves challenges in personnel training and adaptation of operational processes, especially in mature operations.

9	CONCLUSIONS

•	The comparative discussion and case analysis of commingling implementation at Antamina have enabled the identification and selection of strategic alternatives that maximize the economic, environmental, and social value of the project.

•	The use of decision trees and scenario evaluation has facilitated informed decision-making, taking into account technical and economic feasibility as well as regulatory and environmental constraints.

•	The integration of commingling into strategic mine planning has proven to be a key tool for optimizing waste rock and tailings management, achieving a significant reduction in capital costs (up to 30%), greater efficiency in land use, and an extension of tailings storage facility (TSF) life. This approach has also aligned operations with international sustainability standards (GISTM) and strengthened risk management, contributing to operational continuity and long-term reduction of environmental impacts.

•	The Antamina experience demonstrates that structured case discussions and the application of comparative methodologies are fundamental for strategic planning, as they allow anticipation of challenges, validation of solutions through pilot testing, and adjustment of operational parameters based on real outcomes. Thus, commingling is consolidated as a transformative practice in modern mining, fostering more robust and sustainable decision-making for the development of long-life mining assets.

ACKNOWLEDGEMENTS

The authors express their sincere gratitude to Antamina for their invaluable support. Special thanks are extended to the Tailings Strategy Management team and the Long-Term Planning team for their collaboration and insights.

REFERENCES

Boshoff, J. et al. (2023). A case study on the commingling of tailings and waste rock at a Brownfields open cast mine in Ghana. University of Alberta.

Burden, R., & Wilson, G. W. (2023). Commingling of waste rock and tailings to improve “dry stack” performance: Design and evaluation of mixtures. Minerals, 13(2), 295.

Deza, N., & Montes, K. (2022). Mecanización del desmonte en Compañía Minera Antamina: Propuesta para capturar el valor tangible e intangible de la mecanización. Compañía Minera Antamina.

Green Policy Platform. (2024). Knowledge Gaps Report: Environmental Aspects of Tailings Management (January 2024).

ICMM. (2020). New global industry standard on tailings management. International Council on Mining and Metals.

Ulrich, B. & Coffin, J. (2015). TMW 2015 – Combined Tailings and Mine Waste.

Minemax. (2015). Why mine planning is all about collaboration. Minemax News.

Wheaton Precious Metals. (2015). Antamina Project overview.

INTEGRATING COMMINGLING INTO THE STRATEGIC WASTE ROCK PLANNING AT ANTAMINA MINE

Christa Quiroz1, David Machin1, Olimpia Cabrera1, Fernando Angeles1

1 Antamina Mine, Lima, Peru

(*Presenting author: cquirozc@antamina.com)

ABSTRACT

Antamina is a large open-pit polymetallic mine in Peru, operating with a processing capacity of 145 ktpd. In operation since 2001, the current mine life extends to 2036. Mining is conducted at a rate of 290 Mtpa using conventional truck-and-shovel equipment. This technical paper presents a comprehensive assessment of commingling as a transformative technology in mine planning, positioning waste rock and tailings management as a key factor in the decision-making process. The primary objective of this study is to demonstrate the benefits of this technology within the mine planning process, highlighting waste rock and tailings management as one of the main drivers in strategic decision-making. The methodology applied is a Comparative Case Analysis, which includes a review of the design and sequencing process for mining infrastructure such as waste rock storage facilities, along with a discussion of the key parameters considered. The results show that the implementation of this technology delivers significant economic benefits, including an approximate 30% reduction in capital costs for waste rock storage facilities, improved land-use efficiency, and optimized infrastructure development. Furthermore, the approach enhances alignment with the Global Industry Standard on Tailings Management (GISTM).

KEYWORDS

commingling, mine planning, waste rock management, tailings management, GISTM compliance

10	INTRODUCTION

The mining industry faces growing challenges related to the sustainable management of waste rock and tailings, especially after catastrophic events such as those that occurred in Brumadinho and Samarco (Green Policy Platform, 2024; ICMM, 2020). In this context, the Antamina mine, operating since 2001 as one of the largest polymetallic operations in the world, has developed a pioneering approach that integrates commingling as a central element of its strategic planning (Wheaton Precious Metals, 2015).

Commingling, defined as the homogeneous mixing of dewatered tailings with waste rock to form a geotechnically stable material, represents a natural evolution of traditional co-disposal practices (Burden & Wilson, 2023). However, what distinguishes the Antamina case is the integration of this technology from the initial phases of mine planning, making waste rock management the main driver of operational and strategic decisions.

11	OBJECTIVES

•	Demonstrate the benefits of integrating commingling into mine planning, positioning waste rock and tailings management as a strategic driver for decision-making in open-pit operations.

•	Optimize operational efficiency and environmental sustainability through the adoption of commingling in waste rock and tailings management.

•	Align waste rock and tailings management with international sustainability and safety standards, such as the GISTM, through the application of commingling.

•	Analyze, through comparative case studies, the impact of commingling on land-use optimization and mine life extension.

•	Reduce capital and operating costs associated with waste rock and tailings disposal by implementing commingling technologies.

•	Present the key findings and lessons learned from the integration of commingling into long-term mine planning.

12	TECHNICAL FOUNDATIONS OF COMMINGLING IN MINE PLANNING

12.1	Desing Principles and Material Characterization

Commingling at Antamina is based on engineering principles that seek to combine the superior structural properties of waste rock with the low permeability characteristics of tailings (Boshoff, 2023). This combination results in a material with shear strength similar to waste rock and permeability comparable to tailings, creating conditions that restrict oxygen entry and water filtration, significantly reducing the potential for acid drainage generation (Ulrich & Coffin, 2015; Burden & Wilson, 2023).

The characterization of commingled material is performed through analysis of five distinct structural configurations: rock-dominated uncompacted, fines-dominated uncompacted, rock-dominated compacted, fines-dominated compacted, and the "just-filled" configuration where fines exactly fill the voids between rock particles (Burden & Wilson, 2023). This latter configuration represents the optimal target of the commingling process at Antamina.

Figure 1 Blend configurations after Wickland et al. (2006). Source: Burden & Wilson (2023).

12.2	Integration into the Strategic Planning Process

The integration of commingling into Antamina's mine planning follows a structured methodological approach that places tailings management with waste rock as central elements of decision-making. This global strategy allows for optimizing the mixture of materials from the mine to the plant, considering the operational and environmental constraints associated with the tailings storage facility. By incorporating tailings behavior in the early stages of planning, greater efficiency in resource use and better risk management are achieved.

The global strategy is divided into three key axes:

•	Strategy 1: Ensure Future Expansions: The first strategy focuses on maximizing tailings storage capacity for current and future needs, leveraging Antamina's mineral resource and applying advanced technology to increase TSF (Tailings Storage Facility) capacity to ensure the mine's long-term operational continuity.

•	Strategy 2: Optimized footprint: The second strategy seeks to optimize the overall project footprint by balancing waste and tailings storage space. This approach aligns directly with commingling principles by maximizing efficient use of available land within the current operational footprint.

•	Strategy 3: Reduce upfront CAPEX: The third strategy focuses on reducing initial CAPEX to improve key financial indicators (IRR), which directly aligns with the demonstrated economic benefits of commingling, minimizing operational costs with a nearby TSF and optimizing CAPEX timing.

Figure 2 Strategic vision of mine planning approach with commingling

The strategies implemented in tailings dam and waste dump management seek to maximize the operational life of the mine and optimize asset value. These decisions translate into concrete actions that address sustainability, operational efficiency, and cost reduction, ensuring continuity and long-term success of the mining operation.

•	Long Life Asset: Antamina's vision as a long-life asset involves making decisions based on sustainable exploitation of its exceptional mineral resources. This includes adopting advanced technologies, such as commingling, to improve storage capacity and ensure operational continuity until 2036 and beyond, thus maximizing the value of the mineral resource.

•	Reduce Environmental Social Footprint: The strategic approach prioritizes reducing environmental and social impact through operation within the current footprint, increasing water availability through filtration, optimizing space use, and minimizing risks associated with tailings dams. Commingling contributes not only to technical and economic efficiency but also to sustainability objectives and corporate social responsibility.

•	Reduce Capital and Operational Cost: The cost reduction strategy focuses on optimizing capital investments (CAPEX) and their schedule, as well as minimizing operational costs through strategic facility location and leveraging synergies to avoid duplicate infrastructure. This enables more efficient and profitable resource management.

During the strategic planning phase, conceptual foundations for commingling implementation are established, including site characterization, regulatory framework development, and technical-economic feasibility analysis.

13	ANALYSIS OF THE MINING SYSTEM WITH COMMINGLING APPLICATION

13.1	Methodological Process Structure

The methodological process is organized in sequential stages that ensure effective integration of commingling into mine planning:

•	Comprehensive material evaluation: Detailed analysis of geotechnical and geochemical characteristics of available materials, identifying their suitability for mixtures and joint disposal.

•	Mixture design development: Design of optimized mixtures that maximize desired properties of commingled material, such as low permeability and high stability, minimizing acid water generation and improving mechanical behavior of the deposit.

•	Pilot test implementation: Execution of pilot and industrial scale tests to validate mixture performance and adjust design parameters according to results obtained under real operating conditions.

•	Monitoring and adjustment: Establishment of continuous monitoring system to evaluate consolidation, stability, and geochemical behavior of deposits, allowing operational and design adjustments as new information is collected.

14	IMPLEMENTATION AND EVALUATION METHODOLOGY FOR COMMINGLING IN MINE PLANNING

14.1	Comparative Case Analysis

The methodology applied at Antamina is based on Comparative Case Analysis, allowing for comprehensive review of the design and sequencing process of mining infrastructure, with emphasis on dams, waste rock dumps, and integration of commingling technologies. This methodological approach comprises:

•	Review of design and sequencing processes: Analysis of procedures used in waste dump planning and disposal, considering both geotechnical and geochemical criteria to ensure infrastructure stability and sustainability.

•	Identification of key variables: Selection of fundamental operational parameters, such as granulometry, permeability, geochemical behavior, and storage capacity, which directly affect the viability and performance of dumps and material mixtures.

•	Definition of case tree: Structure of a decision tree that contemplates long-term strategic scenarios, allowing comparison of alternatives under different combinations of variables and operational constraints.

•	Results evaluation: Application of sensitivity analyses and cost-benefit evaluations to determine the impact of each alternative on mine life extension, environmental risk reduction, and resource optimization.

14.2	Decision Tree for Strategic Planning

The decision tree developed for long-term strategic planning at Antamina follows sequential logic that allows evaluation of feasibility and impact of main operational alternatives:

•	Commingling feasibility: The first node of the tree determines whether integration of tailings and waste rock is technically and economically viable, considering material characteristics and regulatory constraints.

•	Entry into new area: If commingling is feasible, the next node evaluates the possibility of disposing material in a new area, analyzing topographic, environmental, and access aspects.

•	Need for new dam: Finally, it determines whether construction of a new tailings dam is essential or if the commingling solution allows dispensing with this infrastructure, optimizing investment and reducing environmental impact.

Each of these decisions conditions mine life extension, efficient space use, and environmental risk management, allowing selection of the alternative that maximizes economic, social, and environmental value of the project.

Figure 3  Decision Tree for Long-Term Strategic Scenario Planning

14.3	Evaluation of Operational Parameters and Modifications by Commingling

Key operational parameters—including waste rock-tailings ratio, mixing methods, transport systems, and processing criteria—are assessed for their impact on commingled system efficiency. The evaluation considers site-specific challenges such as extreme Andean weather, high-altitude logistics, and Peruvian regulations, all of which are especially critical at Antamina and require customized commingling solutions.

•	Modifications to tailings management by commingling:

The commingling process fundamentally transforms tailings management by utilizing voids within waste rock for storage, with available space typically equivalent to about 20% of the crushed rock volume transported by conveyors. To enable mixing, tailings must first be dewatered. The following image shows both the current process (without mechanization) and the transformation that occurs when commingling is incorporated.

Figure 4 Transformation of Tailings Management: Evolution from Conventional Process to Commingling|

•	 Operational parameters and assumptions

Commingling extends tailings facility life by optimizing spatial use of waste rock voids. Implementation requires mechanized systems (WCCS) for transporting crushed material mixed with dewatered tailings, while maintaining two key operational parameters: a) Unchanged transport capacity: 83.2 Mtpa in the conveyor system and b) Volumetric stability: No increase in total volume of waste rock dump.

This approach ensures efficient resource utilization and supports sustainable mine operations.

Table 1: Key operational parameters and assumptions

Component	Units	Without - Case Conventional	With - Case Commingling

Cycloned/Filtered Max. Capacity	Mtpa	-	32.0

Percentage of Voids in Waste Rock	% Volume 	 	20 %

Waste Rock / Tailings Ratio 	Mt Rock/

Mt Tails	-	83 / 16

Conventional Tailings Density	t/m3 	1.81	1.81

Fines Tailings Density	 t/m3	NA	1.60

Tailings Volume, TDR 4195 (to Dic-22)	Mm3	376	376

Tailings Capacity, TDR 4195 (to Dic-22)	Mt	682	652

Mill Feed Rate 	Ktpd	145	145

The following image shows how incorporation of mechanized systems and commingling add capacity, and therefore extend tailings facility life, to increase the life of mine and therefore the overall asset value.

Figure 5 Commingling Extension of Tailings Facility Life through Commingling

The curve below shows the relationship between dry density and optimal mixing proportion, determinant for maximizing commingling efficiency.

Figure 6: Dry density curve vs. optimal mixing proportion for commingling

•	Analysis of Mineable Resources, Production and Value

A three-dimensional diagram is used to strategically analyze the interactions between mineable resource volume, production rate, and generated value—whether economic, environmental, or operational. This tool helps identify how increasing resource volume and production can maximize value, but also highlights key constraints such as tailings dam capacity and property limits. The approach supports comprehensive decision-making to optimize asset value and ensure project sustainability across various operational scenarios.

Figure 7 Value Optimization: Interaction between Resource Volume, Production Rate and Key Restrictions

15	ECONOMIC AND OPERATIONAL ADVANTAGES

15.1	Perspective of Conventional Value Drivers

The following figure illustrates Antamina's main conventional value drivers from a financial perspective. In the current scenario, the amount of mineral resource that can be utilized directly depends on the capital that the company decides to invest (Capex), both in new investments (Growth Capex) and in maintaining existing operations (Sustaining Capex). This generates a necessary balance to determine the optimal scenario that maximizes the value of the existing asset.

Figure 8: Conventional Antamina Value Drivers: Financial Perspective (Without Commingling)

15.2	Impact of Commingling on Financial Value Drivers

From a financial perspective, the adoption of commingling among Antamina’s value drivers removes constraints on mineral resources by increasing tailings storage capacity, enabling the utilization of new dumps, and allowing for an expansion in the volume of usable resources. As a result, the mine’s operational life is extended and asset value is enhanced. Regarding dump utilization, the implementation of commingling has the potential to reduce the requirement for dumps located in karstic zones, which demand significant capital expenditure (Capex). This can lead to a reduction in initial investment by up to 30% in scenarios comparing commingling versus non-commingling approaches. This saving is achieved through the integration of previously separate infrastructures into a single system, which optimizes the use of available area in the East Extension dumps and prioritizes the use of mechanized systems over conventional trucking methods.

Figure 9: Antamina Value Drivers: Financial Perspective with Commingling

The optimization of land use represents another important economic benefit, especially relevant in mountainous locations like Antamina where available land for mining facilities is limited. Commingling allows for the creation of deposits with a smaller footprint compared to separate facilities, freeing up land for other productive uses or reducing the need for additional land acquisition.

16	ALIGNMENT WITH GISTM STANDARDS AND SUSTAINABILITY

The integration of commingling into mine planning strengthens compliance with the Global Industry Standard on Tailings Management (GISTM), by facilitating safer and more sustainable management of waste materials. The GISTM requires the planning, construction, operation, and closure of tailings facilities with a focus on risk reduction and continuous monitoring throughout the lifecycle. Commingling contributes to this objective by improving the geotechnical and geochemical stability of deposits, reducing the risk of failure and acid drainage generation, which in turn reduces environmental impact and long-term treatment costs. Additionally, the consolidation of facilities through commingling reduces the environmental footprint and facilitates supervision and control, aligning with the principles of sustainability and social responsibility in the mining industry.

17	LESSONS LEARNED AND CHALLENGES

Among the main lessons learned and challenges of implementing commingling in mine planning at Antamina, the following stand out:

•	Large-scale operations and mechanization: Commingling is primarily viable in large-volume mechanized operations, where it is possible to achieve a homogeneous and controlled mixture of materials. However, the magnitude of waste rock and tailings tonnages at Antamina implies a new challenge, never seen before.

•	Flexible planning and pilot tests: It is essential to include areas for pilot tests in short and medium-term plans, allowing adjustment of mixing parameters according to material variability.

•	Regulatory communication: Technology acceptance requires identifying precedents and working closely with authorities to develop appropriate regulatory frameworks.

•	Organizational change management: The transition to commingling involves challenges in personnel training and adaptation of operational processes, especially in mature operations.

18	CONCLUSIONS

•	The comparative discussion and case analysis of commingling implementation at Antamina have enabled the identification and selection of strategic alternatives that maximize the economic, environmental, and social value of the project.

•	The use of decision trees and scenario evaluation has facilitated informed decision-making, taking into account technical and economic feasibility as well as regulatory and environmental constraints.

•	The integration of commingling into strategic mine planning has proven to be a key tool for optimizing waste rock and tailings management, achieving a significant reduction in capital costs (up to 30%), greater efficiency in land use, and an extension of tailings storage facility (TSF) life. This approach has also aligned operations with international sustainability standards (GISTM) and strengthened risk management, contributing to operational continuity and long-term reduction of environmental impacts.

•	The Antamina experience demonstrates that structured case discussions and the application of comparative methodologies are fundamental for strategic planning, as they allow anticipation of challenges, validation of solutions through pilot testing, and adjustment of operational parameters based on real outcomes. Thus, commingling is consolidated as a transformative practice in modern mining, fostering more robust and sustainable decision-making for the development of long-life mining assets.

ACKNOWLEDGEMENTS

The authors express their sincere gratitude to Antamina for their invaluable support. Special thanks are extended to the Tailings Strategy Management team and the Long-Term Planning team for their collaboration and insights.

REFERENCES

Boshoff, J. et al. (2023). A case study on the commingling of tailings and waste rock at a Brownfields open cast mine in Ghana. University of Alberta.

Burden, R., & Wilson, G. W. (2023). Commingling of waste rock and tailings to improve “dry stack” performance: Design and evaluation of mixtures. Minerals, 13(2), 295.

Deza, N., & Montes, K. (2022). Mecanización del desmonte en Compañía Minera Antamina: Propuesta para capturar el valor tangible e intangible de la mecanización. Compañía Minera Antamina.

Green Policy Platform. (2024). Knowledge Gaps Report: Environmental Aspects of Tailings Management (January 2024).

ICMM. (2020). New global industry standard on tailings management. International Council on Mining and Metals.

Ulrich, B. & Coffin, J. (2015). TMW 2015 – Combined Tailings and Mine Waste.

Minemax. (2015). Why mine planning is all about collaboration. Minemax News.

Wheaton Precious Metals. (2015). Antamina Project overview.

INTEGRATING COMMINGLING INTO THE STRATEGIC WASTE ROCK PLANNING AT ANTAMINA MINE

Christa Quiroz1, David Machin1, Olimpia Cabrera1, Fernando Angeles1

1 Antamina Mine, Lima, Peru

(*Presenting author: cquirozc@antamina.com)

ABSTRACT

Antamina is a large open-pit polymetallic mine in Peru, operating with a processing capacity of 145 ktpd. In operation since 2001, the current mine life extends to 2036. Mining is conducted at a rate of 290 Mtpa using conventional truck-and-shovel equipment. This technical paper presents a comprehensive assessment of commingling as a transformative technology in mine planning, positioning waste rock and tailings management as a key factor in the decision-making process. The primary objective of this study is to demonstrate the benefits of this technology within the mine planning process, highlighting waste rock and tailings management as one of the main drivers in strategic decision-making. The methodology applied is a Comparative Case Analysis, which includes a review of the design and sequencing process for mining infrastructure such as waste rock storage facilities, along with a discussion of the key parameters considered. The results show that the implementation of this technology delivers significant economic benefits, including an approximate 30% reduction in capital costs for waste rock storage facilities, improved land-use efficiency, and optimized infrastructure development. Furthermore, the approach enhances alignment with the Global Industry Standard on Tailings Management (GISTM).

KEYWORDS

commingling, mine planning, waste rock management, tailings management, GISTM compliance

1	INTRODUCTION

The mining industry faces growing challenges related to the sustainable management of waste rock and tailings, especially after catastrophic events such as those that occurred in Brumadinho and Samarco (Green Policy Platform, 2024; ICMM, 2020). In this context, the Antamina mine, operating since 2001 as one of the largest polymetallic operations in the world, has developed a pioneering approach that integrates commingling as a central element of its strategic planning (Wheaton Precious Metals, 2015).

Commingling, defined as the homogeneous mixing of dewatered tailings with waste rock to form a geotechnically stable material, represents a natural evolution of traditional co-disposal practices (Burden & Wilson, 2023). However, what distinguishes the Antamina case is the integration of this technology from the initial phases of mine planning, making waste rock management the main driver of operational and strategic decisions.

2	OBJECTIVES

•	Demonstrate the benefits of integrating commingling into mine planning, positioning waste rock and tailings management as a strategic driver for decision-making in open-pit operations.

•	Optimize operational efficiency and environmental sustainability through the adoption of commingling in waste rock and tailings management.

•	Align waste rock and tailings management with international sustainability and safety standards, such as the GISTM, through the application of commingling.

•	Analyze, through comparative case studies, the impact of commingling on land-use optimization and mine life extension.

•	Reduce capital and operating costs associated with waste rock and tailings disposal by implementing commingling technologies.

•	Present the key findings and lessons learned from the integration of commingling into long-term mine planning.

3	TECHNICAL FOUNDATIONS OF COMMINGLING IN MINE PLANNING

3.1	Desing Principles and Material Characterization

Commingling at Antamina is based on engineering principles that seek to combine the superior structural properties of waste rock with the low permeability characteristics of tailings (Boshoff, 2023). This combination results in a material with shear strength similar to waste rock and permeability comparable to tailings, creating conditions that restrict oxygen entry and water filtration, significantly reducing the potential for acid drainage generation (Ulrich & Coffin, 2015; Burden & Wilson, 2023).

The characterization of commingled material is performed through analysis of five distinct structural configurations: rock-dominated uncompacted, fines-dominated uncompacted, rock-dominated compacted, fines-dominated compacted, and the "just-filled" configuration where fines exactly fill the voids between rock particles (Burden & Wilson, 2023). This latter configuration represents the optimal target of the commingling process at Antamina.

Figure 1 Blend configurations after Wickland et al. (2006). Source: Burden & Wilson (2023).

3.2	Integration into the Strategic Planning Process

The integration of commingling into Antamina's mine planning follows a structured methodological approach that places tailings management with waste rock as central elements of decision-making. This global strategy allows for optimizing the mixture of materials from the mine to the plant, considering the operational and environmental constraints associated with the tailings storage facility. By incorporating tailings behavior in the early stages of planning, greater efficiency in resource use and better risk management are achieved.

The global strategy is divided into three key axes:

•	Strategy 1: Ensure Future Expansions: The first strategy focuses on maximizing tailings storage capacity for current and future needs, leveraging Antamina's mineral resource and applying advanced technology to increase TSF (Tailings Storage Facility) capacity to ensure the mine's long-term operational continuity.

•	Strategy 2: Optimized footprint: The second strategy seeks to optimize the overall project footprint by balancing waste and tailings storage space. This approach aligns directly with commingling principles by maximizing efficient use of available land within the current operational footprint.

•	Strategy 3: Reduce upfront CAPEX: The third strategy focuses on reducing initial CAPEX to improve key financial indicators (IRR), which directly aligns with the demonstrated economic benefits of commingling, minimizing operational costs with a nearby TSF and optimizing CAPEX timing.

Figure 2 Strategic vision of mine planning approach with commingling

The strategies implemented in tailings dam and waste dump management seek to maximize the operational life of the mine and optimize asset value. These decisions translate into concrete actions that address sustainability, operational efficiency, and cost reduction, ensuring continuity and long-term success of the mining operation.

•	Long Life Asset: Antamina's vision as a long-life asset involves making decisions based on sustainable exploitation of its exceptional mineral resources. This includes adopting advanced technologies, such as commingling, to improve storage capacity and ensure operational continuity until 2036 and beyond, thus maximizing the value of the mineral resource.

•	Reduce Environmental Social Footprint: The strategic approach prioritizes reducing environmental and social impact through operation within the current footprint, increasing water availability through filtration, optimizing space use, and minimizing risks associated with tailings dams. Commingling contributes not only to technical and economic efficiency but also to sustainability objectives and corporate social responsibility.

•	Reduce Capital and Operational Cost: The cost reduction strategy focuses on optimizing capital investments (CAPEX) and their schedule, as well as minimizing operational costs through strategic facility location and leveraging synergies to avoid duplicate infrastructure. This enables more efficient and profitable resource management.

During the strategic planning phase, conceptual foundations for commingling implementation are established, including site characterization, regulatory framework development, and technical-economic feasibility analysis.

4	ANALYSIS OF THE MINING SYSTEM WITH COMMINGLING APPLICATION

4.1	Methodological Process Structure

The methodological process is organized in sequential stages that ensure effective integration of commingling into mine planning:

•	Comprehensive material evaluation: Detailed analysis of geotechnical and geochemical characteristics of available materials, identifying their suitability for mixtures and joint disposal.

•	Mixture design development: Design of optimized mixtures that maximize desired properties of commingled material, such as low permeability and high stability, minimizing acid water generation and improving mechanical behavior of the deposit.

•	Pilot test implementation: Execution of pilot and industrial scale tests to validate mixture performance and adjust design parameters according to results obtained under real operating conditions.

•	Monitoring and adjustment: Establishment of continuous monitoring system to evaluate consolidation, stability, and geochemical behavior of deposits, allowing operational and design adjustments as new information is collected.

5	IMPLEMENTATION AND EVALUATION METHODOLOGY FOR COMMINGLING IN MINE PLANNING

5.1	Comparative Case Analysis

The methodology applied at Antamina is based on Comparative Case Analysis, allowing for comprehensive review of the design and sequencing process of mining infrastructure, with emphasis on dams, waste rock dumps, and integration of commingling technologies. This methodological approach comprises:

•	Review of design and sequencing processes: Analysis of procedures used in waste dump planning and disposal, considering both geotechnical and geochemical criteria to ensure infrastructure stability and sustainability.

•	Identification of key variables: Selection of fundamental operational parameters, such as granulometry, permeability, geochemical behavior, and storage capacity, which directly affect the viability and performance of dumps and material mixtures.

•	Definition of case tree: Structure of a decision tree that contemplates long-term strategic scenarios, allowing comparison of alternatives under different combinations of variables and operational constraints.

•	Results evaluation: Application of sensitivity analyses and cost-benefit evaluations to determine the impact of each alternative on mine life extension, environmental risk reduction, and resource optimization.

5.2	Decision Tree for Strategic Planning

The decision tree developed for long-term strategic planning at Antamina follows sequential logic that allows evaluation of feasibility and impact of main operational alternatives:

•	Commingling feasibility: The first node of the tree determines whether integration of tailings and waste rock is technically and economically viable, considering material characteristics and regulatory constraints.

•	Entry into new area: If commingling is feasible, the next node evaluates the possibility of disposing material in a new area, analyzing topographic, environmental, and access aspects.

•	Need for new dam: Finally, it determines whether construction of a new tailings dam is essential or if the commingling solution allows dispensing with this infrastructure, optimizing investment and reducing environmental impact.

Each of these decisions conditions mine life extension, efficient space use, and environmental risk management, allowing selection of the alternative that maximizes economic, social, and environmental value of the project.

Figure 3  Decision Tree for Long-Term Strategic Scenario Planning

5.3	Evaluation of Operational Parameters and Modifications by Commingling

Key operational parameters—including waste rock-tailings ratio, mixing methods, transport systems, and processing criteria—are assessed for their impact on commingled system efficiency. The evaluation considers site-specific challenges such as extreme Andean weather, high-altitude logistics, and Peruvian regulations, all of which are especially critical at Antamina and require customized commingling solutions.

•	Modifications to tailings management by commingling:

The commingling process fundamentally transforms tailings management by utilizing voids within waste rock for storage, with available space typically equivalent to about 20% of the crushed rock volume transported by conveyors. To enable mixing, tailings must first be dewatered. The following image shows both the current process (without mechanization) and the transformation that occurs when commingling is incorporated.

Figure 4 Transformation of Tailings Management: Evolution from Conventional Process to Commingling|

•	 Operational parameters and assumptions

Commingling extends tailings facility life by optimizing spatial use of waste rock voids. Implementation requires mechanized systems (WCCS) for transporting crushed material mixed with dewatered tailings, while maintaining two key operational parameters: a) Unchanged transport capacity: 83.2 Mtpa in the conveyor system and b) Volumetric stability: No increase in total volume of waste rock dump.

This approach ensures efficient resource utilization and supports sustainable mine operations.

Table 1: Key operational parameters and assumptions

Component	Units	Without - Case Conventional	With - Case Commingling

Cycloned/Filtered Max. Capacity	Mtpa	-	32.0

Percentage of Voids in Waste Rock	% Volume 	 	20 %

Waste Rock / Tailings Ratio 	Mt Rock/

Mt Tails	-	83 / 16

Conventional Tailings Density	t/m3 	1.81	1.81

Fines Tailings Density	 t/m3	NA	1.60

Tailings Volume, TDR 4195 (to Dic-22)	Mm3	376	376

Tailings Capacity, TDR 4195 (to Dic-22)	Mt	682	652

Mill Feed Rate 	Ktpd	145	145

The following image shows how incorporation of mechanized systems and commingling add capacity, and therefore extend tailings facility life, to increase the life of mine and therefore the overall asset value.

Figure 5 Commingling Extension of Tailings Facility Life through Commingling

The curve below shows the relationship between dry density and optimal mixing proportion, determinant for maximizing commingling efficiency.

Figure 6: Dry density curve vs. optimal mixing proportion for commingling

•	Analysis of Mineable Resources, Production and Value

A three-dimensional diagram is used to strategically analyze the interactions between mineable resource volume, production rate, and generated value—whether economic, environmental, or operational. This tool helps identify how increasing resource volume and production can maximize value, but also highlights key constraints such as tailings dam capacity and property limits. The approach supports comprehensive decision-making to optimize asset value and ensure project sustainability across various operational scenarios.

Figure 7 Value Optimization: Interaction between Resource Volume, Production Rate and Key Restrictions

6	ECONOMIC AND OPERATIONAL ADVANTAGES

6.1	Perspective of Conventional Value Drivers

The following figure illustrates Antamina's main conventional value drivers from a financial perspective. In the current scenario, the amount of mineral resource that can be utilized directly depends on the capital that the company decides to invest (Capex), both in new investments (Growth Capex) and in maintaining existing operations (Sustaining Capex). This generates a necessary balance to determine the optimal scenario that maximizes the value of the existing asset.

Figure 8: Conventional Antamina Value Drivers: Financial Perspective (Without Commingling)

6.2	Impact of Commingling on Financial Value Drivers

From a financial perspective, the adoption of commingling among Antamina’s value drivers removes constraints on mineral resources by increasing tailings storage capacity, enabling the utilization of new dumps, and allowing for an expansion in the volume of usable resources. As a result, the mine’s operational life is extended and asset value is enhanced. Regarding dump utilization, the implementation of commingling has the potential to reduce the requirement for dumps located in karstic zones, which demand significant capital expenditure (Capex). This can lead to a reduction in initial investment by up to 30% in scenarios comparing commingling versus non-commingling approaches. This saving is achieved through the integration of previously separate infrastructures into a single system, which optimizes the use of available area in the East Extension dumps and prioritizes the use of mechanized systems over conventional trucking methods.

Figure 9: Antamina Value Drivers: Financial Perspective with Commingling

The optimization of land use represents another important economic benefit, especially relevant in mountainous locations like Antamina where available land for mining facilities is limited. Commingling allows for the creation of deposits with a smaller footprint compared to separate facilities, freeing up land for other productive uses or reducing the need for additional land acquisition.

7	ALIGNMENT WITH GISTM STANDARDS AND SUSTAINABILITY

The integration of commingling into mine planning strengthens compliance with the Global Industry Standard on Tailings Management (GISTM), by facilitating safer and more sustainable management of waste materials. The GISTM requires the planning, construction, operation, and closure of tailings facilities with a focus on risk reduction and continuous monitoring throughout the lifecycle. Commingling contributes to this objective by improving the geotechnical and geochemical stability of deposits, reducing the risk of failure and acid drainage generation, which in turn reduces environmental impact and long-term treatment costs. Additionally, the consolidation of facilities through commingling reduces the environmental footprint and facilitates supervision and control, aligning with the principles of sustainability and social responsibility in the mining industry.

8	LESSONS LEARNED AND CHALLENGES

Among the main lessons learned and challenges of implementing commingling in mine planning at Antamina, the following stand out:

•	Large-scale operations and mechanization: Commingling is primarily viable in large-volume mechanized operations, where it is possible to achieve a homogeneous and controlled mixture of materials. However, the magnitude of waste rock and tailings tonnages at Antamina implies a new challenge, never seen before.

•	Flexible planning and pilot tests: It is essential to include areas for pilot tests in short and medium-term plans, allowing adjustment of mixing parameters according to material variability.

•	Regulatory communication: Technology acceptance requires identifying precedents and working closely with authorities to develop appropriate regulatory frameworks.

•	Organizational change management: The transition to commingling involves challenges in personnel training and adaptation of operational processes, especially in mature operations.

9	CONCLUSIONS

•	The comparative discussion and case analysis of commingling implementation at Antamina have enabled the identification and selection of strategic alternatives that maximize the economic, environmental, and social value of the project.

•	The use of decision trees and scenario evaluation has facilitated informed decision-making, taking into account technical and economic feasibility as well as regulatory and environmental constraints.

•	The integration of commingling into strategic mine planning has proven to be a key tool for optimizing waste rock and tailings management, achieving a significant reduction in capital costs (up to 30%), greater efficiency in land use, and an extension of tailings storage facility (TSF) life. This approach has also aligned operations with international sustainability standards (GISTM) and strengthened risk management, contributing to operational continuity and long-term reduction of environmental impacts.

•	The Antamina experience demonstrates that structured case discussions and the application of comparative methodologies are fundamental for strategic planning, as they allow anticipation of challenges, validation of solutions through pilot testing, and adjustment of operational parameters based on real outcomes. Thus, commingling is consolidated as a transformative practice in modern mining, fostering more robust and sustainable decision-making for the development of long-life mining assets.

ACKNOWLEDGEMENTS

The authors express their sincere gratitude to Antamina for their invaluable support. Special thanks are extended to the Tailings Strategy Management team and the Long-Term Planning team for their collaboration and insights.

REFERENCES

Boshoff, J. et al. (2023). A case study on the commingling of tailings and waste rock at a Brownfields open cast mine in Ghana. University of Alberta.

Burden, R., & Wilson, G. W. (2023). Commingling of waste rock and tailings to improve “dry stack” performance: Design and evaluation of mixtures. Minerals, 13(2), 295.

Deza, N., & Montes, K. (2022). Mecanización del desmonte en Compañía Minera Antamina: Propuesta para capturar el valor tangible e intangible de la mecanización. Compañía Minera Antamina.

Green Policy Platform. (2024). Knowledge Gaps Report: Environmental Aspects of Tailings Management (January 2024).

ICMM. (2020). New global industry standard on tailings management. International Council on Mining and Metals.

Ulrich, B. & Coffin, J. (2015). TMW 2015 – Combined Tailings and Mine Waste.

Minemax. (2015). Why mine planning is all about collaboration. Minemax News.

Wheaton Precious Metals. (2015). Antamina Project overview.

INTEGRATING COMMINGLING INTO THE STRATEGIC WASTE ROCK PLANNING AT ANTAMINA MINE

Christa Quiroz1, David Machin1, Olimpia Cabrera1, Fernando Angeles1

1 Antamina Mine, Lima, Peru

(*Presenting author: cquirozc@antamina.com)

ABSTRACT

Antamina is a large open-pit polymetallic mine in Peru, operating with a processing capacity of 145 ktpd. In operation since 2001, the current mine life extends to 2036. Mining is conducted at a rate of 290 Mtpa using conventional truck-and-shovel equipment. This technical paper presents a comprehensive assessment of commingling as a transformative technology in mine planning, positioning waste rock and tailings management as a key factor in the decision-making process. The primary objective of this study is to demonstrate the benefits of this technology within the mine planning process, highlighting waste rock and tailings management as one of the main drivers in strategic decision-making. The methodology applied is a Comparative Case Analysis, which includes a review of the design and sequencing process for mining infrastructure such as waste rock storage facilities, along with a discussion of the key parameters considered. The results show that the implementation of this technology delivers significant economic benefits, including an approximate 30% reduction in capital costs for waste rock storage facilities, improved land-use efficiency, and optimized infrastructure development. Furthermore, the approach enhances alignment with the Global Industry Standard on Tailings Management (GISTM).

KEYWORDS

commingling, mine planning, waste rock management, tailings management, GISTM compliance

10	INTRODUCTION

The mining industry faces growing challenges related to the sustainable management of waste rock and tailings, especially after catastrophic events such as those that occurred in Brumadinho and Samarco (Green Policy Platform, 2024; ICMM, 2020). In this context, the Antamina mine, operating since 2001 as one of the largest polymetallic operations in the world, has developed a pioneering approach that integrates commingling as a central element of its strategic planning (Wheaton Precious Metals, 2015).

Commingling, defined as the homogeneous mixing of dewatered tailings with waste rock to form a geotechnically stable material, represents a natural evolution of traditional co-disposal practices (Burden & Wilson, 2023). However, what distinguishes the Antamina case is the integration of this technology from the initial phases of mine planning, making waste rock management the main driver of operational and strategic decisions.

11	OBJECTIVES

•	Demonstrate the benefits of integrating commingling into mine planning, positioning waste rock and tailings management as a strategic driver for decision-making in open-pit operations.

•	Optimize operational efficiency and environmental sustainability through the adoption of commingling in waste rock and tailings management.

•	Align waste rock and tailings management with international sustainability and safety standards, such as the GISTM, through the application of commingling.

•	Analyze, through comparative case studies, the impact of commingling on land-use optimization and mine life extension.

•	Reduce capital and operating costs associated with waste rock and tailings disposal by implementing commingling technologies.

•	Present the key findings and lessons learned from the integration of commingling into long-term mine planning.

12	TECHNICAL FOUNDATIONS OF COMMINGLING IN MINE PLANNING

12.1	Desing Principles and Material Characterization

Commingling at Antamina is based on engineering principles that seek to combine the superior structural properties of waste rock with the low permeability characteristics of tailings (Boshoff, 2023). This combination results in a material with shear strength similar to waste rock and permeability comparable to tailings, creating conditions that restrict oxygen entry and water filtration, significantly reducing the potential for acid drainage generation (Ulrich & Coffin, 2015; Burden & Wilson, 2023).

The characterization of commingled material is performed through analysis of five distinct structural configurations: rock-dominated uncompacted, fines-dominated uncompacted, rock-dominated compacted, fines-dominated compacted, and the "just-filled" configuration where fines exactly fill the voids between rock particles (Burden & Wilson, 2023). This latter configuration represents the optimal target of the commingling process at Antamina.

Figure 1 Blend configurations after Wickland et al. (2006). Source: Burden & Wilson (2023).

12.2	Integration into the Strategic Planning Process

The integration of commingling into Antamina's mine planning follows a structured methodological approach that places tailings management with waste rock as central elements of decision-making. This global strategy allows for optimizing the mixture of materials from the mine to the plant, considering the operational and environmental constraints associated with the tailings storage facility. By incorporating tailings behavior in the early stages of planning, greater efficiency in resource use and better risk management are achieved.

The global strategy is divided into three key axes:

•	Strategy 1: Ensure Future Expansions: The first strategy focuses on maximizing tailings storage capacity for current and future needs, leveraging Antamina's mineral resource and applying advanced technology to increase TSF (Tailings Storage Facility) capacity to ensure the mine's long-term operational continuity.

•	Strategy 2: Optimized footprint: The second strategy seeks to optimize the overall project footprint by balancing waste and tailings storage space. This approach aligns directly with commingling principles by maximizing efficient use of available land within the current operational footprint.

•	Strategy 3: Reduce upfront CAPEX: The third strategy focuses on reducing initial CAPEX to improve key financial indicators (IRR), which directly aligns with the demonstrated economic benefits of commingling, minimizing operational costs with a nearby TSF and optimizing CAPEX timing.

Figure 2 Strategic vision of mine planning approach with commingling

The strategies implemented in tailings dam and waste dump management seek to maximize the operational life of the mine and optimize asset value. These decisions translate into concrete actions that address sustainability, operational efficiency, and cost reduction, ensuring continuity and long-term success of the mining operation.

•	Long Life Asset: Antamina's vision as a long-life asset involves making decisions based on sustainable exploitation of its exceptional mineral resources. This includes adopting advanced technologies, such as commingling, to improve storage capacity and ensure operational continuity until 2036 and beyond, thus maximizing the value of the mineral resource.

•	Reduce Environmental Social Footprint: The strategic approach prioritizes reducing environmental and social impact through operation within the current footprint, increasing water availability through filtration, optimizing space use, and minimizing risks associated with tailings dams. Commingling contributes not only to technical and economic efficiency but also to sustainability objectives and corporate social responsibility.

•	Reduce Capital and Operational Cost: The cost reduction strategy focuses on optimizing capital investments (CAPEX) and their schedule, as well as minimizing operational costs through strategic facility location and leveraging synergies to avoid duplicate infrastructure. This enables more efficient and profitable resource management.

During the strategic planning phase, conceptual foundations for commingling implementation are established, including site characterization, regulatory framework development, and technical-economic feasibility analysis.

13	ANALYSIS OF THE MINING SYSTEM WITH COMMINGLING APPLICATION

13.1	Methodological Process Structure

The methodological process is organized in sequential stages that ensure effective integration of commingling into mine planning:

•	Comprehensive material evaluation: Detailed analysis of geotechnical and geochemical characteristics of available materials, identifying their suitability for mixtures and joint disposal.

•	Mixture design development: Design of optimized mixtures that maximize desired properties of commingled material, such as low permeability and high stability, minimizing acid water generation and improving mechanical behavior of the deposit.

•	Pilot test implementation: Execution of pilot and industrial scale tests to validate mixture performance and adjust design parameters according to results obtained under real operating conditions.

•	Monitoring and adjustment: Establishment of continuous monitoring system to evaluate consolidation, stability, and geochemical behavior of deposits, allowing operational and design adjustments as new information is collected.

14	IMPLEMENTATION AND EVALUATION METHODOLOGY FOR COMMINGLING IN MINE PLANNING

14.1	Comparative Case Analysis

The methodology applied at Antamina is based on Comparative Case Analysis, allowing for comprehensive review of the design and sequencing process of mining infrastructure, with emphasis on dams, waste rock dumps, and integration of commingling technologies. This methodological approach comprises:

•	Review of design and sequencing processes: Analysis of procedures used in waste dump planning and disposal, considering both geotechnical and geochemical criteria to ensure infrastructure stability and sustainability.

•	Identification of key variables: Selection of fundamental operational parameters, such as granulometry, permeability, geochemical behavior, and storage capacity, which directly affect the viability and performance of dumps and material mixtures.

•	Definition of case tree: Structure of a decision tree that contemplates long-term strategic scenarios, allowing comparison of alternatives under different combinations of variables and operational constraints.

•	Results evaluation: Application of sensitivity analyses and cost-benefit evaluations to determine the impact of each alternative on mine life extension, environmental risk reduction, and resource optimization.

14.2	Decision Tree for Strategic Planning

The decision tree developed for long-term strategic planning at Antamina follows sequential logic that allows evaluation of feasibility and impact of main operational alternatives:

•	Commingling feasibility: The first node of the tree determines whether integration of tailings and waste rock is technically and economically viable, considering material characteristics and regulatory constraints.

•	Entry into new area: If commingling is feasible, the next node evaluates the possibility of disposing material in a new area, analyzing topographic, environmental, and access aspects.

•	Need for new dam: Finally, it determines whether construction of a new tailings dam is essential or if the commingling solution allows dispensing with this infrastructure, optimizing investment and reducing environmental impact.

Each of these decisions conditions mine life extension, efficient space use, and environmental risk management, allowing selection of the alternative that maximizes economic, social, and environmental value of the project.

Figure 3  Decision Tree for Long-Term Strategic Scenario Planning

14.3	Evaluation of Operational Parameters and Modifications by Commingling

Key operational parameters—including waste rock-tailings ratio, mixing methods, transport systems, and processing criteria—are assessed for their impact on commingled system efficiency. The evaluation considers site-specific challenges such as extreme Andean weather, high-altitude logistics, and Peruvian regulations, all of which are especially critical at Antamina and require customized commingling solutions.

•	Modifications to tailings management by commingling:

The commingling process fundamentally transforms tailings management by utilizing voids within waste rock for storage, with available space typically equivalent to about 20% of the crushed rock volume transported by conveyors. To enable mixing, tailings must first be dewatered. The following image shows both the current process (without mechanization) and the transformation that occurs when commingling is incorporated.

Figure 4 Transformation of Tailings Management: Evolution from Conventional Process to Commingling|

•	 Operational parameters and assumptions

Commingling extends tailings facility life by optimizing spatial use of waste rock voids. Implementation requires mechanized systems (WCCS) for transporting crushed material mixed with dewatered tailings, while maintaining two key operational parameters: a) Unchanged transport capacity: 83.2 Mtpa in the conveyor system and b) Volumetric stability: No increase in total volume of waste rock dump.

This approach ensures efficient resource utilization and supports sustainable mine operations.

Table 1: Key operational parameters and assumptions

Component	Units	Without - Case Conventional	With - Case Commingling

Cycloned/Filtered Max. Capacity	Mtpa	-	32.0

Percentage of Voids in Waste Rock	% Volume 	 	20 %

Waste Rock / Tailings Ratio 	Mt Rock/

Mt Tails	-	83 / 16

Conventional Tailings Density	t/m3 	1.81	1.81

Fines Tailings Density	 t/m3	NA	1.60

Tailings Volume, TDR 4195 (to Dic-22)	Mm3	376	376

Tailings Capacity, TDR 4195 (to Dic-22)	Mt	682	652

Mill Feed Rate 	Ktpd	145	145

The following image shows how incorporation of mechanized systems and commingling add capacity, and therefore extend tailings facility life, to increase the life of mine and therefore the overall asset value.

Figure 5 Commingling Extension of Tailings Facility Life through Commingling

The curve below shows the relationship between dry density and optimal mixing proportion, determinant for maximizing commingling efficiency.

Figure 6: Dry density curve vs. optimal mixing proportion for commingling

•	Analysis of Mineable Resources, Production and Value

A three-dimensional diagram is used to strategically analyze the interactions between mineable resource volume, production rate, and generated value—whether economic, environmental, or operational. This tool helps identify how increasing resource volume and production can maximize value, but also highlights key constraints such as tailings dam capacity and property limits. The approach supports comprehensive decision-making to optimize asset value and ensure project sustainability across various operational scenarios.

Figure 7 Value Optimization: Interaction between Resource Volume, Production Rate and Key Restrictions

15	ECONOMIC AND OPERATIONAL ADVANTAGES

15.1	Perspective of Conventional Value Drivers

The following figure illustrates Antamina's main conventional value drivers from a financial perspective. In the current scenario, the amount of mineral resource that can be utilized directly depends on the capital that the company decides to invest (Capex), both in new investments (Growth Capex) and in maintaining existing operations (Sustaining Capex). This generates a necessary balance to determine the optimal scenario that maximizes the value of the existing asset.

Figure 8: Conventional Antamina Value Drivers: Financial Perspective (Without Commingling)

15.2	Impact of Commingling on Financial Value Drivers

From a financial perspective, the adoption of commingling among Antamina’s value drivers removes constraints on mineral resources by increasing tailings storage capacity, enabling the utilization of new dumps, and allowing for an expansion in the volume of usable resources. As a result, the mine’s operational life is extended and asset value is enhanced. Regarding dump utilization, the implementation of commingling has the potential to reduce the requirement for dumps located in karstic zones, which demand significant capital expenditure (Capex). This can lead to a reduction in initial investment by up to 30% in scenarios comparing commingling versus non-commingling approaches. This saving is achieved through the integration of previously separate infrastructures into a single system, which optimizes the use of available area in the East Extension dumps and prioritizes the use of mechanized systems over conventional trucking methods.

Figure 9: Antamina Value Drivers: Financial Perspective with Commingling

The optimization of land use represents another important economic benefit, especially relevant in mountainous locations like Antamina where available land for mining facilities is limited. Commingling allows for the creation of deposits with a smaller footprint compared to separate facilities, freeing up land for other productive uses or reducing the need for additional land acquisition.

16	ALIGNMENT WITH GISTM STANDARDS AND SUSTAINABILITY

The integration of commingling into mine planning strengthens compliance with the Global Industry Standard on Tailings Management (GISTM), by facilitating safer and more sustainable management of waste materials. The GISTM requires the planning, construction, operation, and closure of tailings facilities with a focus on risk reduction and continuous monitoring throughout the lifecycle. Commingling contributes to this objective by improving the geotechnical and geochemical stability of deposits, reducing the risk of failure and acid drainage generation, which in turn reduces environmental impact and long-term treatment costs. Additionally, the consolidation of facilities through commingling reduces the environmental footprint and facilitates supervision and control, aligning with the principles of sustainability and social responsibility in the mining industry.

17	LESSONS LEARNED AND CHALLENGES

Among the main lessons learned and challenges of implementing commingling in mine planning at Antamina, the following stand out:

•	Large-scale operations and mechanization: Commingling is primarily viable in large-volume mechanized operations, where it is possible to achieve a homogeneous and controlled mixture of materials. However, the magnitude of waste rock and tailings tonnages at Antamina implies a new challenge, never seen before.

•	Flexible planning and pilot tests: It is essential to include areas for pilot tests in short and medium-term plans, allowing adjustment of mixing parameters according to material variability.

•	Regulatory communication: Technology acceptance requires identifying precedents and working closely with authorities to develop appropriate regulatory frameworks.

•	Organizational change management: The transition to commingling involves challenges in personnel training and adaptation of operational processes, especially in mature operations.

18	CONCLUSIONS

•	The comparative discussion and case analysis of commingling implementation at Antamina have enabled the identification and selection of strategic alternatives that maximize the economic, environmental, and social value of the project.

•	The use of decision trees and scenario evaluation has facilitated informed decision-making, taking into account technical and economic feasibility as well as regulatory and environmental constraints.

•	The integration of commingling into strategic mine planning has proven to be a key tool for optimizing waste rock and tailings management, achieving a significant reduction in capital costs (up to 30%), greater efficiency in land use, and an extension of tailings storage facility (TSF) life. This approach has also aligned operations with international sustainability standards (GISTM) and strengthened risk management, contributing to operational continuity and long-term reduction of environmental impacts.

•	The Antamina experience demonstrates that structured case discussions and the application of comparative methodologies are fundamental for strategic planning, as they allow anticipation of challenges, validation of solutions through pilot testing, and adjustment of operational parameters based on real outcomes. Thus, commingling is consolidated as a transformative practice in modern mining, fostering more robust and sustainable decision-making for the development of long-life mining assets.

ACKNOWLEDGEMENTS

The authors express their sincere gratitude to Antamina for their invaluable support. Special thanks are extended to the Tailings Strategy Management team and the Long-Term Planning team for their collaboration and insights.

REFERENCES

Boshoff, J. et al. (2023). A case study on the commingling of tailings and waste rock at a Brownfields open cast mine in Ghana. University of Alberta.

Burden, R., & Wilson, G. W. (2023). Commingling of waste rock and tailings to improve “dry stack” performance: Design and evaluation of mixtures. Minerals, 13(2), 295.

Deza, N., & Montes, K. (2022). Mecanización del desmonte en Compañía Minera Antamina: Propuesta para capturar el valor tangible e intangible de la mecanización. Compañía Minera Antamina.

Green Policy Platform. (2024). Knowledge Gaps Report: Environmental Aspects of Tailings Management (January 2024).

ICMM. (2020). New global industry standard on tailings management. International Council on Mining and Metals.

Ulrich, B. & Coffin, J. (2015). TMW 2015 – Combined Tailings and Mine Waste.

Minemax. (2015). Why mine planning is all about collaboration. Minemax News.

Wheaton Precious Metals. (2015). Antamina Project overview.

INTEGRATING COMMINGLING INTO THE STRATEGIC WASTE ROCK PLANNING AT ANTAMINA MINE

Christa Quiroz1, David Machin1, Olimpia Cabrera1, Fernando Angeles1

1 Antamina Mine, Lima, Peru

(*Presenting author: cquirozc@antamina.com)

ABSTRACT

Antamina is a large open-pit polymetallic mine in Peru, operating with a processing capacity of 145 ktpd. In operation since 2001, the current mine life extends to 2036. Mining is conducted at a rate of 290 Mtpa using conventional truck-and-shovel equipment. This technical paper presents a comprehensive assessment of commingling as a transformative technology in mine planning, positioning waste rock and tailings management as a key factor in the decision-making process. The primary objective of this study is to demonstrate the benefits of this technology within the mine planning process, highlighting waste rock and tailings management as one of the main drivers in strategic decision-making. The methodology applied is a Comparative Case Analysis, which includes a review of the design and sequencing process for mining infrastructure such as waste rock storage facilities, along with a discussion of the key parameters considered. The results show that the implementation of this technology delivers significant economic benefits, including an approximate 30% reduction in capital costs for waste rock storage facilities, improved land-use efficiency, and optimized infrastructure development. Furthermore, the approach enhances alignment with the Global Industry Standard on Tailings Management (GISTM).

KEYWORDS

commingling, mine planning, waste rock management, tailings management, GISTM compliance

1	INTRODUCTION

The mining industry faces growing challenges related to the sustainable management of waste rock and tailings, especially after catastrophic events such as those that occurred in Brumadinho and Samarco (Green Policy Platform, 2024; ICMM, 2020). In this context, the Antamina mine, operating since 2001 as one of the largest polymetallic operations in the world, has developed a pioneering approach that integrates commingling as a central element of its strategic planning (Wheaton Precious Metals, 2015).

Commingling, defined as the homogeneous mixing of dewatered tailings with waste rock to form a geotechnically stable material, represents a natural evolution of traditional co-disposal practices (Burden & Wilson, 2023). However, what distinguishes the Antamina case is the integration of this technology from the initial phases of mine planning, making waste rock management the main driver of operational and strategic decisions.

2	OBJECTIVES

•	Demonstrate the benefits of integrating commingling into mine planning, positioning waste rock and tailings management as a strategic driver for decision-making in open-pit operations.

•	Optimize operational efficiency and environmental sustainability through the adoption of commingling in waste rock and tailings management.

•	Align waste rock and tailings management with international sustainability and safety standards, such as the GISTM, through the application of commingling.

•	Analyze, through comparative case studies, the impact of commingling on land-use optimization and mine life extension.

•	Reduce capital and operating costs associated with waste rock and tailings disposal by implementing commingling technologies.

•	Present the key findings and lessons learned from the integration of commingling into long-term mine planning.

3	TECHNICAL FOUNDATIONS OF COMMINGLING IN MINE PLANNING

3.1	Desing Principles and Material Characterization

Commingling at Antamina is based on engineering principles that seek to combine the superior structural properties of waste rock with the low permeability characteristics of tailings (Boshoff, 2023). This combination results in a material with shear strength similar to waste rock and permeability comparable to tailings, creating conditions that restrict oxygen entry and water filtration, significantly reducing the potential for acid drainage generation (Ulrich & Coffin, 2015; Burden & Wilson, 2023).

The characterization of commingled material is performed through analysis of five distinct structural configurations: rock-dominated uncompacted, fines-dominated uncompacted, rock-dominated compacted, fines-dominated compacted, and the "just-filled" configuration where fines exactly fill the voids between rock particles (Burden & Wilson, 2023). This latter configuration represents the optimal target of the commingling process at Antamina.

Figure 1 Blend configurations after Wickland et al. (2006). Source: Burden & Wilson (2023).

3.2	Integration into the Strategic Planning Process

The integration of commingling into Antamina's mine planning follows a structured methodological approach that places tailings management with waste rock as central elements of decision-making. This global strategy allows for optimizing the mixture of materials from the mine to the plant, considering the operational and environmental constraints associated with the tailings storage facility. By incorporating tailings behavior in the early stages of planning, greater efficiency in resource use and better risk management are achieved.

The global strategy is divided into three key axes:

•	Strategy 1: Ensure Future Expansions: The first strategy focuses on maximizing tailings storage capacity for current and future needs, leveraging Antamina's mineral resource and applying advanced technology to increase TSF (Tailings Storage Facility) capacity to ensure the mine's long-term operational continuity.

•	Strategy 2: Optimized footprint: The second strategy seeks to optimize the overall project footprint by balancing waste and tailings storage space. This approach aligns directly with commingling principles by maximizing efficient use of available land within the current operational footprint.

•	Strategy 3: Reduce upfront CAPEX: The third strategy focuses on reducing initial CAPEX to improve key financial indicators (IRR), which directly aligns with the demonstrated economic benefits of commingling, minimizing operational costs with a nearby TSF and optimizing CAPEX timing.

Figure 2 Strategic vision of mine planning approach with commingling

The strategies implemented in tailings dam and waste dump management seek to maximize the operational life of the mine and optimize asset value. These decisions translate into concrete actions that address sustainability, operational efficiency, and cost reduction, ensuring continuity and long-term success of the mining operation.

•	Long Life Asset: Antamina's vision as a long-life asset involves making decisions based on sustainable exploitation of its exceptional mineral resources. This includes adopting advanced technologies, such as commingling, to improve storage capacity and ensure operational continuity until 2036 and beyond, thus maximizing the value of the mineral resource.

•	Reduce Environmental Social Footprint: The strategic approach prioritizes reducing environmental and social impact through operation within the current footprint, increasing water availability through filtration, optimizing space use, and minimizing risks associated with tailings dams. Commingling contributes not only to technical and economic efficiency but also to sustainability objectives and corporate social responsibility.

•	Reduce Capital and Operational Cost: The cost reduction strategy focuses on optimizing capital investments (CAPEX) and their schedule, as well as minimizing operational costs through strategic facility location and leveraging synergies to avoid duplicate infrastructure. This enables more efficient and profitable resource management.

During the strategic planning phase, conceptual foundations for commingling implementation are established, including site characterization, regulatory framework development, and technical-economic feasibility analysis.

4	ANALYSIS OF THE MINING SYSTEM WITH COMMINGLING APPLICATION

4.1	Methodological Process Structure

The methodological process is organized in sequential stages that ensure effective integration of commingling into mine planning:

•	Comprehensive material evaluation: Detailed analysis of geotechnical and geochemical characteristics of available materials, identifying their suitability for mixtures and joint disposal.

•	Mixture design development: Design of optimized mixtures that maximize desired properties of commingled material, such as low permeability and high stability, minimizing acid water generation and improving mechanical behavior of the deposit.

•	Pilot test implementation: Execution of pilot and industrial scale tests to validate mixture performance and adjust design parameters according to results obtained under real operating conditions.

•	Monitoring and adjustment: Establishment of continuous monitoring system to evaluate consolidation, stability, and geochemical behavior of deposits, allowing operational and design adjustments as new information is collected.

5	IMPLEMENTATION AND EVALUATION METHODOLOGY FOR COMMINGLING IN MINE PLANNING

5.1	Comparative Case Analysis

The methodology applied at Antamina is based on Comparative Case Analysis, allowing for comprehensive review of the design and sequencing process of mining infrastructure, with emphasis on dams, waste rock dumps, and integration of commingling technologies. This methodological approach comprises:

•	Review of design and sequencing processes: Analysis of procedures used in waste dump planning and disposal, considering both geotechnical and geochemical criteria to ensure infrastructure stability and sustainability.

•	Identification of key variables: Selection of fundamental operational parameters, such as granulometry, permeability, geochemical behavior, and storage capacity, which directly affect the viability and performance of dumps and material mixtures.

•	Definition of case tree: Structure of a decision tree that contemplates long-term strategic scenarios, allowing comparison of alternatives under different combinations of variables and operational constraints.

•	Results evaluation: Application of sensitivity analyses and cost-benefit evaluations to determine the impact of each alternative on mine life extension, environmental risk reduction, and resource optimization.

5.2	Decision Tree for Strategic Planning

The decision tree developed for long-term strategic planning at Antamina follows sequential logic that allows evaluation of feasibility and impact of main operational alternatives:

•	Commingling feasibility: The first node of the tree determines whether integration of tailings and waste rock is technically and economically viable, considering material characteristics and regulatory constraints.

•	Entry into new area: If commingling is feasible, the next node evaluates the possibility of disposing material in a new area, analyzing topographic, environmental, and access aspects.

•	Need for new dam: Finally, it determines whether construction of a new tailings dam is essential or if the commingling solution allows dispensing with this infrastructure, optimizing investment and reducing environmental impact.

Each of these decisions conditions mine life extension, efficient space use, and environmental risk management, allowing selection of the alternative that maximizes economic, social, and environmental value of the project.

Figure 3  Decision Tree for Long-Term Strategic Scenario Planning

5.3	Evaluation of Operational Parameters and Modifications by Commingling

Key operational parameters—including waste rock-tailings ratio, mixing methods, transport systems, and processing criteria—are assessed for their impact on commingled system efficiency. The evaluation considers site-specific challenges such as extreme Andean weather, high-altitude logistics, and Peruvian regulations, all of which are especially critical at Antamina and require customized commingling solutions.

•	Modifications to tailings management by commingling:

The commingling process fundamentally transforms tailings management by utilizing voids within waste rock for storage, with available space typically equivalent to about 20% of the crushed rock volume transported by conveyors. To enable mixing, tailings must first be dewatered. The following image shows both the current process (without mechanization) and the transformation that occurs when commingling is incorporated.

Figure 4 Transformation of Tailings Management: Evolution from Conventional Process to Commingling|

•	 Operational parameters and assumptions

Commingling extends tailings facility life by optimizing spatial use of waste rock voids. Implementation requires mechanized systems (WCCS) for transporting crushed material mixed with dewatered tailings, while maintaining two key operational parameters: a) Unchanged transport capacity: 83.2 Mtpa in the conveyor system and b) Volumetric stability: No increase in total volume of waste rock dump.

This approach ensures efficient resource utilization and supports sustainable mine operations.

Table 1: Key operational parameters and assumptions

Component	Units	Without - Case Conventional	With - Case Commingling

Cycloned/Filtered Max. Capacity	Mtpa	-	32.0

Percentage of Voids in Waste Rock	% Volume 	 	20 %

Waste Rock / Tailings Ratio 	Mt Rock/

Mt Tails	-	83 / 16

Conventional Tailings Density	t/m3 	1.81	1.81

Fines Tailings Density	 t/m3	NA	1.60

Tailings Volume, TDR 4195 (to Dic-22)	Mm3	376	376

Tailings Capacity, TDR 4195 (to Dic-22)	Mt	682	652

Mill Feed Rate 	Ktpd	145	145

The following image shows how incorporation of mechanized systems and commingling add capacity, and therefore extend tailings facility life, to increase the life of mine and therefore the overall asset value.

Figure 5 Commingling Extension of Tailings Facility Life through Commingling

The curve below shows the relationship between dry density and optimal mixing proportion, determinant for maximizing commingling efficiency.

Figure 6: Dry density curve vs. optimal mixing proportion for commingling

•	Analysis of Mineable Resources, Production and Value

A three-dimensional diagram is used to strategically analyze the interactions between mineable resource volume, production rate, and generated value—whether economic, environmental, or operational. This tool helps identify how increasing resource volume and production can maximize value, but also highlights key constraints such as tailings dam capacity and property limits. The approach supports comprehensive decision-making to optimize asset value and ensure project sustainability across various operational scenarios.

Figure 7 Value Optimization: Interaction between Resource Volume, Production Rate and Key Restrictions

6	ECONOMIC AND OPERATIONAL ADVANTAGES

6.1	Perspective of Conventional Value Drivers

The following figure illustrates Antamina's main conventional value drivers from a financial perspective. In the current scenario, the amount of mineral resource that can be utilized directly depends on the capital that the company decides to invest (Capex), both in new investments (Growth Capex) and in maintaining existing operations (Sustaining Capex). This generates a necessary balance to determine the optimal scenario that maximizes the value of the existing asset.

Figure 8: Conventional Antamina Value Drivers: Financial Perspective (Without Commingling)

6.2	Impact of Commingling on Financial Value Drivers

From a financial perspective, the adoption of commingling among Antamina’s value drivers removes constraints on mineral resources by increasing tailings storage capacity, enabling the utilization of new dumps, and allowing for an expansion in the volume of usable resources. As a result, the mine’s operational life is extended and asset value is enhanced. Regarding dump utilization, the implementation of commingling has the potential to reduce the requirement for dumps located in karstic zones, which demand significant capital expenditure (Capex). This can lead to a reduction in initial investment by up to 30% in scenarios comparing commingling versus non-commingling approaches. This saving is achieved through the integration of previously separate infrastructures into a single system, which optimizes the use of available area in the East Extension dumps and prioritizes the use of mechanized systems over conventional trucking methods.

Figure 9: Antamina Value Drivers: Financial Perspective with Commingling

The optimization of land use represents another important economic benefit, especially relevant in mountainous locations like Antamina where available land for mining facilities is limited. Commingling allows for the creation of deposits with a smaller footprint compared to separate facilities, freeing up land for other productive uses or reducing the need for additional land acquisition.

7	ALIGNMENT WITH GISTM STANDARDS AND SUSTAINABILITY

The integration of commingling into mine planning strengthens compliance with the Global Industry Standard on Tailings Management (GISTM), by facilitating safer and more sustainable management of waste materials. The GISTM requires the planning, construction, operation, and closure of tailings facilities with a focus on risk reduction and continuous monitoring throughout the lifecycle. Commingling contributes to this objective by improving the geotechnical and geochemical stability of deposits, reducing the risk of failure and acid drainage generation, which in turn reduces environmental impact and long-term treatment costs. Additionally, the consolidation of facilities through commingling reduces the environmental footprint and facilitates supervision and control, aligning with the principles of sustainability and social responsibility in the mining industry.

8	LESSONS LEARNED AND CHALLENGES

Among the main lessons learned and challenges of implementing commingling in mine planning at Antamina, the following stand out:

•	Large-scale operations and mechanization: Commingling is primarily viable in large-volume mechanized operations, where it is possible to achieve a homogeneous and controlled mixture of materials. However, the magnitude of waste rock and tailings tonnages at Antamina implies a new challenge, never seen before.

•	Flexible planning and pilot tests: It is essential to include areas for pilot tests in short and medium-term plans, allowing adjustment of mixing parameters according to material variability.

•	Regulatory communication: Technology acceptance requires identifying precedents and working closely with authorities to develop appropriate regulatory frameworks.

•	Organizational change management: The transition to commingling involves challenges in personnel training and adaptation of operational processes, especially in mature operations.

9	CONCLUSIONS

•	The comparative discussion and case analysis of commingling implementation at Antamina have enabled the identification and selection of strategic alternatives that maximize the economic, environmental, and social value of the project.

•	The use of decision trees and scenario evaluation has facilitated informed decision-making, taking into account technical and economic feasibility as well as regulatory and environmental constraints.

•	The integration of commingling into strategic mine planning has proven to be a key tool for optimizing waste rock and tailings management, achieving a significant reduction in capital costs (up to 30%), greater efficiency in land use, and an extension of tailings storage facility (TSF) life. This approach has also aligned operations with international sustainability standards (GISTM) and strengthened risk management, contributing to operational continuity and long-term reduction of environmental impacts.

•	The Antamina experience demonstrates that structured case discussions and the application of comparative methodologies are fundamental for strategic planning, as they allow anticipation of challenges, validation of solutions through pilot testing, and adjustment of operational parameters based on real outcomes. Thus, commingling is consolidated as a transformative practice in modern mining, fostering more robust and sustainable decision-making for the development of long-life mining assets.

ACKNOWLEDGEMENTS

The authors express their sincere gratitude to Antamina for their invaluable support. Special thanks are extended to the Tailings Strategy Management team and the Long-Term Planning team for their collaboration and insights.

REFERENCES

Boshoff, J. et al. (2023). A case study on the commingling of tailings and waste rock at a Brownfields open cast mine in Ghana. University of Alberta.

Burden, R., & Wilson, G. W. (2023). Commingling of waste rock and tailings to improve “dry stack” performance: Design and evaluation of mixtures. Minerals, 13(2), 295.

Deza, N., & Montes, K. (2022). Mecanización del desmonte en Compañía Minera Antamina: Propuesta para capturar el valor tangible e intangible de la mecanización. Compañía Minera Antamina.

Green Policy Platform. (2024). Knowledge Gaps Report: Environmental Aspects of Tailings Management (January 2024).

ICMM. (2020). New global industry standard on tailings management. International Council on Mining and Metals.

Ulrich, B. & Coffin, J. (2015). TMW 2015 – Combined Tailings and Mine Waste.

Minemax. (2015). Why mine planning is all about collaboration. Minemax News.

Wheaton Precious Metals. (2015). Antamina Project overview.

INTEGRATING COMMINGLING INTO THE STRATEGIC WASTE ROCK PLANNING AT ANTAMINA MINE

Christa Quiroz1, David Machin1, Olimpia Cabrera1, Fernando Angeles1

1 Antamina Mine, Lima, Peru

(*Presenting author: cquirozc@antamina.com)

ABSTRACT

Antamina is a large open-pit polymetallic mine in Peru, operating with a processing capacity of 145 ktpd. In operation since 2001, the current mine life extends to 2036. Mining is conducted at a rate of 290 Mtpa using conventional truck-and-shovel equipment. This technical paper presents a comprehensive assessment of commingling as a transformative technology in mine planning, positioning waste rock and tailings management as a key factor in the decision-making process. The primary objective of this study is to demonstrate the benefits of this technology within the mine planning process, highlighting waste rock and tailings management as one of the main drivers in strategic decision-making. The methodology applied is a Comparative Case Analysis, which includes a review of the design and sequencing process for mining infrastructure such as waste rock storage facilities, along with a discussion of the key parameters considered. The results show that the implementation of this technology delivers significant economic benefits, including an approximate 30% reduction in capital costs for waste rock storage facilities, improved land-use efficiency, and optimized infrastructure development. Furthermore, the approach enhances alignment with the Global Industry Standard on Tailings Management (GISTM).

KEYWORDS

commingling, mine planning, waste rock management, tailings management, GISTM compliance

10	INTRODUCTION

The mining industry faces growing challenges related to the sustainable management of waste rock and tailings, especially after catastrophic events such as those that occurred in Brumadinho and Samarco (Green Policy Platform, 2024; ICMM, 2020). In this context, the Antamina mine, operating since 2001 as one of the largest polymetallic operations in the world, has developed a pioneering approach that integrates commingling as a central element of its strategic planning (Wheaton Precious Metals, 2015).

Commingling, defined as the homogeneous mixing of dewatered tailings with waste rock to form a geotechnically stable material, represents a natural evolution of traditional co-disposal practices (Burden & Wilson, 2023). However, what distinguishes the Antamina case is the integration of this technology from the initial phases of mine planning, making waste rock management the main driver of operational and strategic decisions.

11	OBJECTIVES

•	Demonstrate the benefits of integrating commingling into mine planning, positioning waste rock and tailings management as a strategic driver for decision-making in open-pit operations.

•	Optimize operational efficiency and environmental sustainability through the adoption of commingling in waste rock and tailings management.

•	Align waste rock and tailings management with international sustainability and safety standards, such as the GISTM, through the application of commingling.

•	Analyze, through comparative case studies, the impact of commingling on land-use optimization and mine life extension.

•	Reduce capital and operating costs associated with waste rock and tailings disposal by implementing commingling technologies.

•	Present the key findings and lessons learned from the integration of commingling into long-term mine planning.

12	TECHNICAL FOUNDATIONS OF COMMINGLING IN MINE PLANNING

12.1	Desing Principles and Material Characterization

Commingling at Antamina is based on engineering principles that seek to combine the superior structural properties of waste rock with the low permeability characteristics of tailings (Boshoff, 2023). This combination results in a material with shear strength similar to waste rock and permeability comparable to tailings, creating conditions that restrict oxygen entry and water filtration, significantly reducing the potential for acid drainage generation (Ulrich & Coffin, 2015; Burden & Wilson, 2023).

The characterization of commingled material is performed through analysis of five distinct structural configurations: rock-dominated uncompacted, fines-dominated uncompacted, rock-dominated compacted, fines-dominated compacted, and the "just-filled" configuration where fines exactly fill the voids between rock particles (Burden & Wilson, 2023). This latter configuration represents the optimal target of the commingling process at Antamina.

Figure 1 Blend configurations after Wickland et al. (2006). Source: Burden & Wilson (2023).

12.2	Integration into the Strategic Planning Process

The integration of commingling into Antamina's mine planning follows a structured methodological approach that places tailings management with waste rock as central elements of decision-making. This global strategy allows for optimizing the mixture of materials from the mine to the plant, considering the operational and environmental constraints associated with the tailings storage facility. By incorporating tailings behavior in the early stages of planning, greater efficiency in resource use and better risk management are achieved.

The global strategy is divided into three key axes:

•	Strategy 1: Ensure Future Expansions: The first strategy focuses on maximizing tailings storage capacity for current and future needs, leveraging Antamina's mineral resource and applying advanced technology to increase TSF (Tailings Storage Facility) capacity to ensure the mine's long-term operational continuity.

•	Strategy 2: Optimized footprint: The second strategy seeks to optimize the overall project footprint by balancing waste and tailings storage space. This approach aligns directly with commingling principles by maximizing efficient use of available land within the current operational footprint.

•	Strategy 3: Reduce upfront CAPEX: The third strategy focuses on reducing initial CAPEX to improve key financial indicators (IRR), which directly aligns with the demonstrated economic benefits of commingling, minimizing operational costs with a nearby TSF and optimizing CAPEX timing.

Figure 2 Strategic vision of mine planning approach with commingling

The strategies implemented in tailings dam and waste dump management seek to maximize the operational life of the mine and optimize asset value. These decisions translate into concrete actions that address sustainability, operational efficiency, and cost reduction, ensuring continuity and long-term success of the mining operation.

•	Long Life Asset: Antamina's vision as a long-life asset involves making decisions based on sustainable exploitation of its exceptional mineral resources. This includes adopting advanced technologies, such as commingling, to improve storage capacity and ensure operational continuity until 2036 and beyond, thus maximizing the value of the mineral resource.

•	Reduce Environmental Social Footprint: The strategic approach prioritizes reducing environmental and social impact through operation within the current footprint, increasing water availability through filtration, optimizing space use, and minimizing risks associated with tailings dams. Commingling contributes not only to technical and economic efficiency but also to sustainability objectives and corporate social responsibility.

•	Reduce Capital and Operational Cost: The cost reduction strategy focuses on optimizing capital investments (CAPEX) and their schedule, as well as minimizing operational costs through strategic facility location and leveraging synergies to avoid duplicate infrastructure. This enables more efficient and profitable resource management.

During the strategic planning phase, conceptual foundations for commingling implementation are established, including site characterization, regulatory framework development, and technical-economic feasibility analysis.

13	ANALYSIS OF THE MINING SYSTEM WITH COMMINGLING APPLICATION

13.1	Methodological Process Structure

The methodological process is organized in sequential stages that ensure effective integration of commingling into mine planning:

•	Comprehensive material evaluation: Detailed analysis of geotechnical and geochemical characteristics of available materials, identifying their suitability for mixtures and joint disposal.

•	Mixture design development: Design of optimized mixtures that maximize desired properties of commingled material, such as low permeability and high stability, minimizing acid water generation and improving mechanical behavior of the deposit.

•	Pilot test implementation: Execution of pilot and industrial scale tests to validate mixture performance and adjust design parameters according to results obtained under real operating conditions.

•	Monitoring and adjustment: Establishment of continuous monitoring system to evaluate consolidation, stability, and geochemical behavior of deposits, allowing operational and design adjustments as new information is collected.

14	IMPLEMENTATION AND EVALUATION METHODOLOGY FOR COMMINGLING IN MINE PLANNING

14.1	Comparative Case Analysis

The methodology applied at Antamina is based on Comparative Case Analysis, allowing for comprehensive review of the design and sequencing process of mining infrastructure, with emphasis on dams, waste rock dumps, and integration of commingling technologies. This methodological approach comprises:

•	Review of design and sequencing processes: Analysis of procedures used in waste dump planning and disposal, considering both geotechnical and geochemical criteria to ensure infrastructure stability and sustainability.

•	Identification of key variables: Selection of fundamental operational parameters, such as granulometry, permeability, geochemical behavior, and storage capacity, which directly affect the viability and performance of dumps and material mixtures.

•	Definition of case tree: Structure of a decision tree that contemplates long-term strategic scenarios, allowing comparison of alternatives under different combinations of variables and operational constraints.

•	Results evaluation: Application of sensitivity analyses and cost-benefit evaluations to determine the impact of each alternative on mine life extension, environmental risk reduction, and resource optimization.

14.2	Decision Tree for Strategic Planning

The decision tree developed for long-term strategic planning at Antamina follows sequential logic that allows evaluation of feasibility and impact of main operational alternatives:

•	Commingling feasibility: The first node of the tree determines whether integration of tailings and waste rock is technically and economically viable, considering material characteristics and regulatory constraints.

•	Entry into new area: If commingling is feasible, the next node evaluates the possibility of disposing material in a new area, analyzing topographic, environmental, and access aspects.

•	Need for new dam: Finally, it determines whether construction of a new tailings dam is essential or if the commingling solution allows dispensing with this infrastructure, optimizing investment and reducing environmental impact.

Each of these decisions conditions mine life extension, efficient space use, and environmental risk management, allowing selection of the alternative that maximizes economic, social, and environmental value of the project.

Figure 3  Decision Tree for Long-Term Strategic Scenario Planning

14.3	Evaluation of Operational Parameters and Modifications by Commingling

Key operational parameters—including waste rock-tailings ratio, mixing methods, transport systems, and processing criteria—are assessed for their impact on commingled system efficiency. The evaluation considers site-specific challenges such as extreme Andean weather, high-altitude logistics, and Peruvian regulations, all of which are especially critical at Antamina and require customized commingling solutions.

•	Modifications to tailings management by commingling:

The commingling process fundamentally transforms tailings management by utilizing voids within waste rock for storage, with available space typically equivalent to about 20% of the crushed rock volume transported by conveyors. To enable mixing, tailings must first be dewatered. The following image shows both the current process (without mechanization) and the transformation that occurs when commingling is incorporated.

Figure 4 Transformation of Tailings Management: Evolution from Conventional Process to Commingling|

•	 Operational parameters and assumptions

Commingling extends tailings facility life by optimizing spatial use of waste rock voids. Implementation requires mechanized systems (WCCS) for transporting crushed material mixed with dewatered tailings, while maintaining two key operational parameters: a) Unchanged transport capacity: 83.2 Mtpa in the conveyor system and b) Volumetric stability: No increase in total volume of waste rock dump.

This approach ensures efficient resource utilization and supports sustainable mine operations.

Table 1: Key operational parameters and assumptions

Component	Units	Without - Case Conventional	With - Case Commingling

Cycloned/Filtered Max. Capacity	Mtpa	-	32.0

Percentage of Voids in Waste Rock	% Volume 	 	20 %

Waste Rock / Tailings Ratio 	Mt Rock/

Mt Tails	-	83 / 16

Conventional Tailings Density	t/m3 	1.81	1.81

Fines Tailings Density	 t/m3	NA	1.60

Tailings Volume, TDR 4195 (to Dic-22)	Mm3	376	376

Tailings Capacity, TDR 4195 (to Dic-22)	Mt	682	652

Mill Feed Rate 	Ktpd	145	145

The following image shows how incorporation of mechanized systems and commingling add capacity, and therefore extend tailings facility life, to increase the life of mine and therefore the overall asset value.

Figure 5 Commingling Extension of Tailings Facility Life through Commingling

The curve below shows the relationship between dry density and optimal mixing proportion, determinant for maximizing commingling efficiency.

Figure 6: Dry density curve vs. optimal mixing proportion for commingling

•	Analysis of Mineable Resources, Production and Value

A three-dimensional diagram is used to strategically analyze the interactions between mineable resource volume, production rate, and generated value—whether economic, environmental, or operational. This tool helps identify how increasing resource volume and production can maximize value, but also highlights key constraints such as tailings dam capacity and property limits. The approach supports comprehensive decision-making to optimize asset value and ensure project sustainability across various operational scenarios.

Figure 7 Value Optimization: Interaction between Resource Volume, Production Rate and Key Restrictions

15	ECONOMIC AND OPERATIONAL ADVANTAGES

15.1	Perspective of Conventional Value Drivers

The following figure illustrates Antamina's main conventional value drivers from a financial perspective. In the current scenario, the amount of mineral resource that can be utilized directly depends on the capital that the company decides to invest (Capex), both in new investments (Growth Capex) and in maintaining existing operations (Sustaining Capex). This generates a necessary balance to determine the optimal scenario that maximizes the value of the existing asset.

Figure 8: Conventional Antamina Value Drivers: Financial Perspective (Without Commingling)

15.2	Impact of Commingling on Financial Value Drivers

From a financial perspective, the adoption of commingling among Antamina’s value drivers removes constraints on mineral resources by increasing tailings storage capacity, enabling the utilization of new dumps, and allowing for an expansion in the volume of usable resources. As a result, the mine’s operational life is extended and asset value is enhanced. Regarding dump utilization, the implementation of commingling has the potential to reduce the requirement for dumps located in karstic zones, which demand significant capital expenditure (Capex). This can lead to a reduction in initial investment by up to 30% in scenarios comparing commingling versus non-commingling approaches. This saving is achieved through the integration of previously separate infrastructures into a single system, which optimizes the use of available area in the East Extension dumps and prioritizes the use of mechanized systems over conventional trucking methods.

Figure 9: Antamina Value Drivers: Financial Perspective with Commingling

The optimization of land use represents another important economic benefit, especially relevant in mountainous locations like Antamina where available land for mining facilities is limited. Commingling allows for the creation of deposits with a smaller footprint compared to separate facilities, freeing up land for other productive uses or reducing the need for additional land acquisition.

16	ALIGNMENT WITH GISTM STANDARDS AND SUSTAINABILITY

The integration of commingling into mine planning strengthens compliance with the Global Industry Standard on Tailings Management (GISTM), by facilitating safer and more sustainable management of waste materials. The GISTM requires the planning, construction, operation, and closure of tailings facilities with a focus on risk reduction and continuous monitoring throughout the lifecycle. Commingling contributes to this objective by improving the geotechnical and geochemical stability of deposits, reducing the risk of failure and acid drainage generation, which in turn reduces environmental impact and long-term treatment costs. Additionally, the consolidation of facilities through commingling reduces the environmental footprint and facilitates supervision and control, aligning with the principles of sustainability and social responsibility in the mining industry.

17	LESSONS LEARNED AND CHALLENGES

Among the main lessons learned and challenges of implementing commingling in mine planning at Antamina, the following stand out:

•	Large-scale operations and mechanization: Commingling is primarily viable in large-volume mechanized operations, where it is possible to achieve a homogeneous and controlled mixture of materials. However, the magnitude of waste rock and tailings tonnages at Antamina implies a new challenge, never seen before.

•	Flexible planning and pilot tests: It is essential to include areas for pilot tests in short and medium-term plans, allowing adjustment of mixing parameters according to material variability.

•	Regulatory communication: Technology acceptance requires identifying precedents and working closely with authorities to develop appropriate regulatory frameworks.

•	Organizational change management: The transition to commingling involves challenges in personnel training and adaptation of operational processes, especially in mature operations.

18	CONCLUSIONS

•	The comparative discussion and case analysis of commingling implementation at Antamina have enabled the identification and selection of strategic alternatives that maximize the economic, environmental, and social value of the project.

•	The use of decision trees and scenario evaluation has facilitated informed decision-making, taking into account technical and economic feasibility as well as regulatory and environmental constraints.

•	The integration of commingling into strategic mine planning has proven to be a key tool for optimizing waste rock and tailings management, achieving a significant reduction in capital costs (up to 30%), greater efficiency in land use, and an extension of tailings storage facility (TSF) life. This approach has also aligned operations with international sustainability standards (GISTM) and strengthened risk management, contributing to operational continuity and long-term reduction of environmental impacts.

•	The Antamina experience demonstrates that structured case discussions and the application of comparative methodologies are fundamental for strategic planning, as they allow anticipation of challenges, validation of solutions through pilot testing, and adjustment of operational parameters based on real outcomes. Thus, commingling is consolidated as a transformative practice in modern mining, fostering more robust and sustainable decision-making for the development of long-life mining assets.

ACKNOWLEDGEMENTS

The authors express their sincere gratitude to Antamina for their invaluable support. Special thanks are extended to the Tailings Strategy Management team and the Long-Term Planning team for their collaboration and insights.

REFERENCES

Boshoff, J. et al. (2023). A case study on the commingling of tailings and waste rock at a Brownfields open cast mine in Ghana. University of Alberta.

Burden, R., & Wilson, G. W. (2023). Commingling of waste rock and tailings to improve “dry stack” performance: Design and evaluation of mixtures. Minerals, 13(2), 295.

Deza, N., & Montes, K. (2022). Mecanización del desmonte en Compañía Minera Antamina: Propuesta para capturar el valor tangible e intangible de la mecanización. Compañía Minera Antamina.

Green Policy Platform. (2024). Knowledge Gaps Report: Environmental Aspects of Tailings Management (January 2024).

ICMM. (2020). New global industry standard on tailings management. International Council on Mining and Metals.

Ulrich, B. & Coffin, J. (2015). TMW 2015 – Combined Tailings and Mine Waste.

Minemax. (2015). Why mine planning is all about collaboration. Minemax News.

Wheaton Precious Metals. (2015). Antamina Project overview.

INTEGRATING COMMINGLING INTO THE STRATEGIC WASTE ROCK PLANNING AT ANTAMINA MINE

Christa Quiroz1, David Machin1, Olimpia Cabrera1, Fernando Angeles1

1 Antamina Mine, Lima, Peru

(*Presenting author: cquirozc@antamina.com)

ABSTRACT

Antamina is a large open-pit polymetallic mine in Peru, operating with a processing capacity of 145 ktpd. In operation since 2001, the current mine life extends to 2036. Mining is conducted at a rate of 290 Mtpa using conventional truck-and-shovel equipment. This technical paper presents a comprehensive assessment of commingling as a transformative technology in mine planning, positioning waste rock and tailings management as a key factor in the decision-making process. The primary objective of this study is to demonstrate the benefits of this technology within the mine planning process, highlighting waste rock and tailings management as one of the main drivers in strategic decision-making. The methodology applied is a Comparative Case Analysis, which includes a review of the design and sequencing process for mining infrastructure such as waste rock storage facilities, along with a discussion of the key parameters considered. The results show that the implementation of this technology delivers significant economic benefits, including an approximate 30% reduction in capital costs for waste rock storage facilities, improved land-use efficiency, and optimized infrastructure development. Furthermore, the approach enhances alignment with the Global Industry Standard on Tailings Management (GISTM).

KEYWORDS

commingling, mine planning, waste rock management, tailings management, GISTM compliance

1	INTRODUCTION

The mining industry faces growing challenges related to the sustainable management of waste rock and tailings, especially after catastrophic events such as those that occurred in Brumadinho and Samarco (Green Policy Platform, 2024; ICMM, 2020). In this context, the Antamina mine, operating since 2001 as one of the largest polymetallic operations in the world, has developed a pioneering approach that integrates commingling as a central element of its strategic planning (Wheaton Precious Metals, 2015).

Commingling, defined as the homogeneous mixing of dewatered tailings with waste rock to form a geotechnically stable material, represents a natural evolution of traditional co-disposal practices (Burden & Wilson, 2023). However, what distinguishes the Antamina case is the integration of this technology from the initial phases of mine planning, making waste rock management the main driver of operational and strategic decisions.

2	OBJECTIVES

•	Demonstrate the benefits of integrating commingling into mine planning, positioning waste rock and tailings management as a strategic driver for decision-making in open-pit operations.

•	Optimize operational efficiency and environmental sustainability through the adoption of commingling in waste rock and tailings management.

•	Align waste rock and tailings management with international sustainability and safety standards, such as the GISTM, through the application of commingling.

•	Analyze, through comparative case studies, the impact of commingling on land-use optimization and mine life extension.

•	Reduce capital and operating costs associated with waste rock and tailings disposal by implementing commingling technologies.

•	Present the key findings and lessons learned from the integration of commingling into long-term mine planning.

3	TECHNICAL FOUNDATIONS OF COMMINGLING IN MINE PLANNING

3.1	Desing Principles and Material Characterization

Commingling at Antamina is based on engineering principles that seek to combine the superior structural properties of waste rock with the low permeability characteristics of tailings (Boshoff, 2023). This combination results in a material with shear strength similar to waste rock and permeability comparable to tailings, creating conditions that restrict oxygen entry and water filtration, significantly reducing the potential for acid drainage generation (Ulrich & Coffin, 2015; Burden & Wilson, 2023).

The characterization of commingled material is performed through analysis of five distinct structural configurations: rock-dominated uncompacted, fines-dominated uncompacted, rock-dominated compacted, fines-dominated compacted, and the "just-filled" configuration where fines exactly fill the voids between rock particles (Burden & Wilson, 2023). This latter configuration represents the optimal target of the commingling process at Antamina.

Figure 1 Blend configurations after Wickland et al. (2006). Source: Burden & Wilson (2023).

3.2	Integration into the Strategic Planning Process

The integration of commingling into Antamina's mine planning follows a structured methodological approach that places tailings management with waste rock as central elements of decision-making. This global strategy allows for optimizing the mixture of materials from the mine to the plant, considering the operational and environmental constraints associated with the tailings storage facility. By incorporating tailings behavior in the early stages of planning, greater efficiency in resource use and better risk management are achieved.

The global strategy is divided into three key axes:

•	Strategy 1: Ensure Future Expansions: The first strategy focuses on maximizing tailings storage capacity for current and future needs, leveraging Antamina's mineral resource and applying advanced technology to increase TSF (Tailings Storage Facility) capacity to ensure the mine's long-term operational continuity.

•	Strategy 2: Optimized footprint: The second strategy seeks to optimize the overall project footprint by balancing waste and tailings storage space. This approach aligns directly with commingling principles by maximizing efficient use of available land within the current operational footprint.

•	Strategy 3: Reduce upfront CAPEX: The third strategy focuses on reducing initial CAPEX to improve key financial indicators (IRR), which directly aligns with the demonstrated economic benefits of commingling, minimizing operational costs with a nearby TSF and optimizing CAPEX timing.

Figure 2 Strategic vision of mine planning approach with commingling

The strategies implemented in tailings dam and waste dump management seek to maximize the operational life of the mine and optimize asset value. These decisions translate into concrete actions that address sustainability, operational efficiency, and cost reduction, ensuring continuity and long-term success of the mining operation.

•	Long Life Asset: Antamina's vision as a long-life asset involves making decisions based on sustainable exploitation of its exceptional mineral resources. This includes adopting advanced technologies, such as commingling, to improve storage capacity and ensure operational continuity until 2036 and beyond, thus maximizing the value of the mineral resource.

•	Reduce Environmental Social Footprint: The strategic approach prioritizes reducing environmental and social impact through operation within the current footprint, increasing water availability through filtration, optimizing space use, and minimizing risks associated with tailings dams. Commingling contributes not only to technical and economic efficiency but also to sustainability objectives and corporate social responsibility.

•	Reduce Capital and Operational Cost: The cost reduction strategy focuses on optimizing capital investments (CAPEX) and their schedule, as well as minimizing operational costs through strategic facility location and leveraging synergies to avoid duplicate infrastructure. This enables more efficient and profitable resource management.

During the strategic planning phase, conceptual foundations for commingling implementation are established, including site characterization, regulatory framework development, and technical-economic feasibility analysis.

4	ANALYSIS OF THE MINING SYSTEM WITH COMMINGLING APPLICATION

4.1	Methodological Process Structure

The methodological process is organized in sequential stages that ensure effective integration of commingling into mine planning:

•	Comprehensive material evaluation: Detailed analysis of geotechnical and geochemical characteristics of available materials, identifying their suitability for mixtures and joint disposal.

•	Mixture design development: Design of optimized mixtures that maximize desired properties of commingled material, such as low permeability and high stability, minimizing acid water generation and improving mechanical behavior of the deposit.

•	Pilot test implementation: Execution of pilot and industrial scale tests to validate mixture performance and adjust design parameters according to results obtained under real operating conditions.

•	Monitoring and adjustment: Establishment of continuous monitoring system to evaluate consolidation, stability, and geochemical behavior of deposits, allowing operational and design adjustments as new information is collected.

5	IMPLEMENTATION AND EVALUATION METHODOLOGY FOR COMMINGLING IN MINE PLANNING

5.1	Comparative Case Analysis

The methodology applied at Antamina is based on Comparative Case Analysis, allowing for comprehensive review of the design and sequencing process of mining infrastructure, with emphasis on dams, waste rock dumps, and integration of commingling technologies. This methodological approach comprises:

•	Review of design and sequencing processes: Analysis of procedures used in waste dump planning and disposal, considering both geotechnical and geochemical criteria to ensure infrastructure stability and sustainability.

•	Identification of key variables: Selection of fundamental operational parameters, such as granulometry, permeability, geochemical behavior, and storage capacity, which directly affect the viability and performance of dumps and material mixtures.

•	Definition of case tree: Structure of a decision tree that contemplates long-term strategic scenarios, allowing comparison of alternatives under different combinations of variables and operational constraints.

•	Results evaluation: Application of sensitivity analyses and cost-benefit evaluations to determine the impact of each alternative on mine life extension, environmental risk reduction, and resource optimization.

5.2	Decision Tree for Strategic Planning

The decision tree developed for long-term strategic planning at Antamina follows sequential logic that allows evaluation of feasibility and impact of main operational alternatives:

•	Commingling feasibility: The first node of the tree determines whether integration of tailings and waste rock is technically and economically viable, considering material characteristics and regulatory constraints.

•	Entry into new area: If commingling is feasible, the next node evaluates the possibility of disposing material in a new area, analyzing topographic, environmental, and access aspects.

•	Need for new dam: Finally, it determines whether construction of a new tailings dam is essential or if the commingling solution allows dispensing with this infrastructure, optimizing investment and reducing environmental impact.

Each of these decisions conditions mine life extension, efficient space use, and environmental risk management, allowing selection of the alternative that maximizes economic, social, and environmental value of the project.

Figure 3  Decision Tree for Long-Term Strategic Scenario Planning

5.3	Evaluation of Operational Parameters and Modifications by Commingling

Key operational parameters—including waste rock-tailings ratio, mixing methods, transport systems, and processing criteria—are assessed for their impact on commingled system efficiency. The evaluation considers site-specific challenges such as extreme Andean weather, high-altitude logistics, and Peruvian regulations, all of which are especially critical at Antamina and require customized commingling solutions.

•	Modifications to tailings management by commingling:

The commingling process fundamentally transforms tailings management by utilizing voids within waste rock for storage, with available space typically equivalent to about 20% of the crushed rock volume transported by conveyors. To enable mixing, tailings must first be dewatered. The following image shows both the current process (without mechanization) and the transformation that occurs when commingling is incorporated.

Figure 4 Transformation of Tailings Management: Evolution from Conventional Process to Commingling|

•	 Operational parameters and assumptions

Commingling extends tailings facility life by optimizing spatial use of waste rock voids. Implementation requires mechanized systems (WCCS) for transporting crushed material mixed with dewatered tailings, while maintaining two key operational parameters: a) Unchanged transport capacity: 83.2 Mtpa in the conveyor system and b) Volumetric stability: No increase in total volume of waste rock dump.

This approach ensures efficient resource utilization and supports sustainable mine operations.

Table 1: Key operational parameters and assumptions

Component	Units	Without - Case Conventional	With - Case Commingling

Cycloned/Filtered Max. Capacity	Mtpa	-	32.0

Percentage of Voids in Waste Rock	% Volume 	 	20 %

Waste Rock / Tailings Ratio 	Mt Rock/

Mt Tails	-	83 / 16

Conventional Tailings Density	t/m3 	1.81	1.81

Fines Tailings Density	 t/m3	NA	1.60

Tailings Volume, TDR 4195 (to Dic-22)	Mm3	376	376

Tailings Capacity, TDR 4195 (to Dic-22)	Mt	682	652

Mill Feed Rate 	Ktpd	145	145

The following image shows how incorporation of mechanized systems and commingling add capacity, and therefore extend tailings facility life, to increase the life of mine and therefore the overall asset value.

Figure 5 Commingling Extension of Tailings Facility Life through Commingling

The curve below shows the relationship between dry density and optimal mixing proportion, determinant for maximizing commingling efficiency.

Figure 6: Dry density curve vs. optimal mixing proportion for commingling

•	Analysis of Mineable Resources, Production and Value

A three-dimensional diagram is used to strategically analyze the interactions between mineable resource volume, production rate, and generated value—whether economic, environmental, or operational. This tool helps identify how increasing resource volume and production can maximize value, but also highlights key constraints such as tailings dam capacity and property limits. The approach supports comprehensive decision-making to optimize asset value and ensure project sustainability across various operational scenarios.

Figure 7 Value Optimization: Interaction between Resource Volume, Production Rate and Key Restrictions

6	ECONOMIC AND OPERATIONAL ADVANTAGES

6.1	Perspective of Conventional Value Drivers

The following figure illustrates Antamina's main conventional value drivers from a financial perspective. In the current scenario, the amount of mineral resource that can be utilized directly depends on the capital that the company decides to invest (Capex), both in new investments (Growth Capex) and in maintaining existing operations (Sustaining Capex). This generates a necessary balance to determine the optimal scenario that maximizes the value of the existing asset.

Figure 8: Conventional Antamina Value Drivers: Financial Perspective (Without Commingling)

6.2	Impact of Commingling on Financial Value Drivers

From a financial perspective, the adoption of commingling among Antamina’s value drivers removes constraints on mineral resources by increasing tailings storage capacity, enabling the utilization of new dumps, and allowing for an expansion in the volume of usable resources. As a result, the mine’s operational life is extended and asset value is enhanced. Regarding dump utilization, the implementation of commingling has the potential to reduce the requirement for dumps located in karstic zones, which demand significant capital expenditure (Capex). This can lead to a reduction in initial investment by up to 30% in scenarios comparing commingling versus non-commingling approaches. This saving is achieved through the integration of previously separate infrastructures into a single system, which optimizes the use of available area in the East Extension dumps and prioritizes the use of mechanized systems over conventional trucking methods.

Figure 9: Antamina Value Drivers: Financial Perspective with Commingling

The optimization of land use represents another important economic benefit, especially relevant in mountainous locations like Antamina where available land for mining facilities is limited. Commingling allows for the creation of deposits with a smaller footprint compared to separate facilities, freeing up land for other productive uses or reducing the need for additional land acquisition.

7	ALIGNMENT WITH GISTM STANDARDS AND SUSTAINABILITY

The integration of commingling into mine planning strengthens compliance with the Global Industry Standard on Tailings Management (GISTM), by facilitating safer and more sustainable management of waste materials. The GISTM requires the planning, construction, operation, and closure of tailings facilities with a focus on risk reduction and continuous monitoring throughout the lifecycle. Commingling contributes to this objective by improving the geotechnical and geochemical stability of deposits, reducing the risk of failure and acid drainage generation, which in turn reduces environmental impact and long-term treatment costs. Additionally, the consolidation of facilities through commingling reduces the environmental footprint and facilitates supervision and control, aligning with the principles of sustainability and social responsibility in the mining industry.

8	LESSONS LEARNED AND CHALLENGES

Among the main lessons learned and challenges of implementing commingling in mine planning at Antamina, the following stand out:

•	Large-scale operations and mechanization: Commingling is primarily viable in large-volume mechanized operations, where it is possible to achieve a homogeneous and controlled mixture of materials. However, the magnitude of waste rock and tailings tonnages at Antamina implies a new challenge, never seen before.

•	Flexible planning and pilot tests: It is essential to include areas for pilot tests in short and medium-term plans, allowing adjustment of mixing parameters according to material variability.

•	Regulatory communication: Technology acceptance requires identifying precedents and working closely with authorities to develop appropriate regulatory frameworks.

•	Organizational change management: The transition to commingling involves challenges in personnel training and adaptation of operational processes, especially in mature operations.

9	CONCLUSIONS

•	The comparative discussion and case analysis of commingling implementation at Antamina have enabled the identification and selection of strategic alternatives that maximize the economic, environmental, and social value of the project.

•	The use of decision trees and scenario evaluation has facilitated informed decision-making, taking into account technical and economic feasibility as well as regulatory and environmental constraints.

•	The integration of commingling into strategic mine planning has proven to be a key tool for optimizing waste rock and tailings management, achieving a significant reduction in capital costs (up to 30%), greater efficiency in land use, and an extension of tailings storage facility (TSF) life. This approach has also aligned operations with international sustainability standards (GISTM) and strengthened risk management, contributing to operational continuity and long-term reduction of environmental impacts.

•	The Antamina experience demonstrates that structured case discussions and the application of comparative methodologies are fundamental for strategic planning, as they allow anticipation of challenges, validation of solutions through pilot testing, and adjustment of operational parameters based on real outcomes. Thus, commingling is consolidated as a transformative practice in modern mining, fostering more robust and sustainable decision-making for the development of long-life mining assets.

ACKNOWLEDGEMENTS

The authors express their sincere gratitude to Antamina for their invaluable support. Special thanks are extended to the Tailings Strategy Management team and the Long-Term Planning team for their collaboration and insights.

REFERENCES

Boshoff, J. et al. (2023). A case study on the commingling of tailings and waste rock at a Brownfields open cast mine in Ghana. University of Alberta.

Burden, R., & Wilson, G. W. (2023). Commingling of waste rock and tailings to improve “dry stack” performance: Design and evaluation of mixtures. Minerals, 13(2), 295.

Deza, N., & Montes, K. (2022). Mecanización del desmonte en Compañía Minera Antamina: Propuesta para capturar el valor tangible e intangible de la mecanización. Compañía Minera Antamina.

Green Policy Platform. (2024). Knowledge Gaps Report: Environmental Aspects of Tailings Management (January 2024).

ICMM. (2020). New global industry standard on tailings management. International Council on Mining and Metals.

Ulrich, B. & Coffin, J. (2015). TMW 2015 – Combined Tailings and Mine Waste.

Minemax. (2015). Why mine planning is all about collaboration. Minemax News.

Wheaton Precious Metals. (2015). Antamina Project overview.

INTEGRATING COMMINGLING INTO THE STRATEGIC WASTE ROCK PLANNING AT ANTAMINA MINE

Christa Quiroz1, David Machin1, Olimpia Cabrera1, Fernando Angeles1

1 Antamina Mine, Lima, Peru

(*Presenting author: cquirozc@antamina.com)

ABSTRACT

Antamina is a large open-pit polymetallic mine in Peru, operating with a processing capacity of 145 ktpd. In operation since 2001, the current mine life extends to 2036. Mining is conducted at a rate of 290 Mtpa using conventional truck-and-shovel equipment. This technical paper presents a comprehensive assessment of commingling as a transformative technology in mine planning, positioning waste rock and tailings management as a key factor in the decision-making process. The primary objective of this study is to demonstrate the benefits of this technology within the mine planning process, highlighting waste rock and tailings management as one of the main drivers in strategic decision-making. The methodology applied is a Comparative Case Analysis, which includes a review of the design and sequencing process for mining infrastructure such as waste rock storage facilities, along with a discussion of the key parameters considered. The results show that the implementation of this technology delivers significant economic benefits, including an approximate 30% reduction in capital costs for waste rock storage facilities, improved land-use efficiency, and optimized infrastructure development. Furthermore, the approach enhances alignment with the Global Industry Standard on Tailings Management (GISTM).

KEYWORDS

commingling, mine planning, waste rock management, tailings management, GISTM compliance

10	INTRODUCTION

The mining industry faces growing challenges related to the sustainable management of waste rock and tailings, especially after catastrophic events such as those that occurred in Brumadinho and Samarco (Green Policy Platform, 2024; ICMM, 2020). In this context, the Antamina mine, operating since 2001 as one of the largest polymetallic operations in the world, has developed a pioneering approach that integrates commingling as a central element of its strategic planning (Wheaton Precious Metals, 2015).

Commingling, defined as the homogeneous mixing of dewatered tailings with waste rock to form a geotechnically stable material, represents a natural evolution of traditional co-disposal practices (Burden & Wilson, 2023). However, what distinguishes the Antamina case is the integration of this technology from the initial phases of mine planning, making waste rock management the main driver of operational and strategic decisions.

11	OBJECTIVES

•	Demonstrate the benefits of integrating commingling into mine planning, positioning waste rock and tailings management as a strategic driver for decision-making in open-pit operations.

•	Optimize operational efficiency and environmental sustainability through the adoption of commingling in waste rock and tailings management.

•	Align waste rock and tailings management with international sustainability and safety standards, such as the GISTM, through the application of commingling.

•	Analyze, through comparative case studies, the impact of commingling on land-use optimization and mine life extension.

•	Reduce capital and operating costs associated with waste rock and tailings disposal by implementing commingling technologies.

•	Present the key findings and lessons learned from the integration of commingling into long-term mine planning.

12	TECHNICAL FOUNDATIONS OF COMMINGLING IN MINE PLANNING

12.1	Desing Principles and Material Characterization

Commingling at Antamina is based on engineering principles that seek to combine the superior structural properties of waste rock with the low permeability characteristics of tailings (Boshoff, 2023). This combination results in a material with shear strength similar to waste rock and permeability comparable to tailings, creating conditions that restrict oxygen entry and water filtration, significantly reducing the potential for acid drainage generation (Ulrich & Coffin, 2015; Burden & Wilson, 2023).

The characterization of commingled material is performed through analysis of five distinct structural configurations: rock-dominated uncompacted, fines-dominated uncompacted, rock-dominated compacted, fines-dominated compacted, and the "just-filled" configuration where fines exactly fill the voids between rock particles (Burden & Wilson, 2023). This latter configuration represents the optimal target of the commingling process at Antamina.

Figure 1 Blend configurations after Wickland et al. (2006). Source: Burden & Wilson (2023).

12.2	Integration into the Strategic Planning Process

The integration of commingling into Antamina's mine planning follows a structured methodological approach that places tailings management with waste rock as central elements of decision-making. This global strategy allows for optimizing the mixture of materials from the mine to the plant, considering the operational and environmental constraints associated with the tailings storage facility. By incorporating tailings behavior in the early stages of planning, greater efficiency in resource use and better risk management are achieved.

The global strategy is divided into three key axes:

•	Strategy 1: Ensure Future Expansions: The first strategy focuses on maximizing tailings storage capacity for current and future needs, leveraging Antamina's mineral resource and applying advanced technology to increase TSF (Tailings Storage Facility) capacity to ensure the mine's long-term operational continuity.

•	Strategy 2: Optimized footprint: The second strategy seeks to optimize the overall project footprint by balancing waste and tailings storage space. This approach aligns directly with commingling principles by maximizing efficient use of available land within the current operational footprint.

•	Strategy 3: Reduce upfront CAPEX: The third strategy focuses on reducing initial CAPEX to improve key financial indicators (IRR), which directly aligns with the demonstrated economic benefits of commingling, minimizing operational costs with a nearby TSF and optimizing CAPEX timing.

Figure 2 Strategic vision of mine planning approach with commingling

The strategies implemented in tailings dam and waste dump management seek to maximize the operational life of the mine and optimize asset value. These decisions translate into concrete actions that address sustainability, operational efficiency, and cost reduction, ensuring continuity and long-term success of the mining operation.

•	Long Life Asset: Antamina's vision as a long-life asset involves making decisions based on sustainable exploitation of its exceptional mineral resources. This includes adopting advanced technologies, such as commingling, to improve storage capacity and ensure operational continuity until 2036 and beyond, thus maximizing the value of the mineral resource.

•	Reduce Environmental Social Footprint: The strategic approach prioritizes reducing environmental and social impact through operation within the current footprint, increasing water availability through filtration, optimizing space use, and minimizing risks associated with tailings dams. Commingling contributes not only to technical and economic efficiency but also to sustainability objectives and corporate social responsibility.

•	Reduce Capital and Operational Cost: The cost reduction strategy focuses on optimizing capital investments (CAPEX) and their schedule, as well as minimizing operational costs through strategic facility location and leveraging synergies to avoid duplicate infrastructure. This enables more efficient and profitable resource management.

During the strategic planning phase, conceptual foundations for commingling implementation are established, including site characterization, regulatory framework development, and technical-economic feasibility analysis.

13	ANALYSIS OF THE MINING SYSTEM WITH COMMINGLING APPLICATION

13.1	Methodological Process Structure

The methodological process is organized in sequential stages that ensure effective integration of commingling into mine planning:

•	Comprehensive material evaluation: Detailed analysis of geotechnical and geochemical characteristics of available materials, identifying their suitability for mixtures and joint disposal.

•	Mixture design development: Design of optimized mixtures that maximize desired properties of commingled material, such as low permeability and high stability, minimizing acid water generation and improving mechanical behavior of the deposit.

•	Pilot test implementation: Execution of pilot and industrial scale tests to validate mixture performance and adjust design parameters according to results obtained under real operating conditions.

•	Monitoring and adjustment: Establishment of continuous monitoring system to evaluate consolidation, stability, and geochemical behavior of deposits, allowing operational and design adjustments as new information is collected.

14	IMPLEMENTATION AND EVALUATION METHODOLOGY FOR COMMINGLING IN MINE PLANNING

14.1	Comparative Case Analysis

The methodology applied at Antamina is based on Comparative Case Analysis, allowing for comprehensive review of the design and sequencing process of mining infrastructure, with emphasis on dams, waste rock dumps, and integration of commingling technologies. This methodological approach comprises:

•	Review of design and sequencing processes: Analysis of procedures used in waste dump planning and disposal, considering both geotechnical and geochemical criteria to ensure infrastructure stability and sustainability.

•	Identification of key variables: Selection of fundamental operational parameters, such as granulometry, permeability, geochemical behavior, and storage capacity, which directly affect the viability and performance of dumps and material mixtures.

•	Definition of case tree: Structure of a decision tree that contemplates long-term strategic scenarios, allowing comparison of alternatives under different combinations of variables and operational constraints.

•	Results evaluation: Application of sensitivity analyses and cost-benefit evaluations to determine the impact of each alternative on mine life extension, environmental risk reduction, and resource optimization.

14.2	Decision Tree for Strategic Planning

The decision tree developed for long-term strategic planning at Antamina follows sequential logic that allows evaluation of feasibility and impact of main operational alternatives:

•	Commingling feasibility: The first node of the tree determines whether integration of tailings and waste rock is technically and economically viable, considering material characteristics and regulatory constraints.

•	Entry into new area: If commingling is feasible, the next node evaluates the possibility of disposing material in a new area, analyzing topographic, environmental, and access aspects.

•	Need for new dam: Finally, it determines whether construction of a new tailings dam is essential or if the commingling solution allows dispensing with this infrastructure, optimizing investment and reducing environmental impact.

Each of these decisions conditions mine life extension, efficient space use, and environmental risk management, allowing selection of the alternative that maximizes economic, social, and environmental value of the project.

Figure 3  Decision Tree for Long-Term Strategic Scenario Planning

14.3	Evaluation of Operational Parameters and Modifications by Commingling

Key operational parameters—including waste rock-tailings ratio, mixing methods, transport systems, and processing criteria—are assessed for their impact on commingled system efficiency. The evaluation considers site-specific challenges such as extreme Andean weather, high-altitude logistics, and Peruvian regulations, all of which are especially critical at Antamina and require customized commingling solutions.

•	Modifications to tailings management by commingling:

The commingling process fundamentally transforms tailings management by utilizing voids within waste rock for storage, with available space typically equivalent to about 20% of the crushed rock volume transported by conveyors. To enable mixing, tailings must first be dewatered. The following image shows both the current process (without mechanization) and the transformation that occurs when commingling is incorporated.

Figure 4 Transformation of Tailings Management: Evolution from Conventional Process to Commingling|

•	 Operational parameters and assumptions

Commingling extends tailings facility life by optimizing spatial use of waste rock voids. Implementation requires mechanized systems (WCCS) for transporting crushed material mixed with dewatered tailings, while maintaining two key operational parameters: a) Unchanged transport capacity: 83.2 Mtpa in the conveyor system and b) Volumetric stability: No increase in total volume of waste rock dump.

This approach ensures efficient resource utilization and supports sustainable mine operations.

Table 1: Key operational parameters and assumptions

Component	Units	Without - Case Conventional	With - Case Commingling

Cycloned/Filtered Max. Capacity	Mtpa	-	32.0

Percentage of Voids in Waste Rock	% Volume 	 	20 %

Waste Rock / Tailings Ratio 	Mt Rock/

Mt Tails	-	83 / 16

Conventional Tailings Density	t/m3 	1.81	1.81

Fines Tailings Density	 t/m3	NA	1.60

Tailings Volume, TDR 4195 (to Dic-22)	Mm3	376	376

Tailings Capacity, TDR 4195 (to Dic-22)	Mt	682	652

Mill Feed Rate 	Ktpd	145	145

The following image shows how incorporation of mechanized systems and commingling add capacity, and therefore extend tailings facility life, to increase the life of mine and therefore the overall asset value.

Figure 5 Commingling Extension of Tailings Facility Life through Commingling

The curve below shows the relationship between dry density and optimal mixing proportion, determinant for maximizing commingling efficiency.

Figure 6: Dry density curve vs. optimal mixing proportion for commingling

•	Analysis of Mineable Resources, Production and Value

A three-dimensional diagram is used to strategically analyze the interactions between mineable resource volume, production rate, and generated value—whether economic, environmental, or operational. This tool helps identify how increasing resource volume and production can maximize value, but also highlights key constraints such as tailings dam capacity and property limits. The approach supports comprehensive decision-making to optimize asset value and ensure project sustainability across various operational scenarios.

Figure 7 Value Optimization: Interaction between Resource Volume, Production Rate and Key Restrictions

15	ECONOMIC AND OPERATIONAL ADVANTAGES

15.1	Perspective of Conventional Value Drivers

The following figure illustrates Antamina's main conventional value drivers from a financial perspective. In the current scenario, the amount of mineral resource that can be utilized directly depends on the capital that the company decides to invest (Capex), both in new investments (Growth Capex) and in maintaining existing operations (Sustaining Capex). This generates a necessary balance to determine the optimal scenario that maximizes the value of the existing asset.

Figure 8: Conventional Antamina Value Drivers: Financial Perspective (Without Commingling)

15.2	Impact of Commingling on Financial Value Drivers

From a financial perspective, the adoption of commingling among Antamina’s value drivers removes constraints on mineral resources by increasing tailings storage capacity, enabling the utilization of new dumps, and allowing for an expansion in the volume of usable resources. As a result, the mine’s operational life is extended and asset value is enhanced. Regarding dump utilization, the implementation of commingling has the potential to reduce the requirement for dumps located in karstic zones, which demand significant capital expenditure (Capex). This can lead to a reduction in initial investment by up to 30% in scenarios comparing commingling versus non-commingling approaches. This saving is achieved through the integration of previously separate infrastructures into a single system, which optimizes the use of available area in the East Extension dumps and prioritizes the use of mechanized systems over conventional trucking methods.

Figure 9: Antamina Value Drivers: Financial Perspective with Commingling

The optimization of land use represents another important economic benefit, especially relevant in mountainous locations like Antamina where available land for mining facilities is limited. Commingling allows for the creation of deposits with a smaller footprint compared to separate facilities, freeing up land for other productive uses or reducing the need for additional land acquisition.

16	ALIGNMENT WITH GISTM STANDARDS AND SUSTAINABILITY

The integration of commingling into mine planning strengthens compliance with the Global Industry Standard on Tailings Management (GISTM), by facilitating safer and more sustainable management of waste materials. The GISTM requires the planning, construction, operation, and closure of tailings facilities with a focus on risk reduction and continuous monitoring throughout the lifecycle. Commingling contributes to this objective by improving the geotechnical and geochemical stability of deposits, reducing the risk of failure and acid drainage generation, which in turn reduces environmental impact and long-term treatment costs. Additionally, the consolidation of facilities through commingling reduces the environmental footprint and facilitates supervision and control, aligning with the principles of sustainability and social responsibility in the mining industry.

17	LESSONS LEARNED AND CHALLENGES

Among the main lessons learned and challenges of implementing commingling in mine planning at Antamina, the following stand out:

•	Large-scale operations and mechanization: Commingling is primarily viable in large-volume mechanized operations, where it is possible to achieve a homogeneous and controlled mixture of materials. However, the magnitude of waste rock and tailings tonnages at Antamina implies a new challenge, never seen before.

•	Flexible planning and pilot tests: It is essential to include areas for pilot tests in short and medium-term plans, allowing adjustment of mixing parameters according to material variability.

•	Regulatory communication: Technology acceptance requires identifying precedents and working closely with authorities to develop appropriate regulatory frameworks.

•	Organizational change management: The transition to commingling involves challenges in personnel training and adaptation of operational processes, especially in mature operations.

18	CONCLUSIONS

•	The comparative discussion and case analysis of commingling implementation at Antamina have enabled the identification and selection of strategic alternatives that maximize the economic, environmental, and social value of the project.

•	The use of decision trees and scenario evaluation has facilitated informed decision-making, taking into account technical and economic feasibility as well as regulatory and environmental constraints.

•	The integration of commingling into strategic mine planning has proven to be a key tool for optimizing waste rock and tailings management, achieving a significant reduction in capital costs (up to 30%), greater efficiency in land use, and an extension of tailings storage facility (TSF) life. This approach has also aligned operations with international sustainability standards (GISTM) and strengthened risk management, contributing to operational continuity and long-term reduction of environmental impacts.

•	The Antamina experience demonstrates that structured case discussions and the application of comparative methodologies are fundamental for strategic planning, as they allow anticipation of challenges, validation of solutions through pilot testing, and adjustment of operational parameters based on real outcomes. Thus, commingling is consolidated as a transformative practice in modern mining, fostering more robust and sustainable decision-making for the development of long-life mining assets.

ACKNOWLEDGEMENTS

The authors express their sincere gratitude to Antamina for their invaluable support. Special thanks are extended to the Tailings Strategy Management team and the Long-Term Planning team for their collaboration and insights.

REFERENCES

Boshoff, J. et al. (2023). A case study on the commingling of tailings and waste rock at a Brownfields open cast mine in Ghana. University of Alberta.

Burden, R., & Wilson, G. W. (2023). Commingling of waste rock and tailings to improve “dry stack” performance: Design and evaluation of mixtures. Minerals, 13(2), 295.

Deza, N., & Montes, K. (2022). Mecanización del desmonte en Compañía Minera Antamina: Propuesta para capturar el valor tangible e intangible de la mecanización. Compañía Minera Antamina.

Green Policy Platform. (2024). Knowledge Gaps Report: Environmental Aspects of Tailings Management (January 2024).

ICMM. (2020). New global industry standard on tailings management. International Council on Mining and Metals.

Ulrich, B. & Coffin, J. (2015). TMW 2015 – Combined Tailings and Mine Waste.

Minemax. (2015). Why mine planning is all about collaboration. Minemax News.

Wheaton Precious Metals. (2015). Antamina Project overview.

INTEGRATING COMMINGLING INTO THE STRATEGIC WASTE ROCK PLANNING AT ANTAMINA MINE

Christa Quiroz1, David Machin1, Olimpia Cabrera1, Fernando Angeles1

1 Antamina Mine, Lima, Peru

(*Presenting author: cquirozc@antamina.com)

ABSTRACT

Antamina is a large open-pit polymetallic mine in Peru, operating with a processing capacity of 145 ktpd. In operation since 2001, the current mine life extends to 2036. Mining is conducted at a rate of 290 Mtpa using conventional truck-and-shovel equipment. This technical paper presents a comprehensive assessment of commingling as a transformative technology in mine planning, positioning waste rock and tailings management as a key factor in the decision-making process. The primary objective of this study is to demonstrate the benefits of this technology within the mine planning process, highlighting waste rock and tailings management as one of the main drivers in strategic decision-making. The methodology applied is a Comparative Case Analysis, which includes a review of the design and sequencing process for mining infrastructure such as waste rock storage facilities, along with a discussion of the key parameters considered. The results show that the implementation of this technology delivers significant economic benefits, including an approximate 30% reduction in capital costs for waste rock storage facilities, improved land-use efficiency, and optimized infrastructure development. Furthermore, the approach enhances alignment with the Global Industry Standard on Tailings Management (GISTM).

KEYWORDS

commingling, mine planning, waste rock management, tailings management, GISTM compliance

1	INTRODUCTION

The mining industry faces growing challenges related to the sustainable management of waste rock and tailings, especially after catastrophic events such as those that occurred in Brumadinho and Samarco (Green Policy Platform, 2024; ICMM, 2020). In this context, the Antamina mine, operating since 2001 as one of the largest polymetallic operations in the world, has developed a pioneering approach that integrates commingling as a central element of its strategic planning (Wheaton Precious Metals, 2015).

Commingling, defined as the homogeneous mixing of dewatered tailings with waste rock to form a geotechnically stable material, represents a natural evolution of traditional co-disposal practices (Burden & Wilson, 2023). However, what distinguishes the Antamina case is the integration of this technology from the initial phases of mine planning, making waste rock management the main driver of operational and strategic decisions.

2	OBJECTIVES

•	Demonstrate the benefits of integrating commingling into mine planning, positioning waste rock and tailings management as a strategic driver for decision-making in open-pit operations.

•	Optimize operational efficiency and environmental sustainability through the adoption of commingling in waste rock and tailings management.

•	Align waste rock and tailings management with international sustainability and safety standards, such as the GISTM, through the application of commingling.

•	Analyze, through comparative case studies, the impact of commingling on land-use optimization and mine life extension.

•	Reduce capital and operating costs associated with waste rock and tailings disposal by implementing commingling technologies.

•	Present the key findings and lessons learned from the integration of commingling into long-term mine planning.

3	TECHNICAL FOUNDATIONS OF COMMINGLING IN MINE PLANNING

3.1	Desing Principles and Material Characterization

Commingling at Antamina is based on engineering principles that seek to combine the superior structural properties of waste rock with the low permeability characteristics of tailings (Boshoff, 2023). This combination results in a material with shear strength similar to waste rock and permeability comparable to tailings, creating conditions that restrict oxygen entry and water filtration, significantly reducing the potential for acid drainage generation (Ulrich & Coffin, 2015; Burden & Wilson, 2023).

The characterization of commingled material is performed through analysis of five distinct structural configurations: rock-dominated uncompacted, fines-dominated uncompacted, rock-dominated compacted, fines-dominated compacted, and the "just-filled" configuration where fines exactly fill the voids between rock particles (Burden & Wilson, 2023). This latter configuration represents the optimal target of the commingling process at Antamina.

Figure 1 Blend configurations after Wickland et al. (2006). Source: Burden & Wilson (2023).

3.2	Integration into the Strategic Planning Process

The integration of commingling into Antamina's mine planning follows a structured methodological approach that places tailings management with waste rock as central elements of decision-making. This global strategy allows for optimizing the mixture of materials from the mine to the plant, considering the operational and environmental constraints associated with the tailings storage facility. By incorporating tailings behavior in the early stages of planning, greater efficiency in resource use and better risk management are achieved.

The global strategy is divided into three key axes:

•	Strategy 1: Ensure Future Expansions: The first strategy focuses on maximizing tailings storage capacity for current and future needs, leveraging Antamina's mineral resource and applying advanced technology to increase TSF (Tailings Storage Facility) capacity to ensure the mine's long-term operational continuity.

•	Strategy 2: Optimized footprint: The second strategy seeks to optimize the overall project footprint by balancing waste and tailings storage space. This approach aligns directly with commingling principles by maximizing efficient use of available land within the current operational footprint.

•	Strategy 3: Reduce upfront CAPEX: The third strategy focuses on reducing initial CAPEX to improve key financial indicators (IRR), which directly aligns with the demonstrated economic benefits of commingling, minimizing operational costs with a nearby TSF and optimizing CAPEX timing.

Figure 2 Strategic vision of mine planning approach with commingling

The strategies implemented in tailings dam and waste dump management seek to maximize the operational life of the mine and optimize asset value. These decisions translate into concrete actions that address sustainability, operational efficiency, and cost reduction, ensuring continuity and long-term success of the mining operation.

•	Long Life Asset: Antamina's vision as a long-life asset involves making decisions based on sustainable exploitation of its exceptional mineral resources. This includes adopting advanced technologies, such as commingling, to improve storage capacity and ensure operational continuity until 2036 and beyond, thus maximizing the value of the mineral resource.

•	Reduce Environmental Social Footprint: The strategic approach prioritizes reducing environmental and social impact through operation within the current footprint, increasing water availability through filtration, optimizing space use, and minimizing risks associated with tailings dams. Commingling contributes not only to technical and economic efficiency but also to sustainability objectives and corporate social responsibility.

•	Reduce Capital and Operational Cost: The cost reduction strategy focuses on optimizing capital investments (CAPEX) and their schedule, as well as minimizing operational costs through strategic facility location and leveraging synergies to avoid duplicate infrastructure. This enables more efficient and profitable resource management.

During the strategic planning phase, conceptual foundations for commingling implementation are established, including site characterization, regulatory framework development, and technical-economic feasibility analysis.

4	ANALYSIS OF THE MINING SYSTEM WITH COMMINGLING APPLICATION

4.1	Methodological Process Structure

The methodological process is organized in sequential stages that ensure effective integration of commingling into mine planning:

•	Comprehensive material evaluation: Detailed analysis of geotechnical and geochemical characteristics of available materials, identifying their suitability for mixtures and joint disposal.

•	Mixture design development: Design of optimized mixtures that maximize desired properties of commingled material, such as low permeability and high stability, minimizing acid water generation and improving mechanical behavior of the deposit.

•	Pilot test implementation: Execution of pilot and industrial scale tests to validate mixture performance and adjust design parameters according to results obtained under real operating conditions.

•	Monitoring and adjustment: Establishment of continuous monitoring system to evaluate consolidation, stability, and geochemical behavior of deposits, allowing operational and design adjustments as new information is collected.

5	IMPLEMENTATION AND EVALUATION METHODOLOGY FOR COMMINGLING IN MINE PLANNING

5.1	Comparative Case Analysis

The methodology applied at Antamina is based on Comparative Case Analysis, allowing for comprehensive review of the design and sequencing process of mining infrastructure, with emphasis on dams, waste rock dumps, and integration of commingling technologies. This methodological approach comprises:

•	Review of design and sequencing processes: Analysis of procedures used in waste dump planning and disposal, considering both geotechnical and geochemical criteria to ensure infrastructure stability and sustainability.

•	Identification of key variables: Selection of fundamental operational parameters, such as granulometry, permeability, geochemical behavior, and storage capacity, which directly affect the viability and performance of dumps and material mixtures.

•	Definition of case tree: Structure of a decision tree that contemplates long-term strategic scenarios, allowing comparison of alternatives under different combinations of variables and operational constraints.

•	Results evaluation: Application of sensitivity analyses and cost-benefit evaluations to determine the impact of each alternative on mine life extension, environmental risk reduction, and resource optimization.

5.2	Decision Tree for Strategic Planning

The decision tree developed for long-term strategic planning at Antamina follows sequential logic that allows evaluation of feasibility and impact of main operational alternatives:

•	Commingling feasibility: The first node of the tree determines whether integration of tailings and waste rock is technically and economically viable, considering material characteristics and regulatory constraints.

•	Entry into new area: If commingling is feasible, the next node evaluates the possibility of disposing material in a new area, analyzing topographic, environmental, and access aspects.

•	Need for new dam: Finally, it determines whether construction of a new tailings dam is essential or if the commingling solution allows dispensing with this infrastructure, optimizing investment and reducing environmental impact.

Each of these decisions conditions mine life extension, efficient space use, and environmental risk management, allowing selection of the alternative that maximizes economic, social, and environmental value of the project.

Figure 3  Decision Tree for Long-Term Strategic Scenario Planning

5.3	Evaluation of Operational Parameters and Modifications by Commingling

Key operational parameters—including waste rock-tailings ratio, mixing methods, transport systems, and processing criteria—are assessed for their impact on commingled system efficiency. The evaluation considers site-specific challenges such as extreme Andean weather, high-altitude logistics, and Peruvian regulations, all of which are especially critical at Antamina and require customized commingling solutions.

•	Modifications to tailings management by commingling:

The commingling process fundamentally transforms tailings management by utilizing voids within waste rock for storage, with available space typically equivalent to about 20% of the crushed rock volume transported by conveyors. To enable mixing, tailings must first be dewatered. The following image shows both the current process (without mechanization) and the transformation that occurs when commingling is incorporated.

Figure 4 Transformation of Tailings Management: Evolution from Conventional Process to Commingling|

•	 Operational parameters and assumptions

Commingling extends tailings facility life by optimizing spatial use of waste rock voids. Implementation requires mechanized systems (WCCS) for transporting crushed material mixed with dewatered tailings, while maintaining two key operational parameters: a) Unchanged transport capacity: 83.2 Mtpa in the conveyor system and b) Volumetric stability: No increase in total volume of waste rock dump.

This approach ensures efficient resource utilization and supports sustainable mine operations.

Table 1: Key operational parameters and assumptions

Component	Units	Without - Case Conventional	With - Case Commingling

Cycloned/Filtered Max. Capacity	Mtpa	-	32.0

Percentage of Voids in Waste Rock	% Volume 	 	20 %

Waste Rock / Tailings Ratio 	Mt Rock/

Mt Tails	-	83 / 16

Conventional Tailings Density	t/m3 	1.81	1.81

Fines Tailings Density	 t/m3	NA	1.60

Tailings Volume, TDR 4195 (to Dic-22)	Mm3	376	376

Tailings Capacity, TDR 4195 (to Dic-22)	Mt	682	652

Mill Feed Rate 	Ktpd	145	145

The following image shows how incorporation of mechanized systems and commingling add capacity, and therefore extend tailings facility life, to increase the life of mine and therefore the overall asset value.

Figure 5 Commingling Extension of Tailings Facility Life through Commingling

The curve below shows the relationship between dry density and optimal mixing proportion, determinant for maximizing commingling efficiency.

Figure 6: Dry density curve vs. optimal mixing proportion for commingling

•	Analysis of Mineable Resources, Production and Value

A three-dimensional diagram is used to strategically analyze the interactions between mineable resource volume, production rate, and generated value—whether economic, environmental, or operational. This tool helps identify how increasing resource volume and production can maximize value, but also highlights key constraints such as tailings dam capacity and property limits. The approach supports comprehensive decision-making to optimize asset value and ensure project sustainability across various operational scenarios.

Figure 7 Value Optimization: Interaction between Resource Volume, Production Rate and Key Restrictions

6	ECONOMIC AND OPERATIONAL ADVANTAGES

6.1	Perspective of Conventional Value Drivers

The following figure illustrates Antamina's main conventional value drivers from a financial perspective. In the current scenario, the amount of mineral resource that can be utilized directly depends on the capital that the company decides to invest (Capex), both in new investments (Growth Capex) and in maintaining existing operations (Sustaining Capex). This generates a necessary balance to determine the optimal scenario that maximizes the value of the existing asset.

Figure 8: Conventional Antamina Value Drivers: Financial Perspective (Without Commingling)

6.2	Impact of Commingling on Financial Value Drivers

From a financial perspective, the adoption of commingling among Antamina’s value drivers removes constraints on mineral resources by increasing tailings storage capacity, enabling the utilization of new dumps, and allowing for an expansion in the volume of usable resources. As a result, the mine’s operational life is extended and asset value is enhanced. Regarding dump utilization, the implementation of commingling has the potential to reduce the requirement for dumps located in karstic zones, which demand significant capital expenditure (Capex). This can lead to a reduction in initial investment by up to 30% in scenarios comparing commingling versus non-commingling approaches. This saving is achieved through the integration of previously separate infrastructures into a single system, which optimizes the use of available area in the East Extension dumps and prioritizes the use of mechanized systems over conventional trucking methods.

Figure 9: Antamina Value Drivers: Financial Perspective with Commingling

The optimization of land use represents another important economic benefit, especially relevant in mountainous locations like Antamina where available land for mining facilities is limited. Commingling allows for the creation of deposits with a smaller footprint compared to separate facilities, freeing up land for other productive uses or reducing the need for additional land acquisition.

7	ALIGNMENT WITH GISTM STANDARDS AND SUSTAINABILITY

The integration of commingling into mine planning strengthens compliance with the Global Industry Standard on Tailings Management (GISTM), by facilitating safer and more sustainable management of waste materials. The GISTM requires the planning, construction, operation, and closure of tailings facilities with a focus on risk reduction and continuous monitoring throughout the lifecycle. Commingling contributes to this objective by improving the geotechnical and geochemical stability of deposits, reducing the risk of failure and acid drainage generation, which in turn reduces environmental impact and long-term treatment costs. Additionally, the consolidation of facilities through commingling reduces the environmental footprint and facilitates supervision and control, aligning with the principles of sustainability and social responsibility in the mining industry.

8	LESSONS LEARNED AND CHALLENGES

Among the main lessons learned and challenges of implementing commingling in mine planning at Antamina, the following stand out:

•	Large-scale operations and mechanization: Commingling is primarily viable in large-volume mechanized operations, where it is possible to achieve a homogeneous and controlled mixture of materials. However, the magnitude of waste rock and tailings tonnages at Antamina implies a new challenge, never seen before.

•	Flexible planning and pilot tests: It is essential to include areas for pilot tests in short and medium-term plans, allowing adjustment of mixing parameters according to material variability.

•	Regulatory communication: Technology acceptance requires identifying precedents and working closely with authorities to develop appropriate regulatory frameworks.

•	Organizational change management: The transition to commingling involves challenges in personnel training and adaptation of operational processes, especially in mature operations.

9	CONCLUSIONS

•	The comparative discussion and case analysis of commingling implementation at Antamina have enabled the identification and selection of strategic alternatives that maximize the economic, environmental, and social value of the project.

•	The use of decision trees and scenario evaluation has facilitated informed decision-making, taking into account technical and economic feasibility as well as regulatory and environmental constraints.

•	The integration of commingling into strategic mine planning has proven to be a key tool for optimizing waste rock and tailings management, achieving a significant reduction in capital costs (up to 30%), greater efficiency in land use, and an extension of tailings storage facility (TSF) life. This approach has also aligned operations with international sustainability standards (GISTM) and strengthened risk management, contributing to operational continuity and long-term reduction of environmental impacts.

•	The Antamina experience demonstrates that structured case discussions and the application of comparative methodologies are fundamental for strategic planning, as they allow anticipation of challenges, validation of solutions through pilot testing, and adjustment of operational parameters based on real outcomes. Thus, commingling is consolidated as a transformative practice in modern mining, fostering more robust and sustainable decision-making for the development of long-life mining assets.

ACKNOWLEDGEMENTS

The authors express their sincere gratitude to Antamina for their invaluable support. Special thanks are extended to the Tailings Strategy Management team and the Long-Term Planning team for their collaboration and insights.

REFERENCES

Boshoff, J. et al. (2023). A case study on the commingling of tailings and waste rock at a Brownfields open cast mine in Ghana. University of Alberta.

Burden, R., & Wilson, G. W. (2023). Commingling of waste rock and tailings to improve “dry stack” performance: Design and evaluation of mixtures. Minerals, 13(2), 295.

Deza, N., & Montes, K. (2022). Mecanización del desmonte en Compañía Minera Antamina: Propuesta para capturar el valor tangible e intangible de la mecanización. Compañía Minera Antamina.

Green Policy Platform. (2024). Knowledge Gaps Report: Environmental Aspects of Tailings Management (January 2024).

ICMM. (2020). New global industry standard on tailings management. International Council on Mining and Metals.

Ulrich, B. & Coffin, J. (2015). TMW 2015 – Combined Tailings and Mine Waste.

Minemax. (2015). Why mine planning is all about collaboration. Minemax News.

Wheaton Precious Metals. (2015). Antamina Project overview.

INTEGRATING COMMINGLING INTO THE STRATEGIC WASTE ROCK PLANNING AT ANTAMINA MINE

Christa Quiroz1, David Machin1, Olimpia Cabrera1, Fernando Angeles1

1 Antamina Mine, Lima, Peru

(*Presenting author: cquirozc@antamina.com)

ABSTRACT

Antamina is a large open-pit polymetallic mine in Peru, operating with a processing capacity of 145 ktpd. In operation since 2001, the current mine life extends to 2036. Mining is conducted at a rate of 290 Mtpa using conventional truck-and-shovel equipment. This technical paper presents a comprehensive assessment of commingling as a transformative technology in mine planning, positioning waste rock and tailings management as a key factor in the decision-making process. The primary objective of this study is to demonstrate the benefits of this technology within the mine planning process, highlighting waste rock and tailings management as one of the main drivers in strategic decision-making. The methodology applied is a Comparative Case Analysis, which includes a review of the design and sequencing process for mining infrastructure such as waste rock storage facilities, along with a discussion of the key parameters considered. The results show that the implementation of this technology delivers significant economic benefits, including an approximate 30% reduction in capital costs for waste rock storage facilities, improved land-use efficiency, and optimized infrastructure development. Furthermore, the approach enhances alignment with the Global Industry Standard on Tailings Management (GISTM).

KEYWORDS

commingling, mine planning, waste rock management, tailings management, GISTM compliance

10	INTRODUCTION

The mining industry faces growing challenges related to the sustainable management of waste rock and tailings, especially after catastrophic events such as those that occurred in Brumadinho and Samarco (Green Policy Platform, 2024; ICMM, 2020). In this context, the Antamina mine, operating since 2001 as one of the largest polymetallic operations in the world, has developed a pioneering approach that integrates commingling as a central element of its strategic planning (Wheaton Precious Metals, 2015).

Commingling, defined as the homogeneous mixing of dewatered tailings with waste rock to form a geotechnically stable material, represents a natural evolution of traditional co-disposal practices (Burden & Wilson, 2023). However, what distinguishes the Antamina case is the integration of this technology from the initial phases of mine planning, making waste rock management the main driver of operational and strategic decisions.

11	OBJECTIVES

•	Demonstrate the benefits of integrating commingling into mine planning, positioning waste rock and tailings management as a strategic driver for decision-making in open-pit operations.

•	Optimize operational efficiency and environmental sustainability through the adoption of commingling in waste rock and tailings management.

•	Align waste rock and tailings management with international sustainability and safety standards, such as the GISTM, through the application of commingling.

•	Analyze, through comparative case studies, the impact of commingling on land-use optimization and mine life extension.

•	Reduce capital and operating costs associated with waste rock and tailings disposal by implementing commingling technologies.

•	Present the key findings and lessons learned from the integration of commingling into long-term mine planning.

12	TECHNICAL FOUNDATIONS OF COMMINGLING IN MINE PLANNING

12.1	Desing Principles and Material Characterization

Commingling at Antamina is based on engineering principles that seek to combine the superior structural properties of waste rock with the low permeability characteristics of tailings (Boshoff, 2023). This combination results in a material with shear strength similar to waste rock and permeability comparable to tailings, creating conditions that restrict oxygen entry and water filtration, significantly reducing the potential for acid drainage generation (Ulrich & Coffin, 2015; Burden & Wilson, 2023).

The characterization of commingled material is performed through analysis of five distinct structural configurations: rock-dominated uncompacted, fines-dominated uncompacted, rock-dominated compacted, fines-dominated compacted, and the "just-filled" configuration where fines exactly fill the voids between rock particles (Burden & Wilson, 2023). This latter configuration represents the optimal target of the commingling process at Antamina.

Figure 1 Blend configurations after Wickland et al. (2006). Source: Burden & Wilson (2023).

12.2	Integration into the Strategic Planning Process

The integration of commingling into Antamina's mine planning follows a structured methodological approach that places tailings management with waste rock as central elements of decision-making. This global strategy allows for optimizing the mixture of materials from the mine to the plant, considering the operational and environmental constraints associated with the tailings storage facility. By incorporating tailings behavior in the early stages of planning, greater efficiency in resource use and better risk management are achieved.

The global strategy is divided into three key axes:

•	Strategy 1: Ensure Future Expansions: The first strategy focuses on maximizing tailings storage capacity for current and future needs, leveraging Antamina's mineral resource and applying advanced technology to increase TSF (Tailings Storage Facility) capacity to ensure the mine's long-term operational continuity.

•	Strategy 2: Optimized footprint: The second strategy seeks to optimize the overall project footprint by balancing waste and tailings storage space. This approach aligns directly with commingling principles by maximizing efficient use of available land within the current operational footprint.

•	Strategy 3: Reduce upfront CAPEX: The third strategy focuses on reducing initial CAPEX to improve key financial indicators (IRR), which directly aligns with the demonstrated economic benefits of commingling, minimizing operational costs with a nearby TSF and optimizing CAPEX timing.

Figure 2 Strategic vision of mine planning approach with commingling

The strategies implemented in tailings dam and waste dump management seek to maximize the operational life of the mine and optimize asset value. These decisions translate into concrete actions that address sustainability, operational efficiency, and cost reduction, ensuring continuity and long-term success of the mining operation.

•	Long Life Asset: Antamina's vision as a long-life asset involves making decisions based on sustainable exploitation of its exceptional mineral resources. This includes adopting advanced technologies, such as commingling, to improve storage capacity and ensure operational continuity until 2036 and beyond, thus maximizing the value of the mineral resource.

•	Reduce Environmental Social Footprint: The strategic approach prioritizes reducing environmental and social impact through operation within the current footprint, increasing water availability through filtration, optimizing space use, and minimizing risks associated with tailings dams. Commingling contributes not only to technical and economic efficiency but also to sustainability objectives and corporate social responsibility.

•	Reduce Capital and Operational Cost: The cost reduction strategy focuses on optimizing capital investments (CAPEX) and their schedule, as well as minimizing operational costs through strategic facility location and leveraging synergies to avoid duplicate infrastructure. This enables more efficient and profitable resource management.

During the strategic planning phase, conceptual foundations for commingling implementation are established, including site characterization, regulatory framework development, and technical-economic feasibility analysis.

13	ANALYSIS OF THE MINING SYSTEM WITH COMMINGLING APPLICATION

13.1	Methodological Process Structure

The methodological process is organized in sequential stages that ensure effective integration of commingling into mine planning:

•	Comprehensive material evaluation: Detailed analysis of geotechnical and geochemical characteristics of available materials, identifying their suitability for mixtures and joint disposal.

•	Mixture design development: Design of optimized mixtures that maximize desired properties of commingled material, such as low permeability and high stability, minimizing acid water generation and improving mechanical behavior of the deposit.

•	Pilot test implementation: Execution of pilot and industrial scale tests to validate mixture performance and adjust design parameters according to results obtained under real operating conditions.

•	Monitoring and adjustment: Establishment of continuous monitoring system to evaluate consolidation, stability, and geochemical behavior of deposits, allowing operational and design adjustments as new information is collected.

14	IMPLEMENTATION AND EVALUATION METHODOLOGY FOR COMMINGLING IN MINE PLANNING

14.1	Comparative Case Analysis

The methodology applied at Antamina is based on Comparative Case Analysis, allowing for comprehensive review of the design and sequencing process of mining infrastructure, with emphasis on dams, waste rock dumps, and integration of commingling technologies. This methodological approach comprises:

•	Review of design and sequencing processes: Analysis of procedures used in waste dump planning and disposal, considering both geotechnical and geochemical criteria to ensure infrastructure stability and sustainability.

•	Identification of key variables: Selection of fundamental operational parameters, such as granulometry, permeability, geochemical behavior, and storage capacity, which directly affect the viability and performance of dumps and material mixtures.

•	Definition of case tree: Structure of a decision tree that contemplates long-term strategic scenarios, allowing comparison of alternatives under different combinations of variables and operational constraints.

•	Results evaluation: Application of sensitivity analyses and cost-benefit evaluations to determine the impact of each alternative on mine life extension, environmental risk reduction, and resource optimization.

14.2	Decision Tree for Strategic Planning

The decision tree developed for long-term strategic planning at Antamina follows sequential logic that allows evaluation of feasibility and impact of main operational alternatives:

•	Commingling feasibility: The first node of the tree determines whether integration of tailings and waste rock is technically and economically viable, considering material characteristics and regulatory constraints.

•	Entry into new area: If commingling is feasible, the next node evaluates the possibility of disposing material in a new area, analyzing topographic, environmental, and access aspects.

•	Need for new dam: Finally, it determines whether construction of a new tailings dam is essential or if the commingling solution allows dispensing with this infrastructure, optimizing investment and reducing environmental impact.

Each of these decisions conditions mine life extension, efficient space use, and environmental risk management, allowing selection of the alternative that maximizes economic, social, and environmental value of the project.

Figure 3  Decision Tree for Long-Term Strategic Scenario Planning

14.3	Evaluation of Operational Parameters and Modifications by Commingling

Key operational parameters—including waste rock-tailings ratio, mixing methods, transport systems, and processing criteria—are assessed for their impact on commingled system efficiency. The evaluation considers site-specific challenges such as extreme Andean weather, high-altitude logistics, and Peruvian regulations, all of which are especially critical at Antamina and require customized commingling solutions.

•	Modifications to tailings management by commingling:

The commingling process fundamentally transforms tailings management by utilizing voids within waste rock for storage, with available space typically equivalent to about 20% of the crushed rock volume transported by conveyors. To enable mixing, tailings must first be dewatered. The following image shows both the current process (without mechanization) and the transformation that occurs when commingling is incorporated.

Figure 4 Transformation of Tailings Management: Evolution from Conventional Process to Commingling|

•	 Operational parameters and assumptions

Commingling extends tailings facility life by optimizing spatial use of waste rock voids. Implementation requires mechanized systems (WCCS) for transporting crushed material mixed with dewatered tailings, while maintaining two key operational parameters: a) Unchanged transport capacity: 83.2 Mtpa in the conveyor system and b) Volumetric stability: No increase in total volume of waste rock dump.

This approach ensures efficient resource utilization and supports sustainable mine operations.

Table 1: Key operational parameters and assumptions

Component	Units	Without - Case Conventional	With - Case Commingling

Cycloned/Filtered Max. Capacity	Mtpa	-	32.0

Percentage of Voids in Waste Rock	% Volume 	 	20 %

Waste Rock / Tailings Ratio 	Mt Rock/

Mt Tails	-	83 / 16

Conventional Tailings Density	t/m3 	1.81	1.81

Fines Tailings Density	 t/m3	NA	1.60

Tailings Volume, TDR 4195 (to Dic-22)	Mm3	376	376

Tailings Capacity, TDR 4195 (to Dic-22)	Mt	682	652

Mill Feed Rate 	Ktpd	145	145

The following image shows how incorporation of mechanized systems and commingling add capacity, and therefore extend tailings facility life, to increase the life of mine and therefore the overall asset value.

Figure 5 Commingling Extension of Tailings Facility Life through Commingling

The curve below shows the relationship between dry density and optimal mixing proportion, determinant for maximizing commingling efficiency.

Figure 6: Dry density curve vs. optimal mixing proportion for commingling

•	Analysis of Mineable Resources, Production and Value

A three-dimensional diagram is used to strategically analyze the interactions between mineable resource volume, production rate, and generated value—whether economic, environmental, or operational. This tool helps identify how increasing resource volume and production can maximize value, but also highlights key constraints such as tailings dam capacity and property limits. The approach supports comprehensive decision-making to optimize asset value and ensure project sustainability across various operational scenarios.

Figure 7 Value Optimization: Interaction between Resource Volume, Production Rate and Key Restrictions

15	ECONOMIC AND OPERATIONAL ADVANTAGES

15.1	Perspective of Conventional Value Drivers

The following figure illustrates Antamina's main conventional value drivers from a financial perspective. In the current scenario, the amount of mineral resource that can be utilized directly depends on the capital that the company decides to invest (Capex), both in new investments (Growth Capex) and in maintaining existing operations (Sustaining Capex). This generates a necessary balance to determine the optimal scenario that maximizes the value of the existing asset.

Figure 8: Conventional Antamina Value Drivers: Financial Perspective (Without Commingling)

15.2	Impact of Commingling on Financial Value Drivers

From a financial perspective, the adoption of commingling among Antamina’s value drivers removes constraints on mineral resources by increasing tailings storage capacity, enabling the utilization of new dumps, and allowing for an expansion in the volume of usable resources. As a result, the mine’s operational life is extended and asset value is enhanced. Regarding dump utilization, the implementation of commingling has the potential to reduce the requirement for dumps located in karstic zones, which demand significant capital expenditure (Capex). This can lead to a reduction in initial investment by up to 30% in scenarios comparing commingling versus non-commingling approaches. This saving is achieved through the integration of previously separate infrastructures into a single system, which optimizes the use of available area in the East Extension dumps and prioritizes the use of mechanized systems over conventional trucking methods.

Figure 9: Antamina Value Drivers: Financial Perspective with Commingling

The optimization of land use represents another important economic benefit, especially relevant in mountainous locations like Antamina where available land for mining facilities is limited. Commingling allows for the creation of deposits with a smaller footprint compared to separate facilities, freeing up land for other productive uses or reducing the need for additional land acquisition.

16	ALIGNMENT WITH GISTM STANDARDS AND SUSTAINABILITY

The integration of commingling into mine planning strengthens compliance with the Global Industry Standard on Tailings Management (GISTM), by facilitating safer and more sustainable management of waste materials. The GISTM requires the planning, construction, operation, and closure of tailings facilities with a focus on risk reduction and continuous monitoring throughout the lifecycle. Commingling contributes to this objective by improving the geotechnical and geochemical stability of deposits, reducing the risk of failure and acid drainage generation, which in turn reduces environmental impact and long-term treatment costs. Additionally, the consolidation of facilities through commingling reduces the environmental footprint and facilitates supervision and control, aligning with the principles of sustainability and social responsibility in the mining industry.

17	LESSONS LEARNED AND CHALLENGES

Among the main lessons learned and challenges of implementing commingling in mine planning at Antamina, the following stand out:

•	Large-scale operations and mechanization: Commingling is primarily viable in large-volume mechanized operations, where it is possible to achieve a homogeneous and controlled mixture of materials. However, the magnitude of waste rock and tailings tonnages at Antamina implies a new challenge, never seen before.

•	Flexible planning and pilot tests: It is essential to include areas for pilot tests in short and medium-term plans, allowing adjustment of mixing parameters according to material variability.

•	Regulatory communication: Technology acceptance requires identifying precedents and working closely with authorities to develop appropriate regulatory frameworks.

•	Organizational change management: The transition to commingling involves challenges in personnel training and adaptation of operational processes, especially in mature operations.

18	CONCLUSIONS

•	The comparative discussion and case analysis of commingling implementation at Antamina have enabled the identification and selection of strategic alternatives that maximize the economic, environmental, and social value of the project.

•	The use of decision trees and scenario evaluation has facilitated informed decision-making, taking into account technical and economic feasibility as well as regulatory and environmental constraints.

•	The integration of commingling into strategic mine planning has proven to be a key tool for optimizing waste rock and tailings management, achieving a significant reduction in capital costs (up to 30%), greater efficiency in land use, and an extension of tailings storage facility (TSF) life. This approach has also aligned operations with international sustainability standards (GISTM) and strengthened risk management, contributing to operational continuity and long-term reduction of environmental impacts.

•	The Antamina experience demonstrates that structured case discussions and the application of comparative methodologies are fundamental for strategic planning, as they allow anticipation of challenges, validation of solutions through pilot testing, and adjustment of operational parameters based on real outcomes. Thus, commingling is consolidated as a transformative practice in modern mining, fostering more robust and sustainable decision-making for the development of long-life mining assets.

ACKNOWLEDGEMENTS

The authors express their sincere gratitude to Antamina for their invaluable support. Special thanks are extended to the Tailings Strategy Management team and the Long-Term Planning team for their collaboration and insights.

REFERENCES

Boshoff, J. et al. (2023). A case study on the commingling of tailings and waste rock at a Brownfields open cast mine in Ghana. University of Alberta.

Burden, R., & Wilson, G. W. (2023). Commingling of waste rock and tailings to improve “dry stack” performance: Design and evaluation of mixtures. Minerals, 13(2), 295.

Deza, N., & Montes, K. (2022). Mecanización del desmonte en Compañía Minera Antamina: Propuesta para capturar el valor tangible e intangible de la mecanización. Compañía Minera Antamina.

Green Policy Platform. (2024). Knowledge Gaps Report: Environmental Aspects of Tailings Management (January 2024).

ICMM. (2020). New global industry standard on tailings management. International Council on Mining and Metals.

Ulrich, B. & Coffin, J. (2015). TMW 2015 – Combined Tailings and Mine Waste.

Minemax. (2015). Why mine planning is all about collaboration. Minemax News.

Wheaton Precious Metals. (2015). Antamina Project overview.

INTEGRATING COMMINGLING INTO THE STRATEGIC WASTE ROCK PLANNING AT ANTAMINA MINE

Christa Quiroz1, David Machin1, Olimpia Cabrera1, Fernando Angeles1

1 Antamina Mine, Lima, Peru

(*Presenting author: cquirozc@antamina.com)

ABSTRACT

Antamina is a large open-pit polymetallic mine in Peru, operating with a processing capacity of 145 ktpd. In operation since 2001, the current mine life extends to 2036. Mining is conducted at a rate of 290 Mtpa using conventional truck-and-shovel equipment. This technical paper presents a comprehensive assessment of commingling as a transformative technology in mine planning, positioning waste rock and tailings management as a key factor in the decision-making process. The primary objective of this study is to demonstrate the benefits of this technology within the mine planning process, highlighting waste rock and tailings management as one of the main drivers in strategic decision-making. The methodology applied is a Comparative Case Analysis, which includes a review of the design and sequencing process for mining infrastructure such as waste rock storage facilities, along with a discussion of the key parameters considered. The results show that the implementation of this technology delivers significant economic benefits, including an approximate 30% reduction in capital costs for waste rock storage facilities, improved land-use efficiency, and optimized infrastructure development. Furthermore, the approach enhances alignment with the Global Industry Standard on Tailings Management (GISTM).

KEYWORDS

commingling, mine planning, waste rock management, tailings management, GISTM compliance

1	INTRODUCTION

The mining industry faces growing challenges related to the sustainable management of waste rock and tailings, especially after catastrophic events such as those that occurred in Brumadinho and Samarco (Green Policy Platform, 2024; ICMM, 2020). In this context, the Antamina mine, operating since 2001 as one of the largest polymetallic operations in the world, has developed a pioneering approach that integrates commingling as a central element of its strategic planning (Wheaton Precious Metals, 2015).

Commingling, defined as the homogeneous mixing of dewatered tailings with waste rock to form a geotechnically stable material, represents a natural evolution of traditional co-disposal practices (Burden & Wilson, 2023). However, what distinguishes the Antamina case is the integration of this technology from the initial phases of mine planning, making waste rock management the main driver of operational and strategic decisions.

2	OBJECTIVES

•	Demonstrate the benefits of integrating commingling into mine planning, positioning waste rock and tailings management as a strategic driver for decision-making in open-pit operations.

•	Optimize operational efficiency and environmental sustainability through the adoption of commingling in waste rock and tailings management.

•	Align waste rock and tailings management with international sustainability and safety standards, such as the GISTM, through the application of commingling.

•	Analyze, through comparative case studies, the impact of commingling on land-use optimization and mine life extension.

•	Reduce capital and operating costs associated with waste rock and tailings disposal by implementing commingling technologies.

•	Present the key findings and lessons learned from the integration of commingling into long-term mine planning.

3	TECHNICAL FOUNDATIONS OF COMMINGLING IN MINE PLANNING

3.1	Desing Principles and Material Characterization

Commingling at Antamina is based on engineering principles that seek to combine the superior structural properties of waste rock with the low permeability characteristics of tailings (Boshoff, 2023). This combination results in a material with shear strength similar to waste rock and permeability comparable to tailings, creating conditions that restrict oxygen entry and water filtration, significantly reducing the potential for acid drainage generation (Ulrich & Coffin, 2015; Burden & Wilson, 2023).

The characterization of commingled material is performed through analysis of five distinct structural configurations: rock-dominated uncompacted, fines-dominated uncompacted, rock-dominated compacted, fines-dominated compacted, and the "just-filled" configuration where fines exactly fill the voids between rock particles (Burden & Wilson, 2023). This latter configuration represents the optimal target of the commingling process at Antamina.

Figure 1 Blend configurations after Wickland et al. (2006). Source: Burden & Wilson (2023).

3.2	Integration into the Strategic Planning Process

The integration of commingling into Antamina's mine planning follows a structured methodological approach that places tailings management with waste rock as central elements of decision-making. This global strategy allows for optimizing the mixture of materials from the mine to the plant, considering the operational and environmental constraints associated with the tailings storage facility. By incorporating tailings behavior in the early stages of planning, greater efficiency in resource use and better risk management are achieved.

The global strategy is divided into three key axes:

•	Strategy 1: Ensure Future Expansions: The first strategy focuses on maximizing tailings storage capacity for current and future needs, leveraging Antamina's mineral resource and applying advanced technology to increase TSF (Tailings Storage Facility) capacity to ensure the mine's long-term operational continuity.

•	Strategy 2: Optimized footprint: The second strategy seeks to optimize the overall project footprint by balancing waste and tailings storage space. This approach aligns directly with commingling principles by maximizing efficient use of available land within the current operational footprint.

•	Strategy 3: Reduce upfront CAPEX: The third strategy focuses on reducing initial CAPEX to improve key financial indicators (IRR), which directly aligns with the demonstrated economic benefits of commingling, minimizing operational costs with a nearby TSF and optimizing CAPEX timing.

Figure 2 Strategic vision of mine planning approach with commingling

The strategies implemented in tailings dam and waste dump management seek to maximize the operational life of the mine and optimize asset value. These decisions translate into concrete actions that address sustainability, operational efficiency, and cost reduction, ensuring continuity and long-term success of the mining operation.

•	Long Life Asset: Antamina's vision as a long-life asset involves making decisions based on sustainable exploitation of its exceptional mineral resources. This includes adopting advanced technologies, such as commingling, to improve storage capacity and ensure operational continuity until 2036 and beyond, thus maximizing the value of the mineral resource.

•	Reduce Environmental Social Footprint: The strategic approach prioritizes reducing environmental and social impact through operation within the current footprint, increasing water availability through filtration, optimizing space use, and minimizing risks associated with tailings dams. Commingling contributes not only to technical and economic efficiency but also to sustainability objectives and corporate social responsibility.

•	Reduce Capital and Operational Cost: The cost reduction strategy focuses on optimizing capital investments (CAPEX) and their schedule, as well as minimizing operational costs through strategic facility location and leveraging synergies to avoid duplicate infrastructure. This enables more efficient and profitable resource management.

During the strategic planning phase, conceptual foundations for commingling implementation are established, including site characterization, regulatory framework development, and technical-economic feasibility analysis.

4	ANALYSIS OF THE MINING SYSTEM WITH COMMINGLING APPLICATION

4.1	Methodological Process Structure

The methodological process is organized in sequential stages that ensure effective integration of commingling into mine planning:

•	Comprehensive material evaluation: Detailed analysis of geotechnical and geochemical characteristics of available materials, identifying their suitability for mixtures and joint disposal.

•	Mixture design development: Design of optimized mixtures that maximize desired properties of commingled material, such as low permeability and high stability, minimizing acid water generation and improving mechanical behavior of the deposit.

•	Pilot test implementation: Execution of pilot and industrial scale tests to validate mixture performance and adjust design parameters according to results obtained under real operating conditions.

•	Monitoring and adjustment: Establishment of continuous monitoring system to evaluate consolidation, stability, and geochemical behavior of deposits, allowing operational and design adjustments as new information is collected.

5	IMPLEMENTATION AND EVALUATION METHODOLOGY FOR COMMINGLING IN MINE PLANNING

5.1	Comparative Case Analysis

The methodology applied at Antamina is based on Comparative Case Analysis, allowing for comprehensive review of the design and sequencing process of mining infrastructure, with emphasis on dams, waste rock dumps, and integration of commingling technologies. This methodological approach comprises:

•	Review of design and sequencing processes: Analysis of procedures used in waste dump planning and disposal, considering both geotechnical and geochemical criteria to ensure infrastructure stability and sustainability.

•	Identification of key variables: Selection of fundamental operational parameters, such as granulometry, permeability, geochemical behavior, and storage capacity, which directly affect the viability and performance of dumps and material mixtures.

•	Definition of case tree: Structure of a decision tree that contemplates long-term strategic scenarios, allowing comparison of alternatives under different combinations of variables and operational constraints.

•	Results evaluation: Application of sensitivity analyses and cost-benefit evaluations to determine the impact of each alternative on mine life extension, environmental risk reduction, and resource optimization.

5.2	Decision Tree for Strategic Planning

The decision tree developed for long-term strategic planning at Antamina follows sequential logic that allows evaluation of feasibility and impact of main operational alternatives:

•	Commingling feasibility: The first node of the tree determines whether integration of tailings and waste rock is technically and economically viable, considering material characteristics and regulatory constraints.

•	Entry into new area: If commingling is feasible, the next node evaluates the possibility of disposing material in a new area, analyzing topographic, environmental, and access aspects.

•	Need for new dam: Finally, it determines whether construction of a new tailings dam is essential or if the commingling solution allows dispensing with this infrastructure, optimizing investment and reducing environmental impact.

Each of these decisions conditions mine life extension, efficient space use, and environmental risk management, allowing selection of the alternative that maximizes economic, social, and environmental value of the project.

Figure 3  Decision Tree for Long-Term Strategic Scenario Planning

5.3	Evaluation of Operational Parameters and Modifications by Commingling

Key operational parameters—including waste rock-tailings ratio, mixing methods, transport systems, and processing criteria—are assessed for their impact on commingled system efficiency. The evaluation considers site-specific challenges such as extreme Andean weather, high-altitude logistics, and Peruvian regulations, all of which are especially critical at Antamina and require customized commingling solutions.

•	Modifications to tailings management by commingling:

The commingling process fundamentally transforms tailings management by utilizing voids within waste rock for storage, with available space typically equivalent to about 20% of the crushed rock volume transported by conveyors. To enable mixing, tailings must first be dewatered. The following image shows both the current process (without mechanization) and the transformation that occurs when commingling is incorporated.

Figure 4 Transformation of Tailings Management: Evolution from Conventional Process to Commingling|

•	 Operational parameters and assumptions

Commingling extends tailings facility life by optimizing spatial use of waste rock voids. Implementation requires mechanized systems (WCCS) for transporting crushed material mixed with dewatered tailings, while maintaining two key operational parameters: a) Unchanged transport capacity: 83.2 Mtpa in the conveyor system and b) Volumetric stability: No increase in total volume of waste rock dump.

This approach ensures efficient resource utilization and supports sustainable mine operations.

Table 1: Key operational parameters and assumptions

Component	Units	Without - Case Conventional	With - Case Commingling

Cycloned/Filtered Max. Capacity	Mtpa	-	32.0

Percentage of Voids in Waste Rock	% Volume 	 	20 %

Waste Rock / Tailings Ratio 	Mt Rock/

Mt Tails	-	83 / 16

Conventional Tailings Density	t/m3 	1.81	1.81

Fines Tailings Density	 t/m3	NA	1.60

Tailings Volume, TDR 4195 (to Dic-22)	Mm3	376	376

Tailings Capacity, TDR 4195 (to Dic-22)	Mt	682	652

Mill Feed Rate 	Ktpd	145	145

The following image shows how incorporation of mechanized systems and commingling add capacity, and therefore extend tailings facility life, to increase the life of mine and therefore the overall asset value.

Figure 5 Commingling Extension of Tailings Facility Life through Commingling

The curve below shows the relationship between dry density and optimal mixing proportion, determinant for maximizing commingling efficiency.

Figure 6: Dry density curve vs. optimal mixing proportion for commingling

•	Analysis of Mineable Resources, Production and Value

A three-dimensional diagram is used to strategically analyze the interactions between mineable resource volume, production rate, and generated value—whether economic, environmental, or operational. This tool helps identify how increasing resource volume and production can maximize value, but also highlights key constraints such as tailings dam capacity and property limits. The approach supports comprehensive decision-making to optimize asset value and ensure project sustainability across various operational scenarios.

Figure 7 Value Optimization: Interaction between Resource Volume, Production Rate and Key Restrictions

6	ECONOMIC AND OPERATIONAL ADVANTAGES

6.1	Perspective of Conventional Value Drivers

The following figure illustrates Antamina's main conventional value drivers from a financial perspective. In the current scenario, the amount of mineral resource that can be utilized directly depends on the capital that the company decides to invest (Capex), both in new investments (Growth Capex) and in maintaining existing operations (Sustaining Capex). This generates a necessary balance to determine the optimal scenario that maximizes the value of the existing asset.

Figure 8: Conventional Antamina Value Drivers: Financial Perspective (Without Commingling)

6.2	Impact of Commingling on Financial Value Drivers

From a financial perspective, the adoption of commingling among Antamina’s value drivers removes constraints on mineral resources by increasing tailings storage capacity, enabling the utilization of new dumps, and allowing for an expansion in the volume of usable resources. As a result, the mine’s operational life is extended and asset value is enhanced. Regarding dump utilization, the implementation of commingling has the potential to reduce the requirement for dumps located in karstic zones, which demand significant capital expenditure (Capex). This can lead to a reduction in initial investment by up to 30% in scenarios comparing commingling versus non-commingling approaches. This saving is achieved through the integration of previously separate infrastructures into a single system, which optimizes the use of available area in the East Extension dumps and prioritizes the use of mechanized systems over conventional trucking methods.

Figure 9: Antamina Value Drivers: Financial Perspective with Commingling

The optimization of land use represents another important economic benefit, especially relevant in mountainous locations like Antamina where available land for mining facilities is limited. Commingling allows for the creation of deposits with a smaller footprint compared to separate facilities, freeing up land for other productive uses or reducing the need for additional land acquisition.

7	ALIGNMENT WITH GISTM STANDARDS AND SUSTAINABILITY

The integration of commingling into mine planning strengthens compliance with the Global Industry Standard on Tailings Management (GISTM), by facilitating safer and more sustainable management of waste materials. The GISTM requires the planning, construction, operation, and closure of tailings facilities with a focus on risk reduction and continuous monitoring throughout the lifecycle. Commingling contributes to this objective by improving the geotechnical and geochemical stability of deposits, reducing the risk of failure and acid drainage generation, which in turn reduces environmental impact and long-term treatment costs. Additionally, the consolidation of facilities through commingling reduces the environmental footprint and facilitates supervision and control, aligning with the principles of sustainability and social responsibility in the mining industry.

8	LESSONS LEARNED AND CHALLENGES

Among the main lessons learned and challenges of implementing commingling in mine planning at Antamina, the following stand out:

•	Large-scale operations and mechanization: Commingling is primarily viable in large-volume mechanized operations, where it is possible to achieve a homogeneous and controlled mixture of materials. However, the magnitude of waste rock and tailings tonnages at Antamina implies a new challenge, never seen before.

•	Flexible planning and pilot tests: It is essential to include areas for pilot tests in short and medium-term plans, allowing adjustment of mixing parameters according to material variability.

•	Regulatory communication: Technology acceptance requires identifying precedents and working closely with authorities to develop appropriate regulatory frameworks.

•	Organizational change management: The transition to commingling involves challenges in personnel training and adaptation of operational processes, especially in mature operations.

9	CONCLUSIONS

•	The comparative discussion and case analysis of commingling implementation at Antamina have enabled the identification and selection of strategic alternatives that maximize the economic, environmental, and social value of the project.

•	The use of decision trees and scenario evaluation has facilitated informed decision-making, taking into account technical and economic feasibility as well as regulatory and environmental constraints.

•	The integration of commingling into strategic mine planning has proven to be a key tool for optimizing waste rock and tailings management, achieving a significant reduction in capital costs (up to 30%), greater efficiency in land use, and an extension of tailings storage facility (TSF) life. This approach has also aligned operations with international sustainability standards (GISTM) and strengthened risk management, contributing to operational continuity and long-term reduction of environmental impacts.

•	The Antamina experience demonstrates that structured case discussions and the application of comparative methodologies are fundamental for strategic planning, as they allow anticipation of challenges, validation of solutions through pilot testing, and adjustment of operational parameters based on real outcomes. Thus, commingling is consolidated as a transformative practice in modern mining, fostering more robust and sustainable decision-making for the development of long-life mining assets.

ACKNOWLEDGEMENTS

The authors express their sincere gratitude to Antamina for their invaluable support. Special thanks are extended to the Tailings Strategy Management team and the Long-Term Planning team for their collaboration and insights.

REFERENCES

Boshoff, J. et al. (2023). A case study on the commingling of tailings and waste rock at a Brownfields open cast mine in Ghana. University of Alberta.

Burden, R., & Wilson, G. W. (2023). Commingling of waste rock and tailings to improve “dry stack” performance: Design and evaluation of mixtures. Minerals, 13(2), 295.

Deza, N., & Montes, K. (2022). Mecanización del desmonte en Compañía Minera Antamina: Propuesta para capturar el valor tangible e intangible de la mecanización. Compañía Minera Antamina.

Green Policy Platform. (2024). Knowledge Gaps Report: Environmental Aspects of Tailings Management (January 2024).

ICMM. (2020). New global industry standard on tailings management. International Council on Mining and Metals.

Ulrich, B. & Coffin, J. (2015). TMW 2015 – Combined Tailings and Mine Waste.

Minemax. (2015). Why mine planning is all about collaboration. Minemax News.

Wheaton Precious Metals. (2015). Antamina Project overview.

INTEGRATING COMMINGLING INTO THE STRATEGIC WASTE ROCK PLANNING AT ANTAMINA MINE

Christa Quiroz1, David Machin1, Olimpia Cabrera1, Fernando Angeles1

1 Antamina Mine, Lima, Peru

(*Presenting author: cquirozc@antamina.com)

ABSTRACT

Antamina is a large open-pit polymetallic mine in Peru, operating with a processing capacity of 145 ktpd. In operation since 2001, the current mine life extends to 2036. Mining is conducted at a rate of 290 Mtpa using conventional truck-and-shovel equipment. This technical paper presents a comprehensive assessment of commingling as a transformative technology in mine planning, positioning waste rock and tailings management as a key factor in the decision-making process. The primary objective of this study is to demonstrate the benefits of this technology within the mine planning process, highlighting waste rock and tailings management as one of the main drivers in strategic decision-making. The methodology applied is a Comparative Case Analysis, which includes a review of the design and sequencing process for mining infrastructure such as waste rock storage facilities, along with a discussion of the key parameters considered. The results show that the implementation of this technology delivers significant economic benefits, including an approximate 30% reduction in capital costs for waste rock storage facilities, improved land-use efficiency, and optimized infrastructure development. Furthermore, the approach enhances alignment with the Global Industry Standard on Tailings Management (GISTM).

KEYWORDS

commingling, mine planning, waste rock management, tailings management, GISTM compliance

10	INTRODUCTION

The mining industry faces growing challenges related to the sustainable management of waste rock and tailings, especially after catastrophic events such as those that occurred in Brumadinho and Samarco (Green Policy Platform, 2024; ICMM, 2020). In this context, the Antamina mine, operating since 2001 as one of the largest polymetallic operations in the world, has developed a pioneering approach that integrates commingling as a central element of its strategic planning (Wheaton Precious Metals, 2015).

Commingling, defined as the homogeneous mixing of dewatered tailings with waste rock to form a geotechnically stable material, represents a natural evolution of traditional co-disposal practices (Burden & Wilson, 2023). However, what distinguishes the Antamina case is the integration of this technology from the initial phases of mine planning, making waste rock management the main driver of operational and strategic decisions.

11	OBJECTIVES

•	Demonstrate the benefits of integrating commingling into mine planning, positioning waste rock and tailings management as a strategic driver for decision-making in open-pit operations.

•	Optimize operational efficiency and environmental sustainability through the adoption of commingling in waste rock and tailings management.

•	Align waste rock and tailings management with international sustainability and safety standards, such as the GISTM, through the application of commingling.

•	Analyze, through comparative case studies, the impact of commingling on land-use optimization and mine life extension.

•	Reduce capital and operating costs associated with waste rock and tailings disposal by implementing commingling technologies.

•	Present the key findings and lessons learned from the integration of commingling into long-term mine planning.

12	TECHNICAL FOUNDATIONS OF COMMINGLING IN MINE PLANNING

12.1	Desing Principles and Material Characterization

Commingling at Antamina is based on engineering principles that seek to combine the superior structural properties of waste rock with the low permeability characteristics of tailings (Boshoff, 2023). This combination results in a material with shear strength similar to waste rock and permeability comparable to tailings, creating conditions that restrict oxygen entry and water filtration, significantly reducing the potential for acid drainage generation (Ulrich & Coffin, 2015; Burden & Wilson, 2023).

The characterization of commingled material is performed through analysis of five distinct structural configurations: rock-dominated uncompacted, fines-dominated uncompacted, rock-dominated compacted, fines-dominated compacted, and the "just-filled" configuration where fines exactly fill the voids between rock particles (Burden & Wilson, 2023). This latter configuration represents the optimal target of the commingling process at Antamina.

Figure 1 Blend configurations after Wickland et al. (2006). Source: Burden & Wilson (2023).

12.2	Integration into the Strategic Planning Process

The integration of commingling into Antamina's mine planning follows a structured methodological approach that places tailings management with waste rock as central elements of decision-making. This global strategy allows for optimizing the mixture of materials from the mine to the plant, considering the operational and environmental constraints associated with the tailings storage facility. By incorporating tailings behavior in the early stages of planning, greater efficiency in resource use and better risk management are achieved.

The global strategy is divided into three key axes:

•	Strategy 1: Ensure Future Expansions: The first strategy focuses on maximizing tailings storage capacity for current and future needs, leveraging Antamina's mineral resource and applying advanced technology to increase TSF (Tailings Storage Facility) capacity to ensure the mine's long-term operational continuity.

•	Strategy 2: Optimized footprint: The second strategy seeks to optimize the overall project footprint by balancing waste and tailings storage space. This approach aligns directly with commingling principles by maximizing efficient use of available land within the current operational footprint.

•	Strategy 3: Reduce upfront CAPEX: The third strategy focuses on reducing initial CAPEX to improve key financial indicators (IRR), which directly aligns with the demonstrated economic benefits of commingling, minimizing operational costs with a nearby TSF and optimizing CAPEX timing.

Figure 2 Strategic vision of mine planning approach with commingling

The strategies implemented in tailings dam and waste dump management seek to maximize the operational life of the mine and optimize asset value. These decisions translate into concrete actions that address sustainability, operational efficiency, and cost reduction, ensuring continuity and long-term success of the mining operation.

•	Long Life Asset: Antamina's vision as a long-life asset involves making decisions based on sustainable exploitation of its exceptional mineral resources. This includes adopting advanced technologies, such as commingling, to improve storage capacity and ensure operational continuity until 2036 and beyond, thus maximizing the value of the mineral resource.

•	Reduce Environmental Social Footprint: The strategic approach prioritizes reducing environmental and social impact through operation within the current footprint, increasing water availability through filtration, optimizing space use, and minimizing risks associated with tailings dams. Commingling contributes not only to technical and economic efficiency but also to sustainability objectives and corporate social responsibility.

•	Reduce Capital and Operational Cost: The cost reduction strategy focuses on optimizing capital investments (CAPEX) and their schedule, as well as minimizing operational costs through strategic facility location and leveraging synergies to avoid duplicate infrastructure. This enables more efficient and profitable resource management.

During the strategic planning phase, conceptual foundations for commingling implementation are established, including site characterization, regulatory framework development, and technical-economic feasibility analysis.

13	ANALYSIS OF THE MINING SYSTEM WITH COMMINGLING APPLICATION

13.1	Methodological Process Structure

The methodological process is organized in sequential stages that ensure effective integration of commingling into mine planning:

•	Comprehensive material evaluation: Detailed analysis of geotechnical and geochemical characteristics of available materials, identifying their suitability for mixtures and joint disposal.

•	Mixture design development: Design of optimized mixtures that maximize desired properties of commingled material, such as low permeability and high stability, minimizing acid water generation and improving mechanical behavior of the deposit.

•	Pilot test implementation: Execution of pilot and industrial scale tests to validate mixture performance and adjust design parameters according to results obtained under real operating conditions.

•	Monitoring and adjustment: Establishment of continuous monitoring system to evaluate consolidation, stability, and geochemical behavior of deposits, allowing operational and design adjustments as new information is collected.

14	IMPLEMENTATION AND EVALUATION METHODOLOGY FOR COMMINGLING IN MINE PLANNING

14.1	Comparative Case Analysis

The methodology applied at Antamina is based on Comparative Case Analysis, allowing for comprehensive review of the design and sequencing process of mining infrastructure, with emphasis on dams, waste rock dumps, and integration of commingling technologies. This methodological approach comprises:

•	Review of design and sequencing processes: Analysis of procedures used in waste dump planning and disposal, considering both geotechnical and geochemical criteria to ensure infrastructure stability and sustainability.

•	Identification of key variables: Selection of fundamental operational parameters, such as granulometry, permeability, geochemical behavior, and storage capacity, which directly affect the viability and performance of dumps and material mixtures.

•	Definition of case tree: Structure of a decision tree that contemplates long-term strategic scenarios, allowing comparison of alternatives under different combinations of variables and operational constraints.

•	Results evaluation: Application of sensitivity analyses and cost-benefit evaluations to determine the impact of each alternative on mine life extension, environmental risk reduction, and resource optimization.

14.2	Decision Tree for Strategic Planning

The decision tree developed for long-term strategic planning at Antamina follows sequential logic that allows evaluation of feasibility and impact of main operational alternatives:

•	Commingling feasibility: The first node of the tree determines whether integration of tailings and waste rock is technically and economically viable, considering material characteristics and regulatory constraints.

•	Entry into new area: If commingling is feasible, the next node evaluates the possibility of disposing material in a new area, analyzing topographic, environmental, and access aspects.

•	Need for new dam: Finally, it determines whether construction of a new tailings dam is essential or if the commingling solution allows dispensing with this infrastructure, optimizing investment and reducing environmental impact.

Each of these decisions conditions mine life extension, efficient space use, and environmental risk management, allowing selection of the alternative that maximizes economic, social, and environmental value of the project.

Figure 3  Decision Tree for Long-Term Strategic Scenario Planning

14.3	Evaluation of Operational Parameters and Modifications by Commingling

Key operational parameters—including waste rock-tailings ratio, mixing methods, transport systems, and processing criteria—are assessed for their impact on commingled system efficiency. The evaluation considers site-specific challenges such as extreme Andean weather, high-altitude logistics, and Peruvian regulations, all of which are especially critical at Antamina and require customized commingling solutions.

•	Modifications to tailings management by commingling:

The commingling process fundamentally transforms tailings management by utilizing voids within waste rock for storage, with available space typically equivalent to about 20% of the crushed rock volume transported by conveyors. To enable mixing, tailings must first be dewatered. The following image shows both the current process (without mechanization) and the transformation that occurs when commingling is incorporated.

Figure 4 Transformation of Tailings Management: Evolution from Conventional Process to Commingling|

•	 Operational parameters and assumptions

Commingling extends tailings facility life by optimizing spatial use of waste rock voids. Implementation requires mechanized systems (WCCS) for transporting crushed material mixed with dewatered tailings, while maintaining two key operational parameters: a) Unchanged transport capacity: 83.2 Mtpa in the conveyor system and b) Volumetric stability: No increase in total volume of waste rock dump.

This approach ensures efficient resource utilization and supports sustainable mine operations.

Table 1: Key operational parameters and assumptions

Component	Units	Without - Case Conventional	With - Case Commingling

Cycloned/Filtered Max. Capacity	Mtpa	-	32.0

Percentage of Voids in Waste Rock	% Volume 	 	20 %

Waste Rock / Tailings Ratio 	Mt Rock/

Mt Tails	-	83 / 16

Conventional Tailings Density	t/m3 	1.81	1.81

Fines Tailings Density	 t/m3	NA	1.60

Tailings Volume, TDR 4195 (to Dic-22)	Mm3	376	376

Tailings Capacity, TDR 4195 (to Dic-22)	Mt	682	652

Mill Feed Rate 	Ktpd	145	145

The following image shows how incorporation of mechanized systems and commingling add capacity, and therefore extend tailings facility life, to increase the life of mine and therefore the overall asset value.

Figure 5 Commingling Extension of Tailings Facility Life through Commingling

The curve below shows the relationship between dry density and optimal mixing proportion, determinant for maximizing commingling efficiency.

Figure 6: Dry density curve vs. optimal mixing proportion for commingling

•	Analysis of Mineable Resources, Production and Value

A three-dimensional diagram is used to strategically analyze the interactions between mineable resource volume, production rate, and generated value—whether economic, environmental, or operational. This tool helps identify how increasing resource volume and production can maximize value, but also highlights key constraints such as tailings dam capacity and property limits. The approach supports comprehensive decision-making to optimize asset value and ensure project sustainability across various operational scenarios.

Figure 7 Value Optimization: Interaction between Resource Volume, Production Rate and Key Restrictions

15	ECONOMIC AND OPERATIONAL ADVANTAGES

15.1	Perspective of Conventional Value Drivers

The following figure illustrates Antamina's main conventional value drivers from a financial perspective. In the current scenario, the amount of mineral resource that can be utilized directly depends on the capital that the company decides to invest (Capex), both in new investments (Growth Capex) and in maintaining existing operations (Sustaining Capex). This generates a necessary balance to determine the optimal scenario that maximizes the value of the existing asset.

Figure 8: Conventional Antamina Value Drivers: Financial Perspective (Without Commingling)

15.2	Impact of Commingling on Financial Value Drivers

From a financial perspective, the adoption of commingling among Antamina’s value drivers removes constraints on mineral resources by increasing tailings storage capacity, enabling the utilization of new dumps, and allowing for an expansion in the volume of usable resources. As a result, the mine’s operational life is extended and asset value is enhanced. Regarding dump utilization, the implementation of commingling has the potential to reduce the requirement for dumps located in karstic zones, which demand significant capital expenditure (Capex). This can lead to a reduction in initial investment by up to 30% in scenarios comparing commingling versus non-commingling approaches. This saving is achieved through the integration of previously separate infrastructures into a single system, which optimizes the use of available area in the East Extension dumps and prioritizes the use of mechanized systems over conventional trucking methods.

Figure 9: Antamina Value Drivers: Financial Perspective with Commingling

The optimization of land use represents another important economic benefit, especially relevant in mountainous locations like Antamina where available land for mining facilities is limited. Commingling allows for the creation of deposits with a smaller footprint compared to separate facilities, freeing up land for other productive uses or reducing the need for additional land acquisition.

16	ALIGNMENT WITH GISTM STANDARDS AND SUSTAINABILITY

The integration of commingling into mine planning strengthens compliance with the Global Industry Standard on Tailings Management (GISTM), by facilitating safer and more sustainable management of waste materials. The GISTM requires the planning, construction, operation, and closure of tailings facilities with a focus on risk reduction and continuous monitoring throughout the lifecycle. Commingling contributes to this objective by improving the geotechnical and geochemical stability of deposits, reducing the risk of failure and acid drainage generation, which in turn reduces environmental impact and long-term treatment costs. Additionally, the consolidation of facilities through commingling reduces the environmental footprint and facilitates supervision and control, aligning with the principles of sustainability and social responsibility in the mining industry.

17	LESSONS LEARNED AND CHALLENGES

Among the main lessons learned and challenges of implementing commingling in mine planning at Antamina, the following stand out:

•	Large-scale operations and mechanization: Commingling is primarily viable in large-volume mechanized operations, where it is possible to achieve a homogeneous and controlled mixture of materials. However, the magnitude of waste rock and tailings tonnages at Antamina implies a new challenge, never seen before.

•	Flexible planning and pilot tests: It is essential to include areas for pilot tests in short and medium-term plans, allowing adjustment of mixing parameters according to material variability.

•	Regulatory communication: Technology acceptance requires identifying precedents and working closely with authorities to develop appropriate regulatory frameworks.

•	Organizational change management: The transition to commingling involves challenges in personnel training and adaptation of operational processes, especially in mature operations.

18	CONCLUSIONS

•	The comparative discussion and case analysis of commingling implementation at Antamina have enabled the identification and selection of strategic alternatives that maximize the economic, environmental, and social value of the project.

•	The use of decision trees and scenario evaluation has facilitated informed decision-making, taking into account technical and economic feasibility as well as regulatory and environmental constraints.

•	The integration of commingling into strategic mine planning has proven to be a key tool for optimizing waste rock and tailings management, achieving a significant reduction in capital costs (up to 30%), greater efficiency in land use, and an extension of tailings storage facility (TSF) life. This approach has also aligned operations with international sustainability standards (GISTM) and strengthened risk management, contributing to operational continuity and long-term reduction of environmental impacts.

•	The Antamina experience demonstrates that structured case discussions and the application of comparative methodologies are fundamental for strategic planning, as they allow anticipation of challenges, validation of solutions through pilot testing, and adjustment of operational parameters based on real outcomes. Thus, commingling is consolidated as a transformative practice in modern mining, fostering more robust and sustainable decision-making for the development of long-life mining assets.

ACKNOWLEDGEMENTS

The authors express their sincere gratitude to Antamina for their invaluable support. Special thanks are extended to the Tailings Strategy Management team and the Long-Term Planning team for their collaboration and insights.

REFERENCES

Boshoff, J. et al. (2023). A case study on the commingling of tailings and waste rock at a Brownfields open cast mine in Ghana. University of Alberta.

Burden, R., & Wilson, G. W. (2023). Commingling of waste rock and tailings to improve “dry stack” performance: Design and evaluation of mixtures. Minerals, 13(2), 295.

Deza, N., & Montes, K. (2022). Mecanización del desmonte en Compañía Minera Antamina: Propuesta para capturar el valor tangible e intangible de la mecanización. Compañía Minera Antamina.

Green Policy Platform. (2024). Knowledge Gaps Report: Environmental Aspects of Tailings Management (January 2024).

ICMM. (2020). New global industry standard on tailings management. International Council on Mining and Metals.

Ulrich, B. & Coffin, J. (2015). TMW 2015 – Combined Tailings and Mine Waste.

Minemax. (2015). Why mine planning is all about collaboration. Minemax News.

Wheaton Precious Metals. (2015). Antamina Project overview.

INTEGRATING COMMINGLING INTO THE STRATEGIC WASTE ROCK PLANNING AT ANTAMINA MINE

Christa Quiroz1, David Machin1, Olimpia Cabrera1, Fernando Angeles1

1 Antamina Mine, Lima, Peru

(*Presenting author: cquirozc@antamina.com)

ABSTRACT

Antamina is a large open-pit polymetallic mine in Peru, operating with a processing capacity of 145 ktpd. In operation since 2001, the current mine life extends to 2036. Mining is conducted at a rate of 290 Mtpa using conventional truck-and-shovel equipment. This technical paper presents a comprehensive assessment of commingling as a transformative technology in mine planning, positioning waste rock and tailings management as a key factor in the decision-making process. The primary objective of this study is to demonstrate the benefits of this technology within the mine planning process, highlighting waste rock and tailings management as one of the main drivers in strategic decision-making. The methodology applied is a Comparative Case Analysis, which includes a review of the design and sequencing process for mining infrastructure such as waste rock storage facilities, along with a discussion of the key parameters considered. The results show that the implementation of this technology delivers significant economic benefits, including an approximate 30% reduction in capital costs for waste rock storage facilities, improved land-use efficiency, and optimized infrastructure development. Furthermore, the approach enhances alignment with the Global Industry Standard on Tailings Management (GISTM).

KEYWORDS

commingling, mine planning, waste rock management, tailings management, GISTM compliance

1	INTRODUCTION

The mining industry faces growing challenges related to the sustainable management of waste rock and tailings, especially after catastrophic events such as those that occurred in Brumadinho and Samarco (Green Policy Platform, 2024; ICMM, 2020). In this context, the Antamina mine, operating since 2001 as one of the largest polymetallic operations in the world, has developed a pioneering approach that integrates commingling as a central element of its strategic planning (Wheaton Precious Metals, 2015).

Commingling, defined as the homogeneous mixing of dewatered tailings with waste rock to form a geotechnically stable material, represents a natural evolution of traditional co-disposal practices (Burden & Wilson, 2023). However, what distinguishes the Antamina case is the integration of this technology from the initial phases of mine planning, making waste rock management the main driver of operational and strategic decisions.

2	OBJECTIVES

•	Demonstrate the benefits of integrating commingling into mine planning, positioning waste rock and tailings management as a strategic driver for decision-making in open-pit operations.

•	Optimize operational efficiency and environmental sustainability through the adoption of commingling in waste rock and tailings management.

•	Align waste rock and tailings management with international sustainability and safety standards, such as the GISTM, through the application of commingling.

•	Analyze, through comparative case studies, the impact of commingling on land-use optimization and mine life extension.

•	Reduce capital and operating costs associated with waste rock and tailings disposal by implementing commingling technologies.

•	Present the key findings and lessons learned from the integration of commingling into long-term mine planning.

3	TECHNICAL FOUNDATIONS OF COMMINGLING IN MINE PLANNING

3.1	Desing Principles and Material Characterization

Commingling at Antamina is based on engineering principles that seek to combine the superior structural properties of waste rock with the low permeability characteristics of tailings (Boshoff, 2023). This combination results in a material with shear strength similar to waste rock and permeability comparable to tailings, creating conditions that restrict oxygen entry and water filtration, significantly reducing the potential for acid drainage generation (Ulrich & Coffin, 2015; Burden & Wilson, 2023).

The characterization of commingled material is performed through analysis of five distinct structural configurations: rock-dominated uncompacted, fines-dominated uncompacted, rock-dominated compacted, fines-dominated compacted, and the "just-filled" configuration where fines exactly fill the voids between rock particles (Burden & Wilson, 2023). This latter configuration represents the optimal target of the commingling process at Antamina.

Figure 1 Blend configurations after Wickland et al. (2006). Source: Burden & Wilson (2023).

3.2	Integration into the Strategic Planning Process

The integration of commingling into Antamina's mine planning follows a structured methodological approach that places tailings management with waste rock as central elements of decision-making. This global strategy allows for optimizing the mixture of materials from the mine to the plant, considering the operational and environmental constraints associated with the tailings storage facility. By incorporating tailings behavior in the early stages of planning, greater efficiency in resource use and better risk management are achieved.

The global strategy is divided into three key axes:

•	Strategy 1: Ensure Future Expansions: The first strategy focuses on maximizing tailings storage capacity for current and future needs, leveraging Antamina's mineral resource and applying advanced technology to increase TSF (Tailings Storage Facility) capacity to ensure the mine's long-term operational continuity.

•	Strategy 2: Optimized footprint: The second strategy seeks to optimize the overall project footprint by balancing waste and tailings storage space. This approach aligns directly with commingling principles by maximizing efficient use of available land within the current operational footprint.

•	Strategy 3: Reduce upfront CAPEX: The third strategy focuses on reducing initial CAPEX to improve key financial indicators (IRR), which directly aligns with the demonstrated economic benefits of commingling, minimizing operational costs with a nearby TSF and optimizing CAPEX timing.

Figure 2 Strategic vision of mine planning approach with commingling

The strategies implemented in tailings dam and waste dump management seek to maximize the operational life of the mine and optimize asset value. These decisions translate into concrete actions that address sustainability, operational efficiency, and cost reduction, ensuring continuity and long-term success of the mining operation.

•	Long Life Asset: Antamina's vision as a long-life asset involves making decisions based on sustainable exploitation of its exceptional mineral resources. This includes adopting advanced technologies, such as commingling, to improve storage capacity and ensure operational continuity until 2036 and beyond, thus maximizing the value of the mineral resource.

•	Reduce Environmental Social Footprint: The strategic approach prioritizes reducing environmental and social impact through operation within the current footprint, increasing water availability through filtration, optimizing space use, and minimizing risks associated with tailings dams. Commingling contributes not only to technical and economic efficiency but also to sustainability objectives and corporate social responsibility.

•	Reduce Capital and Operational Cost: The cost reduction strategy focuses on optimizing capital investments (CAPEX) and their schedule, as well as minimizing operational costs through strategic facility location and leveraging synergies to avoid duplicate infrastructure. This enables more efficient and profitable resource management.

During the strategic planning phase, conceptual foundations for commingling implementation are established, including site characterization, regulatory framework development, and technical-economic feasibility analysis.

4	ANALYSIS OF THE MINING SYSTEM WITH COMMINGLING APPLICATION

4.1	Methodological Process Structure

The methodological process is organized in sequential stages that ensure effective integration of commingling into mine planning:

•	Comprehensive material evaluation: Detailed analysis of geotechnical and geochemical characteristics of available materials, identifying their suitability for mixtures and joint disposal.

•	Mixture design development: Design of optimized mixtures that maximize desired properties of commingled material, such as low permeability and high stability, minimizing acid water generation and improving mechanical behavior of the deposit.

•	Pilot test implementation: Execution of pilot and industrial scale tests to validate mixture performance and adjust design parameters according to results obtained under real operating conditions.

•	Monitoring and adjustment: Establishment of continuous monitoring system to evaluate consolidation, stability, and geochemical behavior of deposits, allowing operational and design adjustments as new information is collected.

5	IMPLEMENTATION AND EVALUATION METHODOLOGY FOR COMMINGLING IN MINE PLANNING

5.1	Comparative Case Analysis

The methodology applied at Antamina is based on Comparative Case Analysis, allowing for comprehensive review of the design and sequencing process of mining infrastructure, with emphasis on dams, waste rock dumps, and integration of commingling technologies. This methodological approach comprises:

•	Review of design and sequencing processes: Analysis of procedures used in waste dump planning and disposal, considering both geotechnical and geochemical criteria to ensure infrastructure stability and sustainability.

•	Identification of key variables: Selection of fundamental operational parameters, such as granulometry, permeability, geochemical behavior, and storage capacity, which directly affect the viability and performance of dumps and material mixtures.

•	Definition of case tree: Structure of a decision tree that contemplates long-term strategic scenarios, allowing comparison of alternatives under different combinations of variables and operational constraints.

•	Results evaluation: Application of sensitivity analyses and cost-benefit evaluations to determine the impact of each alternative on mine life extension, environmental risk reduction, and resource optimization.

5.2	Decision Tree for Strategic Planning

The decision tree developed for long-term strategic planning at Antamina follows sequential logic that allows evaluation of feasibility and impact of main operational alternatives:

•	Commingling feasibility: The first node of the tree determines whether integration of tailings and waste rock is technically and economically viable, considering material characteristics and regulatory constraints.

•	Entry into new area: If commingling is feasible, the next node evaluates the possibility of disposing material in a new area, analyzing topographic, environmental, and access aspects.

•	Need for new dam: Finally, it determines whether construction of a new tailings dam is essential or if the commingling solution allows dispensing with this infrastructure, optimizing investment and reducing environmental impact.

Each of these decisions conditions mine life extension, efficient space use, and environmental risk management, allowing selection of the alternative that maximizes economic, social, and environmental value of the project.

Figure 3  Decision Tree for Long-Term Strategic Scenario Planning

5.3	Evaluation of Operational Parameters and Modifications by Commingling

Key operational parameters—including waste rock-tailings ratio, mixing methods, transport systems, and processing criteria—are assessed for their impact on commingled system efficiency. The evaluation considers site-specific challenges such as extreme Andean weather, high-altitude logistics, and Peruvian regulations, all of which are especially critical at Antamina and require customized commingling solutions.

•	Modifications to tailings management by commingling:

The commingling process fundamentally transforms tailings management by utilizing voids within waste rock for storage, with available space typically equivalent to about 20% of the crushed rock volume transported by conveyors. To enable mixing, tailings must first be dewatered. The following image shows both the current process (without mechanization) and the transformation that occurs when commingling is incorporated.

Figure 4 Transformation of Tailings Management: Evolution from Conventional Process to Commingling|

•	 Operational parameters and assumptions

Commingling extends tailings facility life by optimizing spatial use of waste rock voids. Implementation requires mechanized systems (WCCS) for transporting crushed material mixed with dewatered tailings, while maintaining two key operational parameters: a) Unchanged transport capacity: 83.2 Mtpa in the conveyor system and b) Volumetric stability: No increase in total volume of waste rock dump.

This approach ensures efficient resource utilization and supports sustainable mine operations.

Table 1: Key operational parameters and assumptions

Component	Units	Without - Case Conventional	With - Case Commingling

Cycloned/Filtered Max. Capacity	Mtpa	-	32.0

Percentage of Voids in Waste Rock	% Volume 	 	20 %

Waste Rock / Tailings Ratio 	Mt Rock/

Mt Tails	-	83 / 16

Conventional Tailings Density	t/m3 	1.81	1.81

Fines Tailings Density	 t/m3	NA	1.60

Tailings Volume, TDR 4195 (to Dic-22)	Mm3	376	376

Tailings Capacity, TDR 4195 (to Dic-22)	Mt	682	652

Mill Feed Rate 	Ktpd	145	145

The following image shows how incorporation of mechanized systems and commingling add capacity, and therefore extend tailings facility life, to increase the life of mine and therefore the overall asset value.

Figure 5 Commingling Extension of Tailings Facility Life through Commingling

The curve below shows the relationship between dry density and optimal mixing proportion, determinant for maximizing commingling efficiency.

Figure 6: Dry density curve vs. optimal mixing proportion for commingling

•	Analysis of Mineable Resources, Production and Value

A three-dimensional diagram is used to strategically analyze the interactions between mineable resource volume, production rate, and generated value—whether economic, environmental, or operational. This tool helps identify how increasing resource volume and production can maximize value, but also highlights key constraints such as tailings dam capacity and property limits. The approach supports comprehensive decision-making to optimize asset value and ensure project sustainability across various operational scenarios.

Figure 7 Value Optimization: Interaction between Resource Volume, Production Rate and Key Restrictions

6	ECONOMIC AND OPERATIONAL ADVANTAGES

6.1	Perspective of Conventional Value Drivers

The following figure illustrates Antamina's main conventional value drivers from a financial perspective. In the current scenario, the amount of mineral resource that can be utilized directly depends on the capital that the company decides to invest (Capex), both in new investments (Growth Capex) and in maintaining existing operations (Sustaining Capex). This generates a necessary balance to determine the optimal scenario that maximizes the value of the existing asset.

Figure 8: Conventional Antamina Value Drivers: Financial Perspective (Without Commingling)

6.2	Impact of Commingling on Financial Value Drivers

From a financial perspective, the adoption of commingling among Antamina’s value drivers removes constraints on mineral resources by increasing tailings storage capacity, enabling the utilization of new dumps, and allowing for an expansion in the volume of usable resources. As a result, the mine’s operational life is extended and asset value is enhanced. Regarding dump utilization, the implementation of commingling has the potential to reduce the requirement for dumps located in karstic zones, which demand significant capital expenditure (Capex). This can lead to a reduction in initial investment by up to 30% in scenarios comparing commingling versus non-commingling approaches. This saving is achieved through the integration of previously separate infrastructures into a single system, which optimizes the use of available area in the East Extension dumps and prioritizes the use of mechanized systems over conventional trucking methods.

Figure 9: Antamina Value Drivers: Financial Perspective with Commingling

The optimization of land use represents another important economic benefit, especially relevant in mountainous locations like Antamina where available land for mining facilities is limited. Commingling allows for the creation of deposits with a smaller footprint compared to separate facilities, freeing up land for other productive uses or reducing the need for additional land acquisition.

7	ALIGNMENT WITH GISTM STANDARDS AND SUSTAINABILITY

The integration of commingling into mine planning strengthens compliance with the Global Industry Standard on Tailings Management (GISTM), by facilitating safer and more sustainable management of waste materials. The GISTM requires the planning, construction, operation, and closure of tailings facilities with a focus on risk reduction and continuous monitoring throughout the lifecycle. Commingling contributes to this objective by improving the geotechnical and geochemical stability of deposits, reducing the risk of failure and acid drainage generation, which in turn reduces environmental impact and long-term treatment costs. Additionally, the consolidation of facilities through commingling reduces the environmental footprint and facilitates supervision and control, aligning with the principles of sustainability and social responsibility in the mining industry.

8	LESSONS LEARNED AND CHALLENGES

Among the main lessons learned and challenges of implementing commingling in mine planning at Antamina, the following stand out:

•	Large-scale operations and mechanization: Commingling is primarily viable in large-volume mechanized operations, where it is possible to achieve a homogeneous and controlled mixture of materials. However, the magnitude of waste rock and tailings tonnages at Antamina implies a new challenge, never seen before.

•	Flexible planning and pilot tests: It is essential to include areas for pilot tests in short and medium-term plans, allowing adjustment of mixing parameters according to material variability.

•	Regulatory communication: Technology acceptance requires identifying precedents and working closely with authorities to develop appropriate regulatory frameworks.

•	Organizational change management: The transition to commingling involves challenges in personnel training and adaptation of operational processes, especially in mature operations.

9	CONCLUSIONS

•	The comparative discussion and case analysis of commingling implementation at Antamina have enabled the identification and selection of strategic alternatives that maximize the economic, environmental, and social value of the project.

•	The use of decision trees and scenario evaluation has facilitated informed decision-making, taking into account technical and economic feasibility as well as regulatory and environmental constraints.

•	The integration of commingling into strategic mine planning has proven to be a key tool for optimizing waste rock and tailings management, achieving a significant reduction in capital costs (up to 30%), greater efficiency in land use, and an extension of tailings storage facility (TSF) life. This approach has also aligned operations with international sustainability standards (GISTM) and strengthened risk management, contributing to operational continuity and long-term reduction of environmental impacts.

•	The Antamina experience demonstrates that structured case discussions and the application of comparative methodologies are fundamental for strategic planning, as they allow anticipation of challenges, validation of solutions through pilot testing, and adjustment of operational parameters based on real outcomes. Thus, commingling is consolidated as a transformative practice in modern mining, fostering more robust and sustainable decision-making for the development of long-life mining assets.

ACKNOWLEDGEMENTS

The authors express their sincere gratitude to Antamina for their invaluable support. Special thanks are extended to the Tailings Strategy Management team and the Long-Term Planning team for their collaboration and insights.

REFERENCES

Boshoff, J. et al. (2023). A case study on the commingling of tailings and waste rock at a Brownfields open cast mine in Ghana. University of Alberta.

Burden, R., & Wilson, G. W. (2023). Commingling of waste rock and tailings to improve “dry stack” performance: Design and evaluation of mixtures. Minerals, 13(2), 295.

Deza, N., & Montes, K. (2022). Mecanización del desmonte en Compañía Minera Antamina: Propuesta para capturar el valor tangible e intangible de la mecanización. Compañía Minera Antamina.

Green Policy Platform. (2024). Knowledge Gaps Report: Environmental Aspects of Tailings Management (January 2024).

ICMM. (2020). New global industry standard on tailings management. International Council on Mining and Metals.

Ulrich, B. & Coffin, J. (2015). TMW 2015 – Combined Tailings and Mine Waste.

Minemax. (2015). Why mine planning is all about collaboration. Minemax News.

Wheaton Precious Metals. (2015). Antamina Project overview.

INTEGRATING COMMINGLING INTO THE STRATEGIC WASTE ROCK PLANNING AT ANTAMINA MINE

Christa Quiroz1, David Machin1, Olimpia Cabrera1, Fernando Angeles1

1 Antamina Mine, Lima, Peru

(*Presenting author: cquirozc@antamina.com)

ABSTRACT

Antamina is a large open-pit polymetallic mine in Peru, operating with a processing capacity of 145 ktpd. In operation since 2001, the current mine life extends to 2036. Mining is conducted at a rate of 290 Mtpa using conventional truck-and-shovel equipment. This technical paper presents a comprehensive assessment of commingling as a transformative technology in mine planning, positioning waste rock and tailings management as a key factor in the decision-making process. The primary objective of this study is to demonstrate the benefits of this technology within the mine planning process, highlighting waste rock and tailings management as one of the main drivers in strategic decision-making. The methodology applied is a Comparative Case Analysis, which includes a review of the design and sequencing process for mining infrastructure such as waste rock storage facilities, along with a discussion of the key parameters considered. The results show that the implementation of this technology delivers significant economic benefits, including an approximate 30% reduction in capital costs for waste rock storage facilities, improved land-use efficiency, and optimized infrastructure development. Furthermore, the approach enhances alignment with the Global Industry Standard on Tailings Management (GISTM).

KEYWORDS

commingling, mine planning, waste rock management, tailings management, GISTM compliance

10	INTRODUCTION

The mining industry faces growing challenges related to the sustainable management of waste rock and tailings, especially after catastrophic events such as those that occurred in Brumadinho and Samarco (Green Policy Platform, 2024; ICMM, 2020). In this context, the Antamina mine, operating since 2001 as one of the largest polymetallic operations in the world, has developed a pioneering approach that integrates commingling as a central element of its strategic planning (Wheaton Precious Metals, 2015).

Commingling, defined as the homogeneous mixing of dewatered tailings with waste rock to form a geotechnically stable material, represents a natural evolution of traditional co-disposal practices (Burden & Wilson, 2023). However, what distinguishes the Antamina case is the integration of this technology from the initial phases of mine planning, making waste rock management the main driver of operational and strategic decisions.

11	OBJECTIVES

•	Demonstrate the benefits of integrating commingling into mine planning, positioning waste rock and tailings management as a strategic driver for decision-making in open-pit operations.

•	Optimize operational efficiency and environmental sustainability through the adoption of commingling in waste rock and tailings management.

•	Align waste rock and tailings management with international sustainability and safety standards, such as the GISTM, through the application of commingling.

•	Analyze, through comparative case studies, the impact of commingling on land-use optimization and mine life extension.

•	Reduce capital and operating costs associated with waste rock and tailings disposal by implementing commingling technologies.

•	Present the key findings and lessons learned from the integration of commingling into long-term mine planning.

12	TECHNICAL FOUNDATIONS OF COMMINGLING IN MINE PLANNING

12.1	Desing Principles and Material Characterization

Commingling at Antamina is based on engineering principles that seek to combine the superior structural properties of waste rock with the low permeability characteristics of tailings (Boshoff, 2023). This combination results in a material with shear strength similar to waste rock and permeability comparable to tailings, creating conditions that restrict oxygen entry and water filtration, significantly reducing the potential for acid drainage generation (Ulrich & Coffin, 2015; Burden & Wilson, 2023).

The characterization of commingled material is performed through analysis of five distinct structural configurations: rock-dominated uncompacted, fines-dominated uncompacted, rock-dominated compacted, fines-dominated compacted, and the "just-filled" configuration where fines exactly fill the voids between rock particles (Burden & Wilson, 2023). This latter configuration represents the optimal target of the commingling process at Antamina.

Figure 1 Blend configurations after Wickland et al. (2006). Source: Burden & Wilson (2023).

12.2	Integration into the Strategic Planning Process

The integration of commingling into Antamina's mine planning follows a structured methodological approach that places tailings management with waste rock as central elements of decision-making. This global strategy allows for optimizing the mixture of materials from the mine to the plant, considering the operational and environmental constraints associated with the tailings storage facility. By incorporating tailings behavior in the early stages of planning, greater efficiency in resource use and better risk management are achieved.

The global strategy is divided into three key axes:

•	Strategy 1: Ensure Future Expansions: The first strategy focuses on maximizing tailings storage capacity for current and future needs, leveraging Antamina's mineral resource and applying advanced technology to increase TSF (Tailings Storage Facility) capacity to ensure the mine's long-term operational continuity.

•	Strategy 2: Optimized footprint: The second strategy seeks to optimize the overall project footprint by balancing waste and tailings storage space. This approach aligns directly with commingling principles by maximizing efficient use of available land within the current operational footprint.

•	Strategy 3: Reduce upfront CAPEX: The third strategy focuses on reducing initial CAPEX to improve key financial indicators (IRR), which directly aligns with the demonstrated economic benefits of commingling, minimizing operational costs with a nearby TSF and optimizing CAPEX timing.

Figure 2 Strategic vision of mine planning approach with commingling

The strategies implemented in tailings dam and waste dump management seek to maximize the operational life of the mine and optimize asset value. These decisions translate into concrete actions that address sustainability, operational efficiency, and cost reduction, ensuring continuity and long-term success of the mining operation.

•	Long Life Asset: Antamina's vision as a long-life asset involves making decisions based on sustainable exploitation of its exceptional mineral resources. This includes adopting advanced technologies, such as commingling, to improve storage capacity and ensure operational continuity until 2036 and beyond, thus maximizing the value of the mineral resource.

•	Reduce Environmental Social Footprint: The strategic approach prioritizes reducing environmental and social impact through operation within the current footprint, increasing water availability through filtration, optimizing space use, and minimizing risks associated with tailings dams. Commingling contributes not only to technical and economic efficiency but also to sustainability objectives and corporate social responsibility.

•	Reduce Capital and Operational Cost: The cost reduction strategy focuses on optimizing capital investments (CAPEX) and their schedule, as well as minimizing operational costs through strategic facility location and leveraging synergies to avoid duplicate infrastructure. This enables more efficient and profitable resource management.

During the strategic planning phase, conceptual foundations for commingling implementation are established, including site characterization, regulatory framework development, and technical-economic feasibility analysis.

13	ANALYSIS OF THE MINING SYSTEM WITH COMMINGLING APPLICATION

13.1	Methodological Process Structure

The methodological process is organized in sequential stages that ensure effective integration of commingling into mine planning:

•	Comprehensive material evaluation: Detailed analysis of geotechnical and geochemical characteristics of available materials, identifying their suitability for mixtures and joint disposal.

•	Mixture design development: Design of optimized mixtures that maximize desired properties of commingled material, such as low permeability and high stability, minimizing acid water generation and improving mechanical behavior of the deposit.

•	Pilot test implementation: Execution of pilot and industrial scale tests to validate mixture performance and adjust design parameters according to results obtained under real operating conditions.

•	Monitoring and adjustment: Establishment of continuous monitoring system to evaluate consolidation, stability, and geochemical behavior of deposits, allowing operational and design adjustments as new information is collected.

14	IMPLEMENTATION AND EVALUATION METHODOLOGY FOR COMMINGLING IN MINE PLANNING

14.1	Comparative Case Analysis

The methodology applied at Antamina is based on Comparative Case Analysis, allowing for comprehensive review of the design and sequencing process of mining infrastructure, with emphasis on dams, waste rock dumps, and integration of commingling technologies. This methodological approach comprises:

•	Review of design and sequencing processes: Analysis of procedures used in waste dump planning and disposal, considering both geotechnical and geochemical criteria to ensure infrastructure stability and sustainability.

•	Identification of key variables: Selection of fundamental operational parameters, such as granulometry, permeability, geochemical behavior, and storage capacity, which directly affect the viability and performance of dumps and material mixtures.

•	Definition of case tree: Structure of a decision tree that contemplates long-term strategic scenarios, allowing comparison of alternatives under different combinations of variables and operational constraints.

•	Results evaluation: Application of sensitivity analyses and cost-benefit evaluations to determine the impact of each alternative on mine life extension, environmental risk reduction, and resource optimization.

14.2	Decision Tree for Strategic Planning

The decision tree developed for long-term strategic planning at Antamina follows sequential logic that allows evaluation of feasibility and impact of main operational alternatives:

•	Commingling feasibility: The first node of the tree determines whether integration of tailings and waste rock is technically and economically viable, considering material characteristics and regulatory constraints.

•	Entry into new area: If commingling is feasible, the next node evaluates the possibility of disposing material in a new area, analyzing topographic, environmental, and access aspects.

•	Need for new dam: Finally, it determines whether construction of a new tailings dam is essential or if the commingling solution allows dispensing with this infrastructure, optimizing investment and reducing environmental impact.

Each of these decisions conditions mine life extension, efficient space use, and environmental risk management, allowing selection of the alternative that maximizes economic, social, and environmental value of the project.

Figure 3  Decision Tree for Long-Term Strategic Scenario Planning

14.3	Evaluation of Operational Parameters and Modifications by Commingling

Key operational parameters—including waste rock-tailings ratio, mixing methods, transport systems, and processing criteria—are assessed for their impact on commingled system efficiency. The evaluation considers site-specific challenges such as extreme Andean weather, high-altitude logistics, and Peruvian regulations, all of which are especially critical at Antamina and require customized commingling solutions.

•	Modifications to tailings management by commingling:

The commingling process fundamentally transforms tailings management by utilizing voids within waste rock for storage, with available space typically equivalent to about 20% of the crushed rock volume transported by conveyors. To enable mixing, tailings must first be dewatered. The following image shows both the current process (without mechanization) and the transformation that occurs when commingling is incorporated.

Figure 4 Transformation of Tailings Management: Evolution from Conventional Process to Commingling|

•	 Operational parameters and assumptions

Commingling extends tailings facility life by optimizing spatial use of waste rock voids. Implementation requires mechanized systems (WCCS) for transporting crushed material mixed with dewatered tailings, while maintaining two key operational parameters: a) Unchanged transport capacity: 83.2 Mtpa in the conveyor system and b) Volumetric stability: No increase in total volume of waste rock dump.

This approach ensures efficient resource utilization and supports sustainable mine operations.

Table 1: Key operational parameters and assumptions

Component	Units	Without - Case Conventional	With - Case Commingling

Cycloned/Filtered Max. Capacity	Mtpa	-	32.0

Percentage of Voids in Waste Rock	% Volume 	 	20 %

Waste Rock / Tailings Ratio 	Mt Rock/

Mt Tails	-	83 / 16

Conventional Tailings Density	t/m3 	1.81	1.81

Fines Tailings Density	 t/m3	NA	1.60

Tailings Volume, TDR 4195 (to Dic-22)	Mm3	376	376

Tailings Capacity, TDR 4195 (to Dic-22)	Mt	682	652

Mill Feed Rate 	Ktpd	145	145

The following image shows how incorporation of mechanized systems and commingling add capacity, and therefore extend tailings facility life, to increase the life of mine and therefore the overall asset value.

Figure 5 Commingling Extension of Tailings Facility Life through Commingling

The curve below shows the relationship between dry density and optimal mixing proportion, determinant for maximizing commingling efficiency.

Figure 6: Dry density curve vs. optimal mixing proportion for commingling

•	Analysis of Mineable Resources, Production and Value

A three-dimensional diagram is used to strategically analyze the interactions between mineable resource volume, production rate, and generated value—whether economic, environmental, or operational. This tool helps identify how increasing resource volume and production can maximize value, but also highlights key constraints such as tailings dam capacity and property limits. The approach supports comprehensive decision-making to optimize asset value and ensure project sustainability across various operational scenarios.

Figure 7 Value Optimization: Interaction between Resource Volume, Production Rate and Key Restrictions

15	ECONOMIC AND OPERATIONAL ADVANTAGES

15.1	Perspective of Conventional Value Drivers

The following figure illustrates Antamina's main conventional value drivers from a financial perspective. In the current scenario, the amount of mineral resource that can be utilized directly depends on the capital that the company decides to invest (Capex), both in new investments (Growth Capex) and in maintaining existing operations (Sustaining Capex). This generates a necessary balance to determine the optimal scenario that maximizes the value of the existing asset.

Figure 8: Conventional Antamina Value Drivers: Financial Perspective (Without Commingling)

15.2	Impact of Commingling on Financial Value Drivers

From a financial perspective, the adoption of commingling among Antamina’s value drivers removes constraints on mineral resources by increasing tailings storage capacity, enabling the utilization of new dumps, and allowing for an expansion in the volume of usable resources. As a result, the mine’s operational life is extended and asset value is enhanced. Regarding dump utilization, the implementation of commingling has the potential to reduce the requirement for dumps located in karstic zones, which demand significant capital expenditure (Capex). This can lead to a reduction in initial investment by up to 30% in scenarios comparing commingling versus non-commingling approaches. This saving is achieved through the integration of previously separate infrastructures into a single system, which optimizes the use of available area in the East Extension dumps and prioritizes the use of mechanized systems over conventional trucking methods.

Figure 9: Antamina Value Drivers: Financial Perspective with Commingling

The optimization of land use represents another important economic benefit, especially relevant in mountainous locations like Antamina where available land for mining facilities is limited. Commingling allows for the creation of deposits with a smaller footprint compared to separate facilities, freeing up land for other productive uses or reducing the need for additional land acquisition.

16	ALIGNMENT WITH GISTM STANDARDS AND SUSTAINABILITY

The integration of commingling into mine planning strengthens compliance with the Global Industry Standard on Tailings Management (GISTM), by facilitating safer and more sustainable management of waste materials. The GISTM requires the planning, construction, operation, and closure of tailings facilities with a focus on risk reduction and continuous monitoring throughout the lifecycle. Commingling contributes to this objective by improving the geotechnical and geochemical stability of deposits, reducing the risk of failure and acid drainage generation, which in turn reduces environmental impact and long-term treatment costs. Additionally, the consolidation of facilities through commingling reduces the environmental footprint and facilitates supervision and control, aligning with the principles of sustainability and social responsibility in the mining industry.

17	LESSONS LEARNED AND CHALLENGES

Among the main lessons learned and challenges of implementing commingling in mine planning at Antamina, the following stand out:

•	Large-scale operations and mechanization: Commingling is primarily viable in large-volume mechanized operations, where it is possible to achieve a homogeneous and controlled mixture of materials. However, the magnitude of waste rock and tailings tonnages at Antamina implies a new challenge, never seen before.

•	Flexible planning and pilot tests: It is essential to include areas for pilot tests in short and medium-term plans, allowing adjustment of mixing parameters according to material variability.

•	Regulatory communication: Technology acceptance requires identifying precedents and working closely with authorities to develop appropriate regulatory frameworks.

•	Organizational change management: The transition to commingling involves challenges in personnel training and adaptation of operational processes, especially in mature operations.

18	CONCLUSIONS

•	The comparative discussion and case analysis of commingling implementation at Antamina have enabled the identification and selection of strategic alternatives that maximize the economic, environmental, and social value of the project.

•	The use of decision trees and scenario evaluation has facilitated informed decision-making, taking into account technical and economic feasibility as well as regulatory and environmental constraints.

•	The integration of commingling into strategic mine planning has proven to be a key tool for optimizing waste rock and tailings management, achieving a significant reduction in capital costs (up to 30%), greater efficiency in land use, and an extension of tailings storage facility (TSF) life. This approach has also aligned operations with international sustainability standards (GISTM) and strengthened risk management, contributing to operational continuity and long-term reduction of environmental impacts.

•	The Antamina experience demonstrates that structured case discussions and the application of comparative methodologies are fundamental for strategic planning, as they allow anticipation of challenges, validation of solutions through pilot testing, and adjustment of operational parameters based on real outcomes. Thus, commingling is consolidated as a transformative practice in modern mining, fostering more robust and sustainable decision-making for the development of long-life mining assets.

ACKNOWLEDGEMENTS

The authors express their sincere gratitude to Antamina for their invaluable support. Special thanks are extended to the Tailings Strategy Management team and the Long-Term Planning team for their collaboration and insights.

REFERENCES

Boshoff, J. et al. (2023). A case study on the commingling of tailings and waste rock at a Brownfields open cast mine in Ghana. University of Alberta.

Burden, R., & Wilson, G. W. (2023). Commingling of waste rock and tailings to improve “dry stack” performance: Design and evaluation of mixtures. Minerals, 13(2), 295.

Deza, N., & Montes, K. (2022). Mecanización del desmonte en Compañía Minera Antamina: Propuesta para capturar el valor tangible e intangible de la mecanización. Compañía Minera Antamina.

Green Policy Platform. (2024). Knowledge Gaps Report: Environmental Aspects of Tailings Management (January 2024).

ICMM. (2020). New global industry standard on tailings management. International Council on Mining and Metals.

Ulrich, B. & Coffin, J. (2015). TMW 2015 – Combined Tailings and Mine Waste.

Minemax. (2015). Why mine planning is all about collaboration. Minemax News.

Wheaton Precious Metals. (2015). Antamina Project overview.

INTEGRATING COMMINGLING INTO THE STRATEGIC WASTE ROCK PLANNING AT ANTAMINA MINE

Christa Quiroz1, David Machin1, Olimpia Cabrera1, Fernando Angeles1

1 Antamina Mine, Lima, Peru

(*Presenting author: cquirozc@antamina.com)

ABSTRACT

Antamina is a large open-pit polymetallic mine in Peru, operating with a processing capacity of 145 ktpd. In operation since 2001, the current mine life extends to 2036. Mining is conducted at a rate of 290 Mtpa using conventional truck-and-shovel equipment. This technical paper presents a comprehensive assessment of commingling as a transformative technology in mine planning, positioning waste rock and tailings management as a key factor in the decision-making process. The primary objective of this study is to demonstrate the benefits of this technology within the mine planning process, highlighting waste rock and tailings management as one of the main drivers in strategic decision-making. The methodology applied is a Comparative Case Analysis, which includes a review of the design and sequencing process for mining infrastructure such as waste rock storage facilities, along with a discussion of the key parameters considered. The results show that the implementation of this technology delivers significant economic benefits, including an approximate 30% reduction in capital costs for waste rock storage facilities, improved land-use efficiency, and optimized infrastructure development. Furthermore, the approach enhances alignment with the Global Industry Standard on Tailings Management (GISTM).

KEYWORDS

commingling, mine planning, waste rock management, tailings management, GISTM compliance

1	INTRODUCTION

The mining industry faces growing challenges related to the sustainable management of waste rock and tailings, especially after catastrophic events such as those that occurred in Brumadinho and Samarco (Green Policy Platform, 2024; ICMM, 2020). In this context, the Antamina mine, operating since 2001 as one of the largest polymetallic operations in the world, has developed a pioneering approach that integrates commingling as a central element of its strategic planning (Wheaton Precious Metals, 2015).

Commingling, defined as the homogeneous mixing of dewatered tailings with waste rock to form a geotechnically stable material, represents a natural evolution of traditional co-disposal practices (Burden & Wilson, 2023). However, what distinguishes the Antamina case is the integration of this technology from the initial phases of mine planning, making waste rock management the main driver of operational and strategic decisions.

2	OBJECTIVES

•	Demonstrate the benefits of integrating commingling into mine planning, positioning waste rock and tailings management as a strategic driver for decision-making in open-pit operations.

•	Optimize operational efficiency and environmental sustainability through the adoption of commingling in waste rock and tailings management.

•	Align waste rock and tailings management with international sustainability and safety standards, such as the GISTM, through the application of commingling.

•	Analyze, through comparative case studies, the impact of commingling on land-use optimization and mine life extension.

•	Reduce capital and operating costs associated with waste rock and tailings disposal by implementing commingling technologies.

•	Present the key findings and lessons learned from the integration of commingling into long-term mine planning.

3	TECHNICAL FOUNDATIONS OF COMMINGLING IN MINE PLANNING

3.1	Desing Principles and Material Characterization

Commingling at Antamina is based on engineering principles that seek to combine the superior structural properties of waste rock with the low permeability characteristics of tailings (Boshoff, 2023). This combination results in a material with shear strength similar to waste rock and permeability comparable to tailings, creating conditions that restrict oxygen entry and water filtration, significantly reducing the potential for acid drainage generation (Ulrich & Coffin, 2015; Burden & Wilson, 2023).

The characterization of commingled material is performed through analysis of five distinct structural configurations: rock-dominated uncompacted, fines-dominated uncompacted, rock-dominated compacted, fines-dominated compacted, and the "just-filled" configuration where fines exactly fill the voids between rock particles (Burden & Wilson, 2023). This latter configuration represents the optimal target of the commingling process at Antamina.

Figure 1 Blend configurations after Wickland et al. (2006). Source: Burden & Wilson (2023).

3.2	Integration into the Strategic Planning Process

The integration of commingling into Antamina's mine planning follows a structured methodological approach that places tailings management with waste rock as central elements of decision-making. This global strategy allows for optimizing the mixture of materials from the mine to the plant, considering the operational and environmental constraints associated with the tailings storage facility. By incorporating tailings behavior in the early stages of planning, greater efficiency in resource use and better risk management are achieved.

The global strategy is divided into three key axes:

•	Strategy 1: Ensure Future Expansions: The first strategy focuses on maximizing tailings storage capacity for current and future needs, leveraging Antamina's mineral resource and applying advanced technology to increase TSF (Tailings Storage Facility) capacity to ensure the mine's long-term operational continuity.

•	Strategy 2: Optimized footprint: The second strategy seeks to optimize the overall project footprint by balancing waste and tailings storage space. This approach aligns directly with commingling principles by maximizing efficient use of available land within the current operational footprint.

•	Strategy 3: Reduce upfront CAPEX: The third strategy focuses on reducing initial CAPEX to improve key financial indicators (IRR), which directly aligns with the demonstrated economic benefits of commingling, minimizing operational costs with a nearby TSF and optimizing CAPEX timing.

Figure 2 Strategic vision of mine planning approach with commingling

The strategies implemented in tailings dam and waste dump management seek to maximize the operational life of the mine and optimize asset value. These decisions translate into concrete actions that address sustainability, operational efficiency, and cost reduction, ensuring continuity and long-term success of the mining operation.

•	Long Life Asset: Antamina's vision as a long-life asset involves making decisions based on sustainable exploitation of its exceptional mineral resources. This includes adopting advanced technologies, such as commingling, to improve storage capacity and ensure operational continuity until 2036 and beyond, thus maximizing the value of the mineral resource.

•	Reduce Environmental Social Footprint: The strategic approach prioritizes reducing environmental and social impact through operation within the current footprint, increasing water availability through filtration, optimizing space use, and minimizing risks associated with tailings dams. Commingling contributes not only to technical and economic efficiency but also to sustainability objectives and corporate social responsibility.

•	Reduce Capital and Operational Cost: The cost reduction strategy focuses on optimizing capital investments (CAPEX) and their schedule, as well as minimizing operational costs through strategic facility location and leveraging synergies to avoid duplicate infrastructure. This enables more efficient and profitable resource management.

During the strategic planning phase, conceptual foundations for commingling implementation are established, including site characterization, regulatory framework development, and technical-economic feasibility analysis.

4	ANALYSIS OF THE MINING SYSTEM WITH COMMINGLING APPLICATION

4.1	Methodological Process Structure

The methodological process is organized in sequential stages that ensure effective integration of commingling into mine planning:

•	Comprehensive material evaluation: Detailed analysis of geotechnical and geochemical characteristics of available materials, identifying their suitability for mixtures and joint disposal.

•	Mixture design development: Design of optimized mixtures that maximize desired properties of commingled material, such as low permeability and high stability, minimizing acid water generation and improving mechanical behavior of the deposit.

•	Pilot test implementation: Execution of pilot and industrial scale tests to validate mixture performance and adjust design parameters according to results obtained under real operating conditions.

•	Monitoring and adjustment: Establishment of continuous monitoring system to evaluate consolidation, stability, and geochemical behavior of deposits, allowing operational and design adjustments as new information is collected.

5	IMPLEMENTATION AND EVALUATION METHODOLOGY FOR COMMINGLING IN MINE PLANNING

5.1	Comparative Case Analysis

The methodology applied at Antamina is based on Comparative Case Analysis, allowing for comprehensive review of the design and sequencing process of mining infrastructure, with emphasis on dams, waste rock dumps, and integration of commingling technologies. This methodological approach comprises:

•	Review of design and sequencing processes: Analysis of procedures used in waste dump planning and disposal, considering both geotechnical and geochemical criteria to ensure infrastructure stability and sustainability.

•	Identification of key variables: Selection of fundamental operational parameters, such as granulometry, permeability, geochemical behavior, and storage capacity, which directly affect the viability and performance of dumps and material mixtures.

•	Definition of case tree: Structure of a decision tree that contemplates long-term strategic scenarios, allowing comparison of alternatives under different combinations of variables and operational constraints.

•	Results evaluation: Application of sensitivity analyses and cost-benefit evaluations to determine the impact of each alternative on mine life extension, environmental risk reduction, and resource optimization.

5.2	Decision Tree for Strategic Planning

The decision tree developed for long-term strategic planning at Antamina follows sequential logic that allows evaluation of feasibility and impact of main operational alternatives:

•	Commingling feasibility: The first node of the tree determines whether integration of tailings and waste rock is technically and economically viable, considering material characteristics and regulatory constraints.

•	Entry into new area: If commingling is feasible, the next node evaluates the possibility of disposing material in a new area, analyzing topographic, environmental, and access aspects.

•	Need for new dam: Finally, it determines whether construction of a new tailings dam is essential or if the commingling solution allows dispensing with this infrastructure, optimizing investment and reducing environmental impact.

Each of these decisions conditions mine life extension, efficient space use, and environmental risk management, allowing selection of the alternative that maximizes economic, social, and environmental value of the project.

Figure 3  Decision Tree for Long-Term Strategic Scenario Planning

5.3	Evaluation of Operational Parameters and Modifications by Commingling

Key operational parameters—including waste rock-tailings ratio, mixing methods, transport systems, and processing criteria—are assessed for their impact on commingled system efficiency. The evaluation considers site-specific challenges such as extreme Andean weather, high-altitude logistics, and Peruvian regulations, all of which are especially critical at Antamina and require customized commingling solutions.

•	Modifications to tailings management by commingling:

The commingling process fundamentally transforms tailings management by utilizing voids within waste rock for storage, with available space typically equivalent to about 20% of the crushed rock volume transported by conveyors. To enable mixing, tailings must first be dewatered. The following image shows both the current process (without mechanization) and the transformation that occurs when commingling is incorporated.

Figure 4 Transformation of Tailings Management: Evolution from Conventional Process to Commingling|

•	 Operational parameters and assumptions

Commingling extends tailings facility life by optimizing spatial use of waste rock voids. Implementation requires mechanized systems (WCCS) for transporting crushed material mixed with dewatered tailings, while maintaining two key operational parameters: a) Unchanged transport capacity: 83.2 Mtpa in the conveyor system and b) Volumetric stability: No increase in total volume of waste rock dump.

This approach ensures efficient resource utilization and supports sustainable mine operations.

Table 1: Key operational parameters and assumptions

Component	Units	Without - Case Conventional	With - Case Commingling

Cycloned/Filtered Max. Capacity	Mtpa	-	32.0

Percentage of Voids in Waste Rock	% Volume 	 	20 %

Waste Rock / Tailings Ratio 	Mt Rock/

Mt Tails	-	83 / 16

Conventional Tailings Density	t/m3 	1.81	1.81

Fines Tailings Density	 t/m3	NA	1.60

Tailings Volume, TDR 4195 (to Dic-22)	Mm3	376	376

Tailings Capacity, TDR 4195 (to Dic-22)	Mt	682	652

Mill Feed Rate 	Ktpd	145	145

The following image shows how incorporation of mechanized systems and commingling add capacity, and therefore extend tailings facility life, to increase the life of mine and therefore the overall asset value.

Figure 5 Commingling Extension of Tailings Facility Life through Commingling

The curve below shows the relationship between dry density and optimal mixing proportion, determinant for maximizing commingling efficiency.

Figure 6: Dry density curve vs. optimal mixing proportion for commingling

•	Analysis of Mineable Resources, Production and Value

A three-dimensional diagram is used to strategically analyze the interactions between mineable resource volume, production rate, and generated value—whether economic, environmental, or operational. This tool helps identify how increasing resource volume and production can maximize value, but also highlights key constraints such as tailings dam capacity and property limits. The approach supports comprehensive decision-making to optimize asset value and ensure project sustainability across various operational scenarios.

Figure 7 Value Optimization: Interaction between Resource Volume, Production Rate and Key Restrictions

6	ECONOMIC AND OPERATIONAL ADVANTAGES

6.1	Perspective of Conventional Value Drivers

The following figure illustrates Antamina's main conventional value drivers from a financial perspective. In the current scenario, the amount of mineral resource that can be utilized directly depends on the capital that the company decides to invest (Capex), both in new investments (Growth Capex) and in maintaining existing operations (Sustaining Capex). This generates a necessary balance to determine the optimal scenario that maximizes the value of the existing asset.

Figure 8: Conventional Antamina Value Drivers: Financial Perspective (Without Commingling)

6.2	Impact of Commingling on Financial Value Drivers

From a financial perspective, the adoption of commingling among Antamina’s value drivers removes constraints on mineral resources by increasing tailings storage capacity, enabling the utilization of new dumps, and allowing for an expansion in the volume of usable resources. As a result, the mine’s operational life is extended and asset value is enhanced. Regarding dump utilization, the implementation of commingling has the potential to reduce the requirement for dumps located in karstic zones, which demand significant capital expenditure (Capex). This can lead to a reduction in initial investment by up to 30% in scenarios comparing commingling versus non-commingling approaches. This saving is achieved through the integration of previously separate infrastructures into a single system, which optimizes the use of available area in the East Extension dumps and prioritizes the use of mechanized systems over conventional trucking methods.

Figure 9: Antamina Value Drivers: Financial Perspective with Commingling

The optimization of land use represents another important economic benefit, especially relevant in mountainous locations like Antamina where available land for mining facilities is limited. Commingling allows for the creation of deposits with a smaller footprint compared to separate facilities, freeing up land for other productive uses or reducing the need for additional land acquisition.

7	ALIGNMENT WITH GISTM STANDARDS AND SUSTAINABILITY

The integration of commingling into mine planning strengthens compliance with the Global Industry Standard on Tailings Management (GISTM), by facilitating safer and more sustainable management of waste materials. The GISTM requires the planning, construction, operation, and closure of tailings facilities with a focus on risk reduction and continuous monitoring throughout the lifecycle. Commingling contributes to this objective by improving the geotechnical and geochemical stability of deposits, reducing the risk of failure and acid drainage generation, which in turn reduces environmental impact and long-term treatment costs. Additionally, the consolidation of facilities through commingling reduces the environmental footprint and facilitates supervision and control, aligning with the principles of sustainability and social responsibility in the mining industry.

8	LESSONS LEARNED AND CHALLENGES

Among the main lessons learned and challenges of implementing commingling in mine planning at Antamina, the following stand out:

•	Large-scale operations and mechanization: Commingling is primarily viable in large-volume mechanized operations, where it is possible to achieve a homogeneous and controlled mixture of materials. However, the magnitude of waste rock and tailings tonnages at Antamina implies a new challenge, never seen before.

•	Flexible planning and pilot tests: It is essential to include areas for pilot tests in short and medium-term plans, allowing adjustment of mixing parameters according to material variability.

•	Regulatory communication: Technology acceptance requires identifying precedents and working closely with authorities to develop appropriate regulatory frameworks.

•	Organizational change management: The transition to commingling involves challenges in personnel training and adaptation of operational processes, especially in mature operations.

9	CONCLUSIONS

•	The comparative discussion and case analysis of commingling implementation at Antamina have enabled the identification and selection of strategic alternatives that maximize the economic, environmental, and social value of the project.

•	The use of decision trees and scenario evaluation has facilitated informed decision-making, taking into account technical and economic feasibility as well as regulatory and environmental constraints.

•	The integration of commingling into strategic mine planning has proven to be a key tool for optimizing waste rock and tailings management, achieving a significant reduction in capital costs (up to 30%), greater efficiency in land use, and an extension of tailings storage facility (TSF) life. This approach has also aligned operations with international sustainability standards (GISTM) and strengthened risk management, contributing to operational continuity and long-term reduction of environmental impacts.

•	The Antamina experience demonstrates that structured case discussions and the application of comparative methodologies are fundamental for strategic planning, as they allow anticipation of challenges, validation of solutions through pilot testing, and adjustment of operational parameters based on real outcomes. Thus, commingling is consolidated as a transformative practice in modern mining, fostering more robust and sustainable decision-making for the development of long-life mining assets.

ACKNOWLEDGEMENTS

The authors express their sincere gratitude to Antamina for their invaluable support. Special thanks are extended to the Tailings Strategy Management team and the Long-Term Planning team for their collaboration and insights.

REFERENCES

Boshoff, J. et al. (2023). A case study on the commingling of tailings and waste rock at a Brownfields open cast mine in Ghana. University of Alberta.

Burden, R., & Wilson, G. W. (2023). Commingling of waste rock and tailings to improve “dry stack” performance: Design and evaluation of mixtures. Minerals, 13(2), 295.

Deza, N., & Montes, K. (2022). Mecanización del desmonte en Compañía Minera Antamina: Propuesta para capturar el valor tangible e intangible de la mecanización. Compañía Minera Antamina.

Green Policy Platform. (2024). Knowledge Gaps Report: Environmental Aspects of Tailings Management (January 2024).

ICMM. (2020). New global industry standard on tailings management. International Council on Mining and Metals.

Ulrich, B. & Coffin, J. (2015). TMW 2015 – Combined Tailings and Mine Waste.

Minemax. (2015). Why mine planning is all about collaboration. Minemax News.

Wheaton Precious Metals. (2015). Antamina Project overview.

INTEGRATING COMMINGLING INTO THE STRATEGIC WASTE ROCK PLANNING AT ANTAMINA MINE

Christa Quiroz1, David Machin1, Olimpia Cabrera1, Fernando Angeles1

1 Antamina Mine, Lima, Peru

(*Presenting author: cquirozc@antamina.com)

ABSTRACT

Antamina is a large open-pit polymetallic mine in Peru, operating with a processing capacity of 145 ktpd. In operation since 2001, the current mine life extends to 2036. Mining is conducted at a rate of 290 Mtpa using conventional truck-and-shovel equipment. This technical paper presents a comprehensive assessment of commingling as a transformative technology in mine planning, positioning waste rock and tailings management as a key factor in the decision-making process. The primary objective of this study is to demonstrate the benefits of this technology within the mine planning process, highlighting waste rock and tailings management as one of the main drivers in strategic decision-making. The methodology applied is a Comparative Case Analysis, which includes a review of the design and sequencing process for mining infrastructure such as waste rock storage facilities, along with a discussion of the key parameters considered. The results show that the implementation of this technology delivers significant economic benefits, including an approximate 30% reduction in capital costs for waste rock storage facilities, improved land-use efficiency, and optimized infrastructure development. Furthermore, the approach enhances alignment with the Global Industry Standard on Tailings Management (GISTM).

KEYWORDS

commingling, mine planning, waste rock management, tailings management, GISTM compliance

10	INTRODUCTION

The mining industry faces growing challenges related to the sustainable management of waste rock and tailings, especially after catastrophic events such as those that occurred in Brumadinho and Samarco (Green Policy Platform, 2024; ICMM, 2020). In this context, the Antamina mine, operating since 2001 as one of the largest polymetallic operations in the world, has developed a pioneering approach that integrates commingling as a central element of its strategic planning (Wheaton Precious Metals, 2015).

Commingling, defined as the homogeneous mixing of dewatered tailings with waste rock to form a geotechnically stable material, represents a natural evolution of traditional co-disposal practices (Burden & Wilson, 2023). However, what distinguishes the Antamina case is the integration of this technology from the initial phases of mine planning, making waste rock management the main driver of operational and strategic decisions.

11	OBJECTIVES

•	Demonstrate the benefits of integrating commingling into mine planning, positioning waste rock and tailings management as a strategic driver for decision-making in open-pit operations.

•	Optimize operational efficiency and environmental sustainability through the adoption of commingling in waste rock and tailings management.

•	Align waste rock and tailings management with international sustainability and safety standards, such as the GISTM, through the application of commingling.

•	Analyze, through comparative case studies, the impact of commingling on land-use optimization and mine life extension.

•	Reduce capital and operating costs associated with waste rock and tailings disposal by implementing commingling technologies.

•	Present the key findings and lessons learned from the integration of commingling into long-term mine planning.

12	TECHNICAL FOUNDATIONS OF COMMINGLING IN MINE PLANNING

12.1	Desing Principles and Material Characterization

Commingling at Antamina is based on engineering principles that seek to combine the superior structural properties of waste rock with the low permeability characteristics of tailings (Boshoff, 2023). This combination results in a material with shear strength similar to waste rock and permeability comparable to tailings, creating conditions that restrict oxygen entry and water filtration, significantly reducing the potential for acid drainage generation (Ulrich & Coffin, 2015; Burden & Wilson, 2023).

The characterization of commingled material is performed through analysis of five distinct structural configurations: rock-dominated uncompacted, fines-dominated uncompacted, rock-dominated compacted, fines-dominated compacted, and the "just-filled" configuration where fines exactly fill the voids between rock particles (Burden & Wilson, 2023). This latter configuration represents the optimal target of the commingling process at Antamina.

Figure 1 Blend configurations after Wickland et al. (2006). Source: Burden & Wilson (2023).

12.2	Integration into the Strategic Planning Process

The integration of commingling into Antamina's mine planning follows a structured methodological approach that places tailings management with waste rock as central elements of decision-making. This global strategy allows for optimizing the mixture of materials from the mine to the plant, considering the operational and environmental constraints associated with the tailings storage facility. By incorporating tailings behavior in the early stages of planning, greater efficiency in resource use and better risk management are achieved.

The global strategy is divided into three key axes:

•	Strategy 1: Ensure Future Expansions: The first strategy focuses on maximizing tailings storage capacity for current and future needs, leveraging Antamina's mineral resource and applying advanced technology to increase TSF (Tailings Storage Facility) capacity to ensure the mine's long-term operational continuity.

•	Strategy 2: Optimized footprint: The second strategy seeks to optimize the overall project footprint by balancing waste and tailings storage space. This approach aligns directly with commingling principles by maximizing efficient use of available land within the current operational footprint.

•	Strategy 3: Reduce upfront CAPEX: The third strategy focuses on reducing initial CAPEX to improve key financial indicators (IRR), which directly aligns with the demonstrated economic benefits of commingling, minimizing operational costs with a nearby TSF and optimizing CAPEX timing.

Figure 2 Strategic vision of mine planning approach with commingling

The strategies implemented in tailings dam and waste dump management seek to maximize the operational life of the mine and optimize asset value. These decisions translate into concrete actions that address sustainability, operational efficiency, and cost reduction, ensuring continuity and long-term success of the mining operation.

•	Long Life Asset: Antamina's vision as a long-life asset involves making decisions based on sustainable exploitation of its exceptional mineral resources. This includes adopting advanced technologies, such as commingling, to improve storage capacity and ensure operational continuity until 2036 and beyond, thus maximizing the value of the mineral resource.

•	Reduce Environmental Social Footprint: The strategic approach prioritizes reducing environmental and social impact through operation within the current footprint, increasing water availability through filtration, optimizing space use, and minimizing risks associated with tailings dams. Commingling contributes not only to technical and economic efficiency but also to sustainability objectives and corporate social responsibility.

•	Reduce Capital and Operational Cost: The cost reduction strategy focuses on optimizing capital investments (CAPEX) and their schedule, as well as minimizing operational costs through strategic facility location and leveraging synergies to avoid duplicate infrastructure. This enables more efficient and profitable resource management.

During the strategic planning phase, conceptual foundations for commingling implementation are established, including site characterization, regulatory framework development, and technical-economic feasibility analysis.

13	ANALYSIS OF THE MINING SYSTEM WITH COMMINGLING APPLICATION

13.1	Methodological Process Structure

The methodological process is organized in sequential stages that ensure effective integration of commingling into mine planning:

•	Comprehensive material evaluation: Detailed analysis of geotechnical and geochemical characteristics of available materials, identifying their suitability for mixtures and joint disposal.

•	Mixture design development: Design of optimized mixtures that maximize desired properties of commingled material, such as low permeability and high stability, minimizing acid water generation and improving mechanical behavior of the deposit.

•	Pilot test implementation: Execution of pilot and industrial scale tests to validate mixture performance and adjust design parameters according to results obtained under real operating conditions.

•	Monitoring and adjustment: Establishment of continuous monitoring system to evaluate consolidation, stability, and geochemical behavior of deposits, allowing operational and design adjustments as new information is collected.

14	IMPLEMENTATION AND EVALUATION METHODOLOGY FOR COMMINGLING IN MINE PLANNING

14.1	Comparative Case Analysis

The methodology applied at Antamina is based on Comparative Case Analysis, allowing for comprehensive review of the design and sequencing process of mining infrastructure, with emphasis on dams, waste rock dumps, and integration of commingling technologies. This methodological approach comprises:

•	Review of design and sequencing processes: Analysis of procedures used in waste dump planning and disposal, considering both geotechnical and geochemical criteria to ensure infrastructure stability and sustainability.

•	Identification of key variables: Selection of fundamental operational parameters, such as granulometry, permeability, geochemical behavior, and storage capacity, which directly affect the viability and performance of dumps and material mixtures.

•	Definition of case tree: Structure of a decision tree that contemplates long-term strategic scenarios, allowing comparison of alternatives under different combinations of variables and operational constraints.

•	Results evaluation: Application of sensitivity analyses and cost-benefit evaluations to determine the impact of each alternative on mine life extension, environmental risk reduction, and resource optimization.

14.2	Decision Tree for Strategic Planning

The decision tree developed for long-term strategic planning at Antamina follows sequential logic that allows evaluation of feasibility and impact of main operational alternatives:

•	Commingling feasibility: The first node of the tree determines whether integration of tailings and waste rock is technically and economically viable, considering material characteristics and regulatory constraints.

•	Entry into new area: If commingling is feasible, the next node evaluates the possibility of disposing material in a new area, analyzing topographic, environmental, and access aspects.

•	Need for new dam: Finally, it determines whether construction of a new tailings dam is essential or if the commingling solution allows dispensing with this infrastructure, optimizing investment and reducing environmental impact.

Each of these decisions conditions mine life extension, efficient space use, and environmental risk management, allowing selection of the alternative that maximizes economic, social, and environmental value of the project.

Figure 3  Decision Tree for Long-Term Strategic Scenario Planning

14.3	Evaluation of Operational Parameters and Modifications by Commingling

Key operational parameters—including waste rock-tailings ratio, mixing methods, transport systems, and processing criteria—are assessed for their impact on commingled system efficiency. The evaluation considers site-specific challenges such as extreme Andean weather, high-altitude logistics, and Peruvian regulations, all of which are especially critical at Antamina and require customized commingling solutions.

•	Modifications to tailings management by commingling:

The commingling process fundamentally transforms tailings management by utilizing voids within waste rock for storage, with available space typically equivalent to about 20% of the crushed rock volume transported by conveyors. To enable mixing, tailings must first be dewatered. The following image shows both the current process (without mechanization) and the transformation that occurs when commingling is incorporated.

Figure 4 Transformation of Tailings Management: Evolution from Conventional Process to Commingling|

•	 Operational parameters and assumptions

Commingling extends tailings facility life by optimizing spatial use of waste rock voids. Implementation requires mechanized systems (WCCS) for transporting crushed material mixed with dewatered tailings, while maintaining two key operational parameters: a) Unchanged transport capacity: 83.2 Mtpa in the conveyor system and b) Volumetric stability: No increase in total volume of waste rock dump.

This approach ensures efficient resource utilization and supports sustainable mine operations.

Table 1: Key operational parameters and assumptions

Component	Units	Without - Case Conventional	With - Case Commingling

Cycloned/Filtered Max. Capacity	Mtpa	-	32.0

Percentage of Voids in Waste Rock	% Volume 	 	20 %

Waste Rock / Tailings Ratio 	Mt Rock/

Mt Tails	-	83 / 16

Conventional Tailings Density	t/m3 	1.81	1.81

Fines Tailings Density	 t/m3	NA	1.60

Tailings Volume, TDR 4195 (to Dic-22)	Mm3	376	376

Tailings Capacity, TDR 4195 (to Dic-22)	Mt	682	652

Mill Feed Rate 	Ktpd	145	145

The following image shows how incorporation of mechanized systems and commingling add capacity, and therefore extend tailings facility life, to increase the life of mine and therefore the overall asset value.

Figure 5 Commingling Extension of Tailings Facility Life through Commingling

The curve below shows the relationship between dry density and optimal mixing proportion, determinant for maximizing commingling efficiency.

Figure 6: Dry density curve vs. optimal mixing proportion for commingling

•	Analysis of Mineable Resources, Production and Value

A three-dimensional diagram is used to strategically analyze the interactions between mineable resource volume, production rate, and generated value—whether economic, environmental, or operational. This tool helps identify how increasing resource volume and production can maximize value, but also highlights key constraints such as tailings dam capacity and property limits. The approach supports comprehensive decision-making to optimize asset value and ensure project sustainability across various operational scenarios.

Figure 7 Value Optimization: Interaction between Resource Volume, Production Rate and Key Restrictions

15	ECONOMIC AND OPERATIONAL ADVANTAGES

15.1	Perspective of Conventional Value Drivers

The following figure illustrates Antamina's main conventional value drivers from a financial perspective. In the current scenario, the amount of mineral resource that can be utilized directly depends on the capital that the company decides to invest (Capex), both in new investments (Growth Capex) and in maintaining existing operations (Sustaining Capex). This generates a necessary balance to determine the optimal scenario that maximizes the value of the existing asset.

Figure 8: Conventional Antamina Value Drivers: Financial Perspective (Without Commingling)

15.2	Impact of Commingling on Financial Value Drivers

From a financial perspective, the adoption of commingling among Antamina’s value drivers removes constraints on mineral resources by increasing tailings storage capacity, enabling the utilization of new dumps, and allowing for an expansion in the volume of usable resources. As a result, the mine’s operational life is extended and asset value is enhanced. Regarding dump utilization, the implementation of commingling has the potential to reduce the requirement for dumps located in karstic zones, which demand significant capital expenditure (Capex). This can lead to a reduction in initial investment by up to 30% in scenarios comparing commingling versus non-commingling approaches. This saving is achieved through the integration of previously separate infrastructures into a single system, which optimizes the use of available area in the East Extension dumps and prioritizes the use of mechanized systems over conventional trucking methods.

Figure 9: Antamina Value Drivers: Financial Perspective with Commingling

The optimization of land use represents another important economic benefit, especially relevant in mountainous locations like Antamina where available land for mining facilities is limited. Commingling allows for the creation of deposits with a smaller footprint compared to separate facilities, freeing up land for other productive uses or reducing the need for additional land acquisition.

16	ALIGNMENT WITH GISTM STANDARDS AND SUSTAINABILITY

The integration of commingling into mine planning strengthens compliance with the Global Industry Standard on Tailings Management (GISTM), by facilitating safer and more sustainable management of waste materials. The GISTM requires the planning, construction, operation, and closure of tailings facilities with a focus on risk reduction and continuous monitoring throughout the lifecycle. Commingling contributes to this objective by improving the geotechnical and geochemical stability of deposits, reducing the risk of failure and acid drainage generation, which in turn reduces environmental impact and long-term treatment costs. Additionally, the consolidation of facilities through commingling reduces the environmental footprint and facilitates supervision and control, aligning with the principles of sustainability and social responsibility in the mining industry.

17	LESSONS LEARNED AND CHALLENGES

Among the main lessons learned and challenges of implementing commingling in mine planning at Antamina, the following stand out:

•	Large-scale operations and mechanization: Commingling is primarily viable in large-volume mechanized operations, where it is possible to achieve a homogeneous and controlled mixture of materials. However, the magnitude of waste rock and tailings tonnages at Antamina implies a new challenge, never seen before.

•	Flexible planning and pilot tests: It is essential to include areas for pilot tests in short and medium-term plans, allowing adjustment of mixing parameters according to material variability.

•	Regulatory communication: Technology acceptance requires identifying precedents and working closely with authorities to develop appropriate regulatory frameworks.

•	Organizational change management: The transition to commingling involves challenges in personnel training and adaptation of operational processes, especially in mature operations.

18	CONCLUSIONS

•	The comparative discussion and case analysis of commingling implementation at Antamina have enabled the identification and selection of strategic alternatives that maximize the economic, environmental, and social value of the project.

•	The use of decision trees and scenario evaluation has facilitated informed decision-making, taking into account technical and economic feasibility as well as regulatory and environmental constraints.

•	The integration of commingling into strategic mine planning has proven to be a key tool for optimizing waste rock and tailings management, achieving a significant reduction in capital costs (up to 30%), greater efficiency in land use, and an extension of tailings storage facility (TSF) life. This approach has also aligned operations with international sustainability standards (GISTM) and strengthened risk management, contributing to operational continuity and long-term reduction of environmental impacts.

•	The Antamina experience demonstrates that structured case discussions and the application of comparative methodologies are fundamental for strategic planning, as they allow anticipation of challenges, validation of solutions through pilot testing, and adjustment of operational parameters based on real outcomes. Thus, commingling is consolidated as a transformative practice in modern mining, fostering more robust and sustainable decision-making for the development of long-life mining assets.

ACKNOWLEDGEMENTS

The authors express their sincere gratitude to Antamina for their invaluable support. Special thanks are extended to the Tailings Strategy Management team and the Long-Term Planning team for their collaboration and insights.

REFERENCES

Boshoff, J. et al. (2023). A case study on the commingling of tailings and waste rock at a Brownfields open cast mine in Ghana. University of Alberta.

Burden, R., & Wilson, G. W. (2023). Commingling of waste rock and tailings to improve “dry stack” performance: Design and evaluation of mixtures. Minerals, 13(2), 295.

Deza, N., & Montes, K. (2022). Mecanización del desmonte en Compañía Minera Antamina: Propuesta para capturar el valor tangible e intangible de la mecanización. Compañía Minera Antamina.

Green Policy Platform. (2024). Knowledge Gaps Report: Environmental Aspects of Tailings Management (January 2024).

ICMM. (2020). New global industry standard on tailings management. International Council on Mining and Metals.

Ulrich, B. & Coffin, J. (2015). TMW 2015 – Combined Tailings and Mine Waste.

Minemax. (2015). Why mine planning is all about collaboration. Minemax News.

Wheaton Precious Metals. (2015). Antamina Project overview.