Abstract

Title: Real-World Trial and Insights Gained from Flotation 360™ and the In-Situ Pulp Sensor at a Copper Plant

Topic: Mining 4.0

Category: Mineral Processing and Extractive Metallurgy

Author and Co-authors Information:

Haasbroek, A.L. ; Auret, L. ; Anes, J. J. ; Basurco, J.E.

Keywords: Flotation, Optimization, Pulp Sensor, Mining 4.0, Advanced Process Control, AI, Machine Learning.

Abstract:

Understanding the complex dynamics within a flotation cell has always been a significant challenge in mineral processing. The different phases within the cell, including the pulp, froth, and gas phases, interact in intricate ways that are difficult to monitor and control. Traditional methods often fall short in providing real-time, accurate data on these interactions, leading to suboptimal flotation performance.

The introduction of the patented in-situ pulp sensor marks a breakthrough in this area. This unique sensor provides real-time hydrodynamic data through sophisticated algorithms, offering unprecedented insights into the flotation cell's behavior. Key measurements, in real time, such as bubble size distribution (D32), gas hold-up (Eg), and superficial gas velocity (Jg) allow for precise control and optimization of the flotation process.

Flotation 360™ builds on this innovation by integrating the in-situ pulp sensor with a comprehensive digital solution designed to optimize flotation circuits. By leveraging predictive analytics, machine learning, and expert insights, Flotation 360™ provides real-time and predictive recommendations to enhance valuable product recovery, reduce manual troubleshooting time, minimize the impact of instrumentation failures, and optimize reagent dosing.

A trial of the Flotation 360™ offering at a copper plant included step tests on air and frother dosage to evaluate the in-situ pulp sensor's performance. The sensor demonstrated its capability to deliver real-time hydrodynamic measurements, representative of what is expected from years of research in the field. These revolutionary new measurements confirm the potential to transform flotation optimization in the industry.

In the presentation, we will explore the results in detail, both visually and quantitatively. We will also highlight some of the findings from the unique capabilities of Flotation 360™. The combination of the in-situ pulp sensor with advanced process control, real-time monitoring, and predictive analytics provides the foundation for a new and comprehensive approach to flotation optimization.

Professional Profile:

https://www.linkedin.com/in/adriaan-haasbroek/

https://www.linkedin.com/in/lidia-auret-s3-sun-uct/

https://www.linkedin.com/in/juananes/

https://www.linkedin.com/in/javier-basurco-0b602052/