Vertimills Applications in Primary Grinding Circuits

Daniel Suarez

Abstract

The mining industry is constantly looking for alternatives to decrease the operating costs of the comminution circuits. The presentation aims to showcase recent Vertimill® installations in primary grinding circuits, presenting them as an alternative to horizontal ball mills to enhance the efficiency and reduce operating costs of primary grinding circuits.  The Vertimilll® developed in the 1980’s as an alternative to decrease operating costs, primarily energy and ball consumption, was originally designed and embraced by the mining industry for applications in regrind circuits. Since then, applications of Vertimills® for brown field and green field projects in primary grinding circuits have been installed, such installations have proven the Vertimill ® to be a better alternative to horizontal ball mills. In the last few years, the installed power of the Vertimills ® has increased significantly, a Vertimill® 7000 (HP) is currently the largest Vertimill® being offered to the mining industry.  The presentation highlights the various installations in primary grinding circuits; it also describes the reported savings in operating costs along with the required operating conditions to make the use of such grinding technology viable in primary grinding circuits.  The presentation will provide a preliminary equipment sizing and expected cost/benefit analysis comparing the installation of Vertimill ® -7000 as an alternative to horizontal ball mills for a 50 ktpd SABC and HPGR comminution circuits.

Categoria

3- Procesamiento de Minerales y Metalurgia Extractiva

Datos del Autor:

Daniel Suarez Pacheco

Consultant,

Cia : D. Suarez Consulting

Address: 841 Heritage Hills Drive , York, PA, USA 17402

Email : Daniel. Suarez23@gmail.com

Cel : 717-887-5622

Resena Profesional:

Ingenierio Metalurgico – graduado de la Universidad de San Agustin Arequipa Peru.

Estudios de post grado – University of Utah – Salt Lake City – grados de Master of Science y PhD in Metallurgy

- 30 years of experience.

  - Metalurgist for Asarco’s Ray Mine Copper Leaching and Electrowinning plant Arizona – USA

  - Senior  Metallurgist – Falconbridge’s Ni and Copper Technology Center in Sudbury, Ontario Canada ( 9 yrs)

  - Plant Superintendent – Falconbridge’s Ferro – Nickel Operation in Dominican Republic (4 yrs)

  - Process Engineering Manager (22 yrs) – Responsible for sizing comminution (grinding) equipment for worldwide projects– Metso – York, PA, USA.

  - Currently – Retired working as a Consultant.