The Uchucchacua / Yumpag mine is wholly owned and operated by Buenaventura. 

Uchucchacua operations began in 1975 and remains our largest single source silver production. The mine has been temporarily closed from October 2021 due to the operational problems that were aggravated by the COVID-19 pandemic, including delays in the preparation and exploration of the mine. The mine resumed its operations in September 2023. It is located in the province of Oyón, in the department of Lima, approximately 265 kilometers northeast of the city of Lima at an altitude of between 4,000 and 5,000 meters above sea level.

 Yumpag is located in the province of Daniel Alcides Carrión, in the department of Pasco, approximately 267 kilometers northeast of the city of Lima at an altitude of between 3,750 and 5,150 meters above sea level. The mine site is accessible through the Panamericana Norte highway, following the Lima - Huacho - Sayán - Churín - Oyón - Uchucchacua route for a distance of 267 kilometers.

History

Uchucchacua is a silver deposit in the central highlands discovered during the viceroyalty. Evidence of this are the many Spanish workings in the areas of Nazareno, Mercedes, Huantajalla and Casualidad. The mines passed into the hands of the Jungbluth, who continued with small scale works and even mined ore in Uchucpaton and Otuto, where there are vestiges of old “mills”.

At the beginning of 1960, Cia. de Minas Buenaventura started prospecting-exploration works in the area. Initial conditions were difficult as there was no road between Oyón and Chacua road until 1965, and the road was only later extended to Yanahuanca. From 1969 to 1973, Buenaventura installed a pilot plant that initially treated ores from the Socorro and Carmen mines. Satisfactory results led to the installation of an industrial plant in 1975, which currently has a treatment capacity of 4,200 metric tonnes per day. Currently, the Socorro, Carmen and Casualidad mines are operating. The Huantajalla area mine is also operating, though to a lesser extent. In September 2023 Yumpag mine initiated an industrial treatment test in Uchucchacua plant.

Title, leases and options

The Uchucchacua mining unit, including Yumpag, comprises 32 mining concessions and one beneficiation concession (concentrator). These 32 concessions represent the area of mines and exploration projects. Mining and exploration activities are carried out within these mining concessions. Uchucchacua’s concessions have a total area of approximately 45,600 hectares.

Mineralization

The Uchucchacua mineral structures are hosted by Mesozoic limestone of the Jumasha Formation and are classified as a mesothermal polymetallic deposit of silver-lead-zinc with important contents of manganese. The main mineralized structures are veins and ore bodies with high-grade silver content.

La mineralización que justifica el proyecto está caracterizada por cuerpos de Ag-Mn-(Pb-Zn) que están emplazados en la intersección de estructuras de dirección N60° a E-W con un nivel de calizas favorables, caracterizadas por ser limpias, con poco contenido de arcillas, de textura packstone a grainstone, con abundante fauna de foraminíferos, la cual se define como un “horizonte prospectivo” dentro de la Formación Jumasha superior; mientras que aquellas facies finas, generalmente margosas del tipo mudstone a wackestone, no han sido mineralizadas o en su defecto, lo fueron débilmente

Operations and infrastructure

Uchucchacua is an underground mine with mechanized bench-and-fill and cut-and-fill mining methods. This method is used to mine ore veins thickness from 1.0 to 3.5 m. Mining is done by us and a service contractor, the equipment used for development and exploration includes single-arm jumbos, bolters, scaler and scoop loaders (4yd3). The ore is transported by 10m3 dump trucks from the loading pockets to the Luz and Master Shafts. Ore haulage to surface is done with locomotive to the process plant.

Yumpag is mined with mechanized sub-level stopping (SLS) and Over Drif and Fill (ODF) mining methods. SLS method is used to mine ore veins thickness from 4.0 to 10m approx.. and ODF method is used galerys 4x4m with a variable length. The mining advance sequence in ODF method is ascending with primary and secondary stopes drifts, filled with cemented rockfill or waste fill. The equipment used for development and exploration includes jumbos, bolters, scalers and scoop loaders (6 yd3). Ore haulage to surface is done with 15 m3 underground dump trucks. The mineral is transported to the Colquicocha stockpile (Uchuchacua mine), to be subsequently processed.

Both Uchucchacua y Yumpag ore are processed at a mill located at Uchucchacua. The mill has a rated capacity of 4,200 tonnes per day and utilizes differential flotation to obtain a lead-silver concentrate and a zinc concentrate. Electric power is obtained from the Peruvian national electricity grid, a hydroelectric plant and a diesel generator. Water for operations at Uchucchacua is obtained from three local lakes.

During 2024, the Rio Seco manganese sulphate plant operated with supply of silver manganese concentrate from the Yumpag mine. A pilot plant was built to produce battery grade manganese sulphate and is currently working towards market specifications. A Metallurgical Laboratory is also in operation to support BVN's operations and projects.